Source:	Huawei

Title:	Distinguishing Two Bootstrap Data Channels with the Same Stream ID value

Document for:	Discussion and Agreement

Agenda Item:	10.11

# 1	Introduction

This contribution discusses how the two bootstrap data channels with the same stream ID value contained in the SDP transferred between the originating network and the terminating network.

# 2	Discussion

Figure 6.2.10.1-1 in TS 26.114 [1] shows the Data Channel Server (DCS) in User A’s network establishes one bootstrap data channel with User A and User B respectively. According to Table 6.2.10.1-2 in TS 26.114 [1], the stream ID value of the bootstrap data channel between User A and the DCS is 0, and the stream ID value of the bootstrap data channel between User B and the DCS is 100.

Then if both User A’s network and User B’s network support IMS data channel, they will establish one bootstrap data channel with User A and User B respectively. Figure 1 below shows this case. Every UE will have one bootstrap data channel with its local network (stream ID = 0) and one bootstrap data channel with its remote network (stream ID = 100).

Figure 1: Distribution of Bootstrap Data Channels

Figure 2 below shows the media negotiation procedure for this case in brevity. UE A initiates an IMS session with IMS data channel capability to UE B. UE A is the originating party and UE B is the terminating party.

Figure 2: Media Negotiation Procedure for Bootstrap Data Channels

1.	UE A sends an INVITE request to Network A with two media descriptions in the SDP offer, each of which contains a "a=dcmap" line respectively. One "a=dcmap:0" line indicates UE A requests to establish one bootstrap data channel with the DCS in Network A and the stream ID value is 0. One "a=dcmap:100" line indicates UE A requests to establish one bootstrap data channel with the DCS in Network B and the stream ID value is 100.

2.	Network A need to terminate the bootstrap data channel identified by "a=dcmap:0" line and transfer the bootstrap data channel identified by "a=dcmap:100" line to Network B. Network A also need to initiate a new bootstrap data channel with UE B, which is identified by one "a=dcmap:100" line in an added media description. Then in the updated INVITE request sent out by Network A, there are two "a=dcmap:100" lines in the SDP offer.

3.	Network B need to distinguish these two "a=dcmap:100" lines, and terminate the one which is established between User A and itself and transfer the other to User B which is established between Network A and User B. In addition, Network B also initiates a bootstrap data channel with UE B by adding a new media description containing one "a=dcmap:0" line in the INVITE request. Then Network B will send out the updated INVITE request to UE B with one "a=dcmap:0" line and one "a=dcmap:100" line in the SDP offer.

4-6.	A 18X response with the SDP answer of bootstrap data channel media description is transferred from UE B to UE A.

In step 3, some mechanism is needed to help Network B to distinguish the two "a=dcmap:100" lines in the SDP offer of the INVITE request from Network A. In some cases, the data channel traffic may be anchored to the DCSs on Network A and Network B. Then, the IP addresses in the SDP offer received by Network B from Network A are all that of the DCS on Network A. Therefore, Network B is unable to distinguish the two "a=dcmap:100" lines in the SDP offer by the IP addresses contained in the "c=" lines. The mechanism shall support the cases that the data channel traffic is anchored to the DCSs.

# 3	Solution

IETF RFC 8864 [2] has specified "a=dcmap" attribute syntax which includes a 'label' parameter. It could be used to distinguish the two bootstrap data channels with the same stream ID value between Network A and Network B.

Figure 3 below shows this solution. That is, Network A adds the 'label' parameter with value "caller" in the "a=dcmap:100" line to indicates the bootstrap data channel is established between UE A and Network B, and adds the 'label' parameter with value "callee" in the "a=dcmap:100" line indicates the bootstrap data channel is established between Network A and UE B.

Figure 3: Mechanism to Distinguish the Two Bootstrap Data Channels with the Same Stream ID

# 4	Proposal

It is proposed to include this solution in TS 26.114 [1] to help the terminating network to distinguish the two bootstrap data channels with the same stream ID value.

# 5	References

[1]	3GPP TS 26.114, "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction (Release 16)"

[2]	IETF RFC 8864, "Negotiation Data Channels Using the Session Description Protocol (SDP)"