3GPP TSG-SA WG4 Meeting #122	S4-230xxx

Athens, Greece, 20 – 24 February 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.565 | CR |  | rev |  | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Harmonizing the SR reference architectures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_MSE |  |  |  |  |  | Date: |  |  | 14-02-2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change

## 5.1	Reference Architecture

### 5.1.1	Introduction

We define different variants of the reference architecture for the split rendering MSE, each representing a different perspective and level of detail.

### 5.1.2 	Client Architecture

Figure 5.1.2-1 depicts the reference client architecture.

Split Rendering Client

Figure 5.1.2-1 – Device architecture of AR UE

The split rendering client consists of the following components:

- The Media Access Functions: allow for fetching and processing of the pre-rendered media in preparation of final display. The MAF is also responsible for the carriage of any metadata or local media to the split rendering server.

- The scene manager and thin Presentation Engine: is responsible for the negotiation of the split rendering session and the parsing of the description of the rendered media as provided by the SR server. It is also responsible for setting up and managing the XR session with the XR runtime.

- The XR source management: is responsible for gathering timed metadata such as pose and action information and sending it to the SR server.

### 5.1.3	End-to-End Architecture

The end-to-end architecture is depicted in figure 5.1.3-1.

Editor’s Note: this architecture variant is TBD based on the MeCAR discussions.

### 5.1.4 Control Architecture

Figure 5.1-3 – Split management architecture

### 5.1.5	User Plane Architecture

Figure 5.1.5-1 depicts the user plane architecture for split rendering.

The SR interfaces are considered to be specializations of their parent RTC interfaces as defined in TS26.506.

## 5.2	Procedures and Call Flows

### 5.2.1 	Call flow for Split Rendering instance discovery

#### 5.2.1.1 Client-driven procedures and call flows

Figure 5.2.1-1 demonstrates a call flow for setting up the split rendering by the client.

Figure 5.2.1-1: High-level call flow for initiating a split

Steps:

- The  Application Service Provider requests the SRS the provisioning a split management session.

- The split management session is announced to the Application as part of the Service Access Information.

- The application requests a split of the client media functions from the SRC.

- The SRC inquires the Media Service Client to discover the client’s media capabilities.

- The SRC requests the SRS split of the client media functions.

- The SRS registers with the RTC AF and provides its capabilities.

- The SRC and SRS negotiate on the acceptable capabilities for the device and agree on the split rendering configuration.

- The RTC AF requests the SRS  to start the split-rendering process on the edge.

- The SRS acknowledges to the SRC that the split-rendering on edge is running and provides its access information.

- The SRC acknowledges to the Application that the split-rendering on edge is running.

- The Application requests starting a media session from the SRC.

- The 5GMS client connects to the split-rendering on the 5GMS AS using provided information in step 9.

#### 5.2.1.2 Network-driven procedures and call flows

Figure 5.2.1.2-1 demonstrates a call flow for setting up the split rendering by the client.

Figure 5.2.1.2-1: High-level call flow for the network-driven split management

Steps:

- The 5GMS Application Service Provider requests the SRF the provisioning a split management session.

- The split management session is announced to the Application as part of the Service Access Information.

- The SRF inquires the 5GMS AS about its capabilities.

- The SRF requests the SRC split of the client media functions.

- The SRC inquires the Media Service Client to discover the client’s media capabilities.

- The SRC and SRF negotiate on the acceptable capabilities for the device and agree on the split option.

- The SRF requests the 5GMS AS to start the split-rendering process on the edge.

- The SRF acknowledges the SRC that the split-rendering on edge is running and provides its access information.

- The SRC acknowledges the Application that the split-rendering on edge is running.

- The 5GMS-aware Application request starting a media session from the 5GMS client.

- The 5GMS client connects to the split-rendering on the 5GMS AS using provided information in step 9.

### 5.2.2 	Call flow for Split Rendering session setup

The split rendering operation can be described by the following call flow:

The steps are:

- The Presentation Engine discovers the split rendering EAS and sets up a connection to it. It provides information about its rendering capabilities and the OpenXR configuration.

- In response, the split rendering EAS creates a description of the split rendering output and the input it expects to receive from the UE.

- The Presentation Engine requests the buffer streams from the MAF, which in turn establishes a connection to the split rendering EAS to stream pose and retrieve split rendering buffers.

- The Source Manager retrieves pose and user input from the XR runtime.

- The Source Manager shares the pose predictions and user input actions with the split rendering EAS

- The split rendering EAS uses that information to render the frame

- The rendered frame is encoded and streamed down to the MAF