Source:	Samsung Electronics Co., Ltd.

Title:	Proposed updates on AR communication architecture and basic call flow

Document for:	Discussion and Agreement

Agenda Item:	10.6

- Introduction

This contributions proposes updates on AR communication architecture and basic call flow in IBACS PD considering related works in SA2.

- Proposed updates on IBACS PD

# 3 AR IMS Communication Architecture

  - 3.1 Potential Reference Architecture (SA2)

[Editor’s Note: this section needs further updates aligned with the work in SA2]

[ The following diagram is captured from S2-2301450 which is a CR on TS 23.228 for supporting AR Telephony Communication endorsed at SA2#154-AH-e and depicts a potential reference architecture that is being discussed by SA2 as part of NG_RTC.

Figure 3.1.1 Potential Reference Architecture

]

# 4 IMS AR Sessions and transport

  - 4.1 Call Flows

    - 4.1.1 Basic Call Flow

The figure shows the call flow for a basic AR call. Enhanced MRF consists of both the Data channel media function and the AR media function. The figure may be updated later based on SA2 work.

Figure 4.1.1.1 Basic AR Call Flow

The steps are as follows:

A. Call Setup

- The calling UE, in this case UE1, sends a SIP invite to S-CSCF through P-CSCF to initiate the call

- The S-CSCF identifies this as an AR call and forward the invite request to the IMS AS

- The IMS AS sends a request to the DCSF to request the data channel resources for the AR call data channel application.

- The DCSF selects a Enhanced MRF and requests the allocation of data channel resources and the execution of the AR call data channel application

- The Enhanced MRF confirms the successful allocation of resources and the DCSF forwards the confirmation to the IMS AS

- The IMS AS then sends the invite which includes the updated SDP offer adding media information of the Enhanced MRF to origination S-CSCF and then to UE2 possibly through UE2’s network entities.

- UE2 accepts the invitation and informs UE1. Both UE1 and UE2 establish a data channel connection and channels for media flows to the Enhanced MRF.

B. Scene description distribution

- Enhanced MRF prepares the scene description based on media descriptions and assets for the call.

- Enhanced MRF delivers the scene description to the UEs.

C. Scene description update

- A UE may trigger a scene update e.g., when a new object is added/removed in the scene, or a spatial information update is sent. The figure shows the update is triggered by UE1, but this can be either UE.

- The Enhanced MRF will process the new information and creates a scene description update. It is also possible for the Enhanced MRF to initiate an update without an update from the UEs.

- Enhanced MRF distributes scene description update to all UEs.

NOTE: Spatial data related updates may be required for collaborative AR calls, e.g., when multiple users are physically collocated and also part of the same AR experience. The type of spatial description updates is FFS.

D. AR Media and Metadata Exchange

- Both UEs will send and receive the AR media required (and negotiated) for the call as part of the call setup and scene description information received. Prestored media can be fetched from the Enhanced MRF. Real-time conversational media can flow via the Enhanced MRF. The Enhanced MRF may process the AR media before delivery.

- Related metadata (e.g. User Pose) may be needed during the AR session. The metadata may be delivered as RTP header extension, RTCP feedback or over data channel; this aspect is FFS.

NOTE 1: Delivery of real-time media over data channel is FFS.

NOTE 2: split rendering for UE1 and/or UE2 needs to adhere to the procedures and formats that are defined by the SR_MSE work item. The Enhanced MRF may be the entity that performs the split rendering for the UE.

NOTE 3: The interactions between AR Application Server and other entities including IMS AS, Enhanced MRF and DCSF is FFS and not shown in Figure 4.1.1.1.

    - 4.1.2 3D video call flow

Figure 4.1.2.1 Call flow for bidirectional 3D video call

Steps 1-13 under A. Call Setup, B. Scene description retrieval and C. scene description update will be the same as for the basic AR call flow. The remaining steps are defined below.

D. AR Media and Metadata Exchange

- UE1 will send 3D video (e.g., RGB-D streams, or volumetric media represented as mesh or point cloud) to the MRF.

- UE2 also sends 3D video to MRF.

- The MRF processes AR media, e.g., for viewport-dependent processing (VDP), combining AR objects into a single frame, transcoding from one 3D representation to another, etc. Note that VDP requires a feedback channel from UEs to MRF not shown in the call flow.

- MRF delivers 3D video to UE1.

- MRF delivers 3D video to UE2.

[Editor Note: this needs to be aligned with Basic AR Call Flow]

NOTE: Delivery of real-time media over data channel is FFS.

- Proposal

We propose to integrate the text in clause 2 to the next version of the IBACS PD.