Source: 	Nokia Corporation

Title: 	[MeCAR] Addition of eye gaze to interaction metadata

Document for	Agreement

Agenda item: 	9.5

# Introduction

MeCAR PD v4 classifies AR/MR data into three categories: device capability, media description and interaction metadata. The listed interaction data includes AR anchor, user pose, FOV, viewport, gesture, body action, facial expression and sensor information. However, eye gaze is missing in the current version.

Eye tracking is widely supported by MR headsets e.g. Microsoft HoloLens 2, Meta Quest Pro. Current devices typically perform tracking using a set of infrared (IR) emitters to illuminate the eye and an IR camera to capture eye images. The user’s gaze is then estimated from the appearance of eye using image processing algorithms.

Eye gaze data enables several functionalities in XR applications. Some example are eye-supported selection and control of objects, automatically scrolling through text and attention tracking.

Eye gaze input is enabled in OpenXR by an extension that defines an interaction profile path and input type. This extension is implemented by several XR engines such as Unity [1] and Unreal [2].

# Proposal

We propose to modify Table 2 as below to add eye gaze as an interaction data type to the MeCAR PD.

Table 2 – AR/MR Data type definitions

| AR/MR Data category | AR/MR Data type | Definition | Media type description (Examples) |
| --- | --- | --- | --- |
| … | … | … | … |
| Interaction | AR Anchor | The AR anchor is meant to identify a point in the user space to be used to anchoring a visual object (2D or 3D) | Type: Metadata allowing accurate overlaying/rendering of text, graphics or video contents to support Use Case 8 of TR 26.928. Organization: None |
|  | User Pose | Clause 4.4.3.1 of 3GPP TR 26.998[1] Representation of the user position and orientation | Type: It consists of a quaternion for orientation and a 3D vector for position. Timestamp is represented by a 64 bit monotonically increasing nano-second-based integer. Organization: Khronos OpenXR |
|  | FOV | Y.6.2.3 of 3GPP TS 26.114[3] The Field of View (FOV) is the extent of observable world at any given moment | Type: It consists of vertical fov and horizontal fov. Organization: None |
|  | Viewport | Y.7.2 of 3GPP TS 26.114[3] The viewport corresponds to the projection of the user View onto a target display | Type: It shall contain all of the parameters Viewport_azimuth, Viewport_elevation, Viewport_tilt, Viewport_azimuth_range and Viewport_elevation_range Organization: None |
|  | Gesture | TBD | Type: A array of finger joint position. For example: https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html#XR_EXT_hand_tracking Organization: OpenXR |
|  | Body action | TBD | Type: bvh format. Frequency: at least 1kHz Organization: BioVision company |
|  | Facial expression | TBD | Type: An array of key point position. For example: https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html#XrSystemFacialTrackingPropertiesHTC Organization: None |
|  | Sensor information | TBD | Type: a new interaction profile path For example, OpenXR EXT format offers the possibility to developers to integrate and benefit from new controllers and sensor subsystems, (e.g., https://registry.khronos.org/OpenXR/specs/1.0/html/xrspec.html#XR_EXT_hp_mixed_reality_controller, etc.). Organization: OpenXR |
|  | Eye gaze | Eye gaze typically consists of a gaze origin (a point positioned between the user’s eyes) and a gaze direction, a ray pointing towards where the user is looking at. | Type: 3D vector for gaze origin and a quaternion for gaze direction. For example, OpenXR defines a new interaction profile path for eye gaze input: https://registry.khronos.org/OpenXR/specs/1.0/html/xrspec.html#XR_EXT_eye_gaze_interaction Organization: OpenXR |

[1] https://docs.unity3d.com/Packages/com.unity.xr.openxr@1.0/manual/features/eyegazeinteraction.html

[2] https://docs.unrealengine.com/4.26/en-US/API/Plugins/OpenXREyeTracker/IOpenXREyeTrackerModule/

Contact: Igor Curcio, Saba Ahsan, Serhan Gül, Nokia Technologies, Finland. Emails: igor.curcio, saba.ahsan, serhan.guel@nokia.com.