Source:	Intel

Title:	Procedure of WebRTC control signaling establishment

Agenda Item:	10.7

Document for:	Discussion and Agreement

- Introduction

In this contribution, we propose the high-level procedure of WebRTC control signaling establishment based on TS 26.506-100[1].

The agreed 5G-RTC general architecture is below:

Figure 4.1-1: 5G-RTC General Architecture (TS 26.506-100)

2. proposed changes:

* * * First Change * * * *

5. x Procedures for control signaling establishment  .

This clause describes the high-level procedure for WebRTC control signaling establishment. The procedure assumes that the MNO offers the trusted ICE function and WebRTC signaling function. It also assumes WebRTC-related configuration information is only accessible from configuration function.

#### 5.x.1 General

The procedures of enabling WebRTC control signaling establishment is illustrated in Figure 5.x-1.

Figure 5.x-1 High-level procedure of enabling WebRTC control signaling establishment

Steps:

- The MSH retrieves configuration information from configuration function.

In case when provisioning function is accessible, step 2-4 may apply

- The configuration function queries ICE functions configuration information (e.g., STUN/TURN server IP address) from provisioning function.

- The provisioning function sends query about ICE-related configuration from RTC application provider

- The configuration function stores retrieved ICE information.

- The configuration function sends ICE information to UE MSH

- The UE MSH sends ICE configuration information to ICE function

- The ICE function gathers MNO’s networking information and sends feedback to MSH

- The MSH starts session setup with WebRTC signalling server

- The WebRTC signalling server relays session negotiation feedback from other WebRTC peers.

NOTE: The steps 8 and 9 may repeat during WebRTC session negotiation phase.

* * * End Change * * * *

- Proposal

It is proposed to add clause 2 into the GA4RTAR TS 26.506 as a base line for stage 3 work.

Reference:

[1] TS 26.506-100 5G Real-time Media Communication Architecture (Stage 2)