Source: 	Nokia Corporation

Title: 	[5G_RTP] Grouping in IRTCW and IBACS

Document for	Agreement

Agenda item: 	10.8

# Introduction

Use cases in WIs IBACS and IRTCW require grouping of different media and metadata. For example, in IBACS permanent document section 4.3.2, there is an Editor’s note stating the need for an SDP parameter for grouping the motion signals with the 3D model. The current document defines a generic grouping mechanism that can be used in 3GPP.

Note that this mechanism is proposed for new group types that will be defined in 3GPP and does not propose any changes to the use of existing SDP a=group types defined in IETF and supported in 3GPP, such as, LS, bundle, etc.

# Grouping RTP Streams

## 3gpp_group attribute

RTP streams and data channels shall use the group attribute as defined by RFC 5888 and RFC 9143 when applicable.

In addition the attribute a=3gpp_group is defined here for grouping mechanisms defined in 3GPP. The syntax of the attribute is listed below:

3gpp-group-attribute     = "a=3gpp_group:" group-type

*(SP identification-tag)

[SP grouping-parameters]

":"[SP "group_id="group-identification-tag]

group-type = token

identification-tag = token

group-identification-tag = token

; token is defined in RFC 4566

The identification-tag and group-identification-tag shall be unique in the SDP. The group-identification-tag is used for identifying the group. There shall not be a group-identification-tag with the same value as an identification-tag in the SDP.

Only group-type defined by the 5G_RTP specification shall be used. An identification-tag can be a media stream id (as defined by the "a=mid:" tag) or a group-identification-tag. The group-type specification shall define whether media stream id, group-identification-tag or both are allowed. The syntax and semantics of the grouping-parameters shall be defined for each group-type if used.

When using IETF defined groups with the a=group SDP attribute, a group-identification-tag may still be assigned using the attribute 3gpp-group-id with the following syntax:

3gpp-group-id     = "a=group-id:" group-identification-tag

The 3gpp-group-id attribute shall be included immediately after the a=group attribute of the group to which it is assigned.

Editor’s Note: group-types for grouping media and metadata can be defined as part of Release 18.

# Proposal

We propose to include the above text in the 5G_RTP permanent document.

Contact: Igor Curcio, Saba Ahsan, Nokia Technologies, Finland. Emails: igor.curcio, saba.ahsan@nokia.com.