Agenda item: 	10.7

Source: 	InterDigital Communications

Title: 	[GA4RTAR] Procedures for Edge Processing

Document for:	Discussion and Agreement

# Introduction

This contribution addresses one of the objectives of the GA4RTAR work item, namely:

“Specify support for AR relevant functionalities such as split-rendering or spatial computing on top of a 5G System based on this architecture”.

In this contribution, we discuss the procedures for 5G-RTC edge processing in the client-driven management and Application Provider-driven management scenarios of 5G-RTC edge processing.

# Procedure for Edge processing

  - Client-driven Management of 5G RTC Edge Processing

The detailed call flow for client-driven management of edge processing session is shown in Figure 1.

Figure 1. Client-driven management of 5G-RTC edge processing

The Edge Computing Provisioning phase is a provisioning phase, that may be repeated several times (e.g., to extend edge processing coverage to new geographical areas or to increase the capacity of an already provisioned area). All steps in this phase are optional and performed on need basis. The steps are:

1.	Spawn ECS: In this step, a new ECS instance is instantiated to manage new or increased demand for edge processing.

2.	Spawn 5G-RTC AF: In this step, a new 5G-RTC AF that is edge-enabled is instantiated to handle new or increased demand for WebRTC sessions with edge processing.

3.	EES Configuration: The EES is configured for a specific Edge Data Network (EDN).

4.	EES Registration with ECS: The EES registers with the ECS that is in authority over the target EDN.

The 5G-RTC Application Provider Provisioning phase is performed prior to the establishment of any related WebRTC sessions by the 5G-RTC Application Provider. Subsequent updates to the provisioning session are possible.

5.	Create Provisioning Session: In this step, the 5G-RTC Application Provider creates a new provisioning session.

6.	Provision 5G-RTC features: In this step, the 5G-RTC Application Provider may create different configurations such as QoS support, charging, collection of consumption, offering STUN/TURN servers, WebRTC signalling servers, Edge Processing, etc.

The WebRTC Application initiates a new RTC session:

7.	Application Initialization: The user launches the WebRTC Application. The WebRTC application performs any required initialization steps.

8.	Start session: The WebRTC Application invokes the WebRTC framework with appropriate real-time streaming access parameters.

9.	Session starting event: The application informs the Media Session Handler about the start of a new WebRTC session over 5G.

10.	Retrieve service access information: The Media Session Handler retrieves Service Access Information from the 5G-RTC AF appropriate to the WebRTC session.

11.	Determine eligibility for requesting edge resources: Using information from the Service Access Information, the Media Session Handler determines whether the WebRTC session is eligible for requesting edge resources.

If the eligibility criteria are met in the previous step, the UE discovers an EAS instance offering 5G-RTC AS functionality in the Client-based Edge Computing Discovery phase:

12.	Locate EAS instances: The MSH asks the EEC to discover the location of one or more suitable EAS instances offering the “5G-RTC AS” capability that can serve the application.

13.	Locate local EES: The EEC queries the ECS for a suitable EES (EDGE-4 API).

14.	Register with EES: The EEC registers with the selected EES (EDGE-1 API).

15.	Request list of “5G-RTC AS” EAS instances: The EEC queries the EES or one or more EAS instances offering the “5G-RTC AS” capability that can serve the session, using EAS discovery filters (see Table 8.5.3.2-2 in [2]) provided by the Application Client, e.g. “5G-RTC AS” for EAS type, appropriate values for service feature(s), and other EAS characteristics.

The optional sub-flow 5G-RTC AS Provisioning is for provisioning an additional 5G-RTC AS instance if a suitable EAS instance offering the "5G-RTC AS" capability cannot be located. The steps are:

16.	Check resource template: The 5G-RTC AF checks the provisioned edge processing resource template for the related application to determine the requirements of the application.

17.	Instantiate new EAS/5M RTC AS: The 5G-RTC AF requests the MnS to instantiate a new “5G-RTC AS” EAS instance with the specified requirements and considering parameters provided in the query by the EEC.

18.	Spawn 5G-RTC AS instance: The MnS creates a new instance of the EAS offering “5G-RTC AS” capability with the requested placement and resources.

19.	EAS configuration: The newly instantiated “5G-RTC AS” EAS instance is configured.

20.	Register EAS with EES: The newly instantiated EAS instance registers itself with the triggering EES.

21.	Configure provisioned features: This may include configuring and launching the server-side application in the 5G-RTC AS.

Completion of UE Edge Computing Discovery phase:

22.	List of suitable “5G-RTC AS” EAS instances: The EES/5G-RTC AF responds to the EEC with a list of “5G-RTC AS” EAS instances and their characteristics in an EAS discovery response (see Table 8.5.3.3-1 in [16]).

23.	Select preferred “5G-RTC AS” EAS instance: The AC and/or EEC select(s) a “5G-RTC AS” EAS instance from the provided list, based on the AC’s desired criteria.

After successful discovery of a “5G-RTC AS” EAS instance, the actual WebRTC session over 5G may start:

24.	Media transfer: The WebRTC Application connects to the selected EAS “5G-RTC AS” and the real-time streaming starts.

25.	Method calls and notifications: Supporting information about the WebRTC session is passed from the WebRTC framework to the Media Session Handler.

26.	Reporting, network assistance, and dynamic policy: The Media Session Handler exchanges supporting information about the WebRTC session with the 5G-RTC AF.

27.	End session: The WebRTC Application informs the WebRTC framework that the RTC session has ended.

28.	Session ending event: The WebRTC framework informs the Media Session Handler about the end of the RTC session.

29.	Final reporting: The Media Session Handler performs any final reporting to the 5G-RTC AF.

  - Application Provider (AP)-driven Management of 5G-RTC Edge Processing

The detailed call flow for the AP-driven management of edge processing is shown in Figure 2. In the client-driven call flow sequence, the optional provisioning of an additional 5G-RTC AS instance occurs in response to an explicit call from the Application Client to an on-board EEC, whereas in this case it occurs, if needed, as part of and in response to Application Provider provisioning. The 5G-RTC AS instance may also be reprovisioned at any time during the lifetime of the 5G-RTC Provisioning Session.

Figure 2. Application Provider-driven management of 5G-RTC edge processing

The Edge Computing Provisioning phase is a provisioning phase, that may be repeated several times (e.g., to extend edge processing coverage to new geographical areas or to increase the capacity of an already provisioned area). All steps in this phase are optional and performed on need basis. Steps 1–4 are identical to those described in clause 2.1.

The 5G-RTC Application Provider Provisioning phase is performed prior to the establishment of any related WebRTC sessions by the 5G-RTC Application Provider. Subsequent updates to the provisioning session are possible. Steps 5–6 are identical to those described in clause 2.1.

The optional sub-flow to provision an additional 5G-RTC AS instance may be repeated multiple times on need basis to add new capacity, to increase existing capacity for edge processing or to reallocate under-used edge processing resources to other tasks. The edge processing capacity is tailored for the specific 5G-RTC Application Provider based on the information in the Provisioning Session. Steps 7–12 in Figure 2 are identical to steps 16–21 described in clause 2.1.

After successful EAS discovery, the actual RTC media streaming session may start in the WebRTC session over 5G phase. Steps 13–16 in Figure 2 are identical to steps 7–10 described in clause 2.1, and steps 17-22 are identical to steps 24-29.

# Proposal

We propose to agree the proposed procedure text in 5G-RTC architecture extension and document it in the clause 5.1 of GA4RTAR TS.

# References

- 3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

- 3GPP TS 23.558: "Architecture for enabling Edge Applications".