SA4#126 RTC SWG	S4-231676

Chicago, US, 13 November 2023	Rev of S4R230119

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | pseudo | rev | - | Current version: | 0.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP] RTP header extension for in-band end-to-end delay measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP |  |  |  |  |  | Date: |  |  | 13/11/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As a next step documented in TR26.806 upon the completion of the study item FS_SmarTAR, RTP header extension for in-band end-to-end delay meansurement is recommended for normative work under 5G_RTP. The 5G_RTP WID SP-230541 was updated accordingly to capture the recommendation at the SA4 #124 meeting. Google gave the permission to register the “RTP Header Extension for Absolute Sender Time” (currently implemented in WebRTC but not registered with IANA) with IANA and that allowed us to reuse it as part of the proposed solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New RTP header extensions formats, the binding between RTP header extensions, and the associated SDP signaling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current mechansisms may not be accurate for low-latency multimedia applications because the delays resulting from the delay measurmenets are not representative of the delays experienced by the media, and may not be able to measure some delays. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Background

The 3GPP SA4 SmarTAR study item recommended specifying RTP header extensions for in-band end-to-end delay measurement, as documented in TR 26.806 [1]. The RTP header extensions can be used for delay compensation by the 5G network when the end-to-end path consists of both a 5G segment and one or more non-5G segments. An example of the end-to-end path is shown in Figure 1. The 5G segment is between the Phone and the UPF. There are two non-5G segments: the segment between the AR Glasses and the Phone which uses Wi-Fi, and the segment between the UPF and the Edge Application Server (EAS) which uses the Internet. The non-5G segments, i.e., Wi-Fi and the Internet, do not guarantee delays. However, if the aggregate non-5G delays, denoted Dn= Dn,1+ Dn,2, where Dn,1 and Dn,2 are the delay between the AR Glasses and the Phone and the delay between the UPF and the EAS respectively, is measured and known to the 5G network, the 5G network may adjust the delay within the 5G network (denoted Dc) to meet the end-to-end delay requirement.

Figure 1: An end-to-end path and the delay breakdown

There exist mechanisms for end-to-end delay measurement, such as the ICMP ping messages and the RTCP packets. However, these mechansisms have the shortcoming that the measured delays may not represent the delays experienced by the media because the delay-measurement packets and the media packets may receive different QoS treatments. For more details, one may refer to clause 6.2 of TR 26.806 [1]. To resolve this issue, the timestamp(s) are sent along the RTP packets that carry the media.

The proposed RTP header extensions and how to use them for delay masurements is shown in Figure 2. From the AR glasses to the EAS is an RTP packet with a header extension that carries one timestamp T1. For convenience, we call it type-1 RTP header extension. In the opposite direction is an RTP packet with a header extension that carries three timestamps (T1, T2, T3), and we call it type-2 RTP header extension.

The one-way delay from the AR Glasses to the Edge Application Server is calculated as T2 - T1. The one-way delay in the oppositie direction is calculated as T4 – T3, and the round-trip time (RTT) is calculated as T4 – T1 – (T3 – T2). The type-2 RTP header extension carries three timestamps: T1, T2, T3. A question is why does the type-2 RTP header extension carry both T1 and T2? It seems that carrying T2-T1 instead is sufficient. A benefit of carrying T1 (and T2) is to allow the AR glasses to distinguish a legitimate return packet from a bogus packet, as is used in the Network Time Protocol (NTP) [4]. Specifically, when the AR glasses transmits a packet, it stores T1 locally. If the T1 in the returning packet doesn’t match the locally stored T1, then the AR glasses consider the returning packet as a bogus packet. It is important to support bogus packet detection and that requires to include T1. Also, T1 can serve as a transaction ID that associate a type-1 RTP header extension and a type-2 RTP header extension. Once T1 is there, we could either include T2 or T2-T1, which are equivalent. T3 must be there for measuring the one-way delay from the EAS to the AR glasses. As a result, the type-2 RTP header extension carries three timestamps. Moreover, the proposed type-2 RTP header extension carrying three timestamps here is in line with the the specifications of the NTP message [4] and the ICMP Timestamp Reply message [5].

Figure 2: In-band end-to-end delay measurement with RTP header extensions

For type-1 RTP header extension, thre are two related works. One of them is RFC 6051 [2], where two RTP header extensions that each carry a single timestamp are specified, with a 56-bits timestamp (with the extension URI “urn:ietf:params:rtp-hdrext:ntp-56” and a 64-bit timestamp (with the extension URI “urn:ietf:params:rtp-hdrext:ntp-64”) respectively. They were originally intended to speed up the synchronization between multiple RTP sessions but can be used for measuring the one-way delay. However, the large number of bits for the timestamps make the communication overheads significant especially when the type-2 RTP header extension adopts the same timestamp format (which will triple the communication overhead) or when delay measurements are taken frequently.

Another related work is the “RTP Header Extension for Absolute Sender Time” RTP header extension [3] that carries only one timestamp (the originate timestamp). The timestamp has 24 bits, taken from the NTP timestamp, with 6 LSB’s of the ‘second’ portion and 18 MSB’s of the ‘fractional’ portion, and its communication overhead is less than the counterparts in RFC 6051. This RTP header extension was developed by Google, but no registry entry was found in the IANA portal. After checking with Google, we found out that this RTP header extension is still implemented in WebRTC, but its use has been abandoned and it has not been registered with IANA. Given its low communication overhead and implementaion in WebRTC, it is advanrageous to reuse this RTP header extension as part of the proposed solution here. One potential issue with this RTP header extension is that the extension URI is an URL [3] does not follow the guidelines in RFC 8285, and another potential potential issue is whether 3GPP/ETSI can register it. Regarding the first potential issu, even if the website associated with the URL is removed, by checking with the IANA registration, there is still sufficient information about the RTP header extension, because the registration has also the “reference” entry that can point to the 3GPP specification which can fully define the RTP header extension. Regarding the second potential issue, we obtained the permission from Google to register the RTP header extension with IANA, whether using its original ABNF synax or a new ABNF syntax. In a companion paper [6], the benefit of reusing the RTP header extension including the original SDP syntax is explained, which is to leverage the support from WebRTC. In this paper we reuse the RTP header extension with the original SDP syntax as proposed in [6].

The proposed changes include the two types of RTP header extensions and the formats. Additionally, it is proposed to bind the two RTP header extensions because the fields T1 and T2 in the second RTP header extension are based on the first RTP header extension.

# Proposed changes

In the Clause 2 References, add the following reference:

# * * * * 1st change * * * *

[10] RTP Header Extension for Absolute Sender Time                     https://webrtc.googlesource.com/src/+/refs/heads/main/docs/native-code/rtp-hdrext/abs-send-time

[11] IETF RFC 5905, Network Time Protocol Version 4: Protocol and Algorithms Specification, June 2010.

[12] RFC 4574, The Session Description Protocol (SDP) Label Attribute, August 2006.

# * * * * End of 1st  change * * * *

In Clause 4, add the following:

# * * * * 2nd change * * * *

### 4.4.3	RTP Header Extension for in-band end-to-end delay measurement

#### 4.4.3.1	General

An RTP Header Extension that allows an RTP packet to carry timestamp(s) may help obtain measured delays that are representative of the end-to-end instantaneous delays experienced by the media in the user plane.

NOTE 1: End-to-end connections may imply in some cases a multi-hop link including non-3GPP network paths, such as a data network link and a tethering link.

Figure 4.4.3.1-1 shows how the RTP Header Extension is used to measure the delays, where T1, T2, T3 and T4 are the Originate Timestamp, the Receive Timestamp, the Transmit Timestamp, and the final timestamp, respectively. The one-way delay from Device A to Device B is calculated as T2 - T1, the one-way delay in the oppositie direction is calculated as T4 – T3, the RTT is calculated as (T4 – T1) – (T3 – T2), and the processing delay on Device B is calculated as T3 – T2.

NOTE 2: NTP synchronization between Device A and Device B is a pre-requisite for computation of one-way delays in any direction.

NOTE 3: Device A may use T1 to group T1, T2, T3, T4 measurements and index them to compute all the above delay measurements and any corresponding statistics.Specific means to achieve this are left to RTP implementers.

Figure 4.4.3.1-1: The RTP header extensions for in-band end-to-end delay measurement.

The RTP Header Extensions defined below follow RFC 8285.

#### 4.4.3.2	One-byte RTP Header Extension Format

The RTP header extension element for the RTP packet that carries ony one timestamp T1 is shown below. This is the same as the “RTP Header Extension for Absolute Sender Time” [10].

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=2   |     Originate Timestamp or T1 (24 bits)       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

The RTP header extension element for the RTP packet that carries three timestamps T1, T2 and T3 is shown below.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=8   |     Originate Timestamp or T1                 |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     Receive Timestamp or T2                   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Transmit Timestamp or T3   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

#### 4.4.3.3	Two-byte RTP Header Extension Format

The RTP header extension element for the RTP packet that carries one timestamp T1 is shown below.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0x100           |appbits|           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID           | L=3           | Originate Timestamp or T1    …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|

+-+-+-+-+-+-+-+-+

The RTP header extension element for the RTP packet that carries three timestamps T1, T2 and T3 is shown below.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0x100           |appbits|           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID           | L=9           | Originate Timestamp or T1  …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|            Receive Timestamp or T2            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     Transmit Timestamp or T3                  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

#### 4.4.3.4	Syntax

T1: consists of 24 bits, taken from the 6 LSB bits of the integer part and the 18 MSB bits of the fractional part of the NTP timestamp format defined in RFC 5905 [11].

T2: follows the syntax of T1.

T3: follows the syntax of T1.

NOTE: The timestamps are expressed in seconds according to the above syntax, with a 64 seconds wraparound and a 3.8 microseconds resolution.

#### 4.4.3.5	Semantics

T1: Originate Timestamp. It represents the time when Device A transmits the RTP packet toward Device B.

T2: Receive Timestamp. It represents the time when Device B receives the RTP packet that carries the Originate Timestamp T1.

T3: Transmit Timestamp. It represents the time when Device B transmits the RTP packet that carries the Originate Timestamp T1, the Receive Timestamp T2, and the Transmit Timestamp T3.

#### 4.4.3.6	SDP signaling

The signaling of the delay measurement RTP header extensions shall follow the SDP signaling design and the syntax and semantics of the "extmap" attribute as outlined in RFC8285.The header extension identifiers shall be registered with IANA.

The ABNF syntax for the “extmap” attribute is as follows:

extmap-attr="a=extmap:" header-ext-ID-1["/" direction] SP http://www.webrtc.org/experiments/rtp-hdrext/abs-send-time [SP format_1]

extmap-attr="a=extmap:" header-ext-ID-2["/" direction] SP urn:3gpp:delay-measurement-3-timestamps:rel-18 [SP format_2] (SP dependent-extmap-ID) [SP processing-ID] [SP m-line-label]

header-ext-ID-1 =1*3DIGIT

header-ext-ID-2 =1*3DIGIT

direction = “sendonly” / “recvonly” / “sendrecv” / “inactive”

format_1 = "short"/“long”

format_2 = "short" / "long"

dependent-extmap-ID = header-ext-ID-1

processing-ID = 1*5DIGIT

m-line-label = 1*(token-char)

The extension attributes have the following semantics:

- header-ext-ID-1: the header extension ID for the first type of RTP header extension, which carries only T1.

- NOTE: http://www.webrtc.org/experiments/rtp-hdrext/abs-send-time is the extension URI for the first type of RTP header extension, and is currently implemented in WebRTC. This extension URI, instead of URN-based ones, allows for support from WebRTC without any change to the WebRTC implementation.

- NOTE: Google gave a written permission to register the first type of RTP header extension with IANA. The “reference” entry in the IANA registry will be “3GPP TS 26.522 [v18.x.x.x]”.

- header-ext-ID-2: the header extension ID for the second type of RTP header extension, which carries T1, T2 and T3.

- format_1: this field is optional. If absent, it indicates that the first type of RTP header extension takes the one-byte format. If present and equal to “short”, it indicates that the first type of RTP header extension takes the one-byte format. If present and equal to “long”, it indicates that the first type of RTP header extension takes the two-byte format.

NOTE: this makes it possible to reuse the “Absolute Sender Time” RTP header extension in WebRTC without changes to the SDP syntax in WebRTC.

- format_2: the semantics are the same as format_1 except that this field is for the second type of RTP header extension.

- processing-ID: identifies a processing module on Device B which takes data carried in RTP packets with the first type of RTP header extension, processes them and produces data carried in RTP packets with the second type of RTP header extension.

NOTE: the details are left to implementation at the application level.

- m-line-label: the SDP “label” attribute defined in RFC 4574 [12], and it identifies a media stream from Device A to Device B and associates the first type of RTP packet header extension in that media stream to the second type of RTP header extension.

- “dependent-extmap-ID = header-ext-ID-1”: this assignment is the last line and it defines the binding between the two types of RTP header extensions. “dependent-extmap-ID” is part of the second line which defines the “extmap” attribute for the second RTP header extension, and “header-ext-ID-1” is used in the first line to identify the first RTP header extension.

# * * * * End of 2nd change * * * *

# References

[1] 3GPP TR 26.806, "Study on Tethering AR Glasses – Architectures, QoS and Media Aspects", V18.0.0 (2023-06).

[2] IETF RFC6051, Rapid Synchronisation of RTP Flows, 2010.

[3] RTP Header Extension for Absolute Send Time, http://www.webrtc.org/experiments/rtp-hdrext/abs-send-time

[4] IETF RFC 5905, Network Time Protocol Version 4: Protocol and Algorithms Specification, June 2010.

[5] IETF RFC 792, Internet Control Message Protocol, 1981.

[6] S4-231836, Adopting the “Absolute Sender Time” RTP header extension in 3GPP, SA4 #126 meeting, Chicago, US, Nov 2023.