Source:	Qualcomm Incorporated

Title:			[MeCAR] A new device type - You see what I see and Audio-Rendering only

Agenda Item:		9.5

Document for:		Agreement

# 1	Introduction

Recent announcements of design AR glasses are reviewed, considering a new device type, or at least some caveats in context of thin AR glasses as defined in TS 26.511.

Despite the efforts from multiple vendors and large brands, smart glasses with displays have remained a niche product. The reason is the complexity of the optics. Karl Guttag on its web page provides some discussion on this matter and provides a view on the issues to be addressed.

A summary of the problem of optical AR is provided here:

The full slide set is attached to this doc, just as a reference. We are not necessarily sharing every detailed opinion, but it gives an excellent idea on the space of AR devices.

# 2	Announcements

On 09/26/2023, Meta announced smart glass with Qualcomm's Snapdragon AR1 Gen 1 processor, which Qualcomm boasts has "on-glass AI" in a sub-one-watt package, and Meta says the battery inside the smart glasses should last between four and six hours of active use. The glass also comes equipped with Meta's new AI assitant and live video streaming . A summary of the technology is for example provided:

- https://www.theverge.com/2023/9/27/23889133/meta-rayban-smart-glasses-price-release-date

- https://www.ray-ban.com/usa/discover-ray-ban-meta-smart-glasses/clp

Some technical highlights:

- The glasses replace your headphones: the smart glasses have a personal audio system

- the specs have five mics, including one in the nose bridge, which should make both your calls and voice commands much clearer.

- The other job of the glasses is as a camera. The smart glasses have small camera lenses on each right temple. These cameras take 12-megapixel photos and 1080p videos. You can store roughly 500 photos and 100 30-second videos (that’s the maximum length the glasses allow) before you fill up the 32GB of internal storage, and everything syncs through the Meta View app.

- In addition to taking photos and videos on the camera, you can also now start a livestream to Facebook or Instagram with just a couple of taps on the stem of the glasses.

- The smart glasses aren’t the screen-having, all-seeing smart glasses.

- It all runs on Qualcomm’s Snapdragon AR1 Gen 1 processor in a sub-one-watt package, and Meta says the battery inside the smart glasses should last between four and six hours of active use. (And when they’re dead, they’re just, you know, glasses.) The included case can also charge the smart glasses another eight times.

- it won’t feature a screen, even though the AR1 would support it. Head-up displays are a much harder problem to solve than putting a camera into glasses.

The Snapdragon AR1 Gen1 provides some technical details that are partially accessible to the Meta Smart Glasses:

- support of displays with a resolution of 1280 x 1280 per eye

- support of three degrees of freedom tracking

- support of dual image signal processor (ISP) for photos and videos based on existing codecs AVC and HEVC.

- The platform also supports up to eight microphones for spatial audio capture

- For optics support of binocular displays that offer more dimensional line-of-sight content including video, wayfinding, and notifications

- Support of on-device AI for dynamic image and audio enhancements and real-time language translation.

Note that the AR1 chips made for more stand-alone smart glasses with cameras for photo and video capture and one or two displays. The AR2 series, meanwhile, features a multichip architecture and is meant for more HoloLens-like immersive augmented reality glasses with support for six degrees of freedom and high-res displays.

# 3	Summary of Technology Features

For real-time media, the following technologies can be assumed:

- Support for 1080p video upstreaming for HEVC and AVC. This matches the HEVC-FullHD-Enc and AVC-Full-HD-Enc capabilities as defined in TS 26.511, but HEVC-HD-Enc and AVC-HD-Enc may be considered as a requirement, whereas Full-HD may be considered as a recommendation.

- If a display is supported, then support for 1280 x 1280 per eye, i.e. resulting to capabilities aligned with HEVC-FullHD-Dec and AVC-Full-HD-Dec.

- Regular audio speech processing with multiple microphone capturing and stereo. The speech and audio capabilities need to be carefully reconsidered

- 3DOF tracking

- Support of on-device AI for dynamic image and audio enhancements and real-time language translation.

# 4	Comparison with Thin AR Glasses

## 10.2	Device type 1: Thin AR glasses

[Editor’s note: For each device type, it is expected that rendering capabilities will also be added before completing the TS.]

### 10.2.1	General

The thin AR glasses device type represents a type of device which is considered to be power-constrained and with limited computing power with respect to the other device types. These limitations typically come from the requirement to design a device with a small and lightweight form factor. Regarding rendering capacity, this device type is expected to rely on remote rendering to be able display complex scenes to the user. For example, such device type may run a split rendering session where the split rendering server delivers pre-rendered views of the scene. However, devices in this category can still operate without external support for applications that do not require complex rendering capabilities, for instance, text messaging, 2D video communication, etc. Lastly, the thin AR glasses offers AR experiences to the user via optical see-through display.

### 10.2.2	XR System support

An XR Device complying to the thin AR glasses device has an XR System with at least the following capabilities:

-	orientationTracking is 'true'

-	positionTracking is 'true'

-	Value 'additive' of the enumeration blendMode

-	Values 'monoscopic' and 'stereoscopic' of the enumeration viewConfigurationPrimary

-	Values 'view', 'local' and 'stage' of the enumeration referenceSpace

-	If swapchainSupported is 'true', numberSwapchainImages is equal to 2

-	Values 'projection' and 'quad' of the enumeration compositionLayer

NOTE:	For the definition of those capabilities, please refer to clause 4.1.3.

[Editor’s note: This list of capabilities is a starting point and more can be added after being defined in clause 4.1.3]

### 10.2.3	Media capabilities support

#### 10.2.3.1	Video

A MeCAR Client of device type 1 shall support decoding capabilities as defined in clause 7.2.1

-	the AVC-UHD-Dec decoding capability.

-	the HEVC-UHD-Dec decoding capability.

-	the AVC-UHD-Dec-4 decoding capability.

-	the HEVC-UHD-Dec-4 decoding capability.

-	the UHD-Dec-4 decoding capability.

A MeCAR Client of device type 1 may support decoding capabilities as defined in clause 7.2.1

-	the HEVC-8K-Dec decoding capability.

-	the 8K-Dec-8 decoding capability.

A MeCAR Client of device type 1 shall support encoding capabilities as defined in clause 7.3.1

-	the AVC-FullHD-Enc encoding capability.

-	the HEVC-FullHD-Enc encoding capability.

A MeCAR Client of device type 1 should support encoding capabilities as defined in clause 7.3.1

-	the HEVC-UHD-Enc encoding capability.

# 4	Proposal

It is proposed to revisit the device classes in TS 26.119 and consider to either adapt the thin glasses to reflect the findings in this document or to create a new class that addresses the issues.

https://kguttag.com/2023/06/03/slides-from-presentation-at-awe-2023-on-optical-versus-passthrough-mr/

https://techcrunch.com/2023/09/27/qualcomm-launches-its-next-gen-chip-for-xr-and-ar-platforms/

https://arinsider.co/2023/09/27/qualcomm-sets-a-new-high-water-mark-in-xr-capabilities/