3GPP TSG SA WG 4 Meeting 126		TDoc S4-231719

Chicago, USA, 13 – 17 November 2023

Title:	LS on Support of interworking between SA4 RTC and IMS

Response to:

Release:	Release 18

Work Item:	FS_eiRTCW

Source:	SA WG 4

To:	SA WG 2

Cc:	CT WG 3

Contact person:			Yoshihiro Inoue

yoshihiro.inoue AT ntt-at DOT co DOT jp

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA4 is addressing the Real-Time media Communication (RTC) architecture and protocols for realizing WebRTC-based immersive real time media communication services on 5G system in the following Release-18 WIs/SI.

- WI (Stage 2): GA4RTAR (SP-220672)

- WI (Stage 3): iRTCW (SP-230977)

- SI: FS_eiRTCW (SP-230166)

In the framework of FS_eiRTCW, SA4 is studying the enhancement on the RTC specifications developed in the above work items. As one of the issues of FS_eiRTCW, SA4 is addressing the issue regarding the interworking between RTC and IMS networks. This solution for the issue is focussing on the interworking of basic & legacy audio call.

A solution for WebRTC-based service has been specified in 3GPP TS 23.228 Annex U, where the WebRTC endpoint can access to IMS network via user-network interface (UNI) by introducing eP-CSCF and eIMS-AGW in the IMS network.

Figure 1: WebRTC IMS architecture and reference model in 3GPP TS 23.228

As another solution, SA4 (FS_eiRTCW) is currently studying a solution that RTC network inter-connect to IMS network as an external IP multimedia network (so-called non-IMS) by applying the interworking model described in 3GPP TS 29.162, with the following motivation:

- For many operators, the existing IMS network is operated as a network for telephony services. Then, the functionalities of IMS network need to comply with regulations for telephony service in the country (e.g., high-availability, support of emergency services, support of lawful interception, high speech quality and high security-level.).

-	On the other hand, most of WebRTC based services are provided on the internet currently, then these services may not need to comply with all aspects of regulation for telephony services. In such case, operators may not choose to enhance the existing IMS network, but to deploy a new RTC network for RTC service and connect the RTC network to IMS network for basic & legacy audio call, with the following aspects:

*	Total cost for realizing RTC service and basic & legacy audio call between RTC and IMS clients.

*	Separation of network operations - avoid that services with different regulatory/reliability requirements need to be handled in the same facility/equipment.

Figure 2: Interworking Model for IM CN Subsystem to IP Multimedia Network in 3GPP TS 29.162

For this NNI-based solution, SA4 is now studying the interworking at the RTC network, as follows:

-	WebRTC specific signalling protocol and media capabilities are terminated by the interworking function at the edge of RTC network.

-	The interworking function defined as a new entity of RTC network connects to IMS network (e.g., IBCF) using SIP and media capabilities comply with IMS specifications via Network to Network Interface (NNI) as described in 3GPP TS 29.162.

SA4 would like to ask SA2 whether the following assumption is correct or not.

-	SA4 assumes that the above NNI-based solution (connecting RTC network to IMS network applying the interworking model specified in 3GPP TS 29.162) has no impact on existing IMS stage 2 specifications, since the solution uses the interface of existing 3GPP IMS specification without any modifications.

2	Actions

To SA2

ACTION: SA4 respectfully asks SA2 to provide feedback to the question.

3	Dates of next TSG SA WG 4 meetings

SA4#127	29th January - 2nd February 2024, 		Sophia Antipolis, France