3GPP TSG-SA WG4 Meeting #126	S4-231731

# Chicago, USA, 13 – 17 November 2023

Source:	NTT

Title:	[FS_eiRTCW] Pseudo-CR on Key Issue #1: Architecture for eiRTCW

Spec:	3GPP TR 26.930

Agenda item:	10.9

Document for:	Agreement

1. Introduction

KI#1 was agreed in FS_eiRTCW Permanent Document v600.

2. Reason for Change

KI#1 needs to be incorporated in TR 26.930 based on the agreement in FS_eiRTCW PD.

3. Proposal

It is proposed to agree on the following changes to 3GPP TR 26.930.

In incorporating the description of FS_eiRCTW PD in TR 26.930, following modifications are deployed.

-	Correction of editorial errors. (e.g., replacement of capital letter with small letter and refining the words)

* * * First Change * * * *

5.2	Key Issue #1: Architecture for eiRTCW

As described in 3GPP TS 26.506 [XX], the detailed scenario and the architecture for collaboration scenario 4 is FFS in this release of this document. This key issue identifies the scenarios and the possible eiRTCW architecture to realize collaboration scenario 4 in addition to collaboration scenario 3, based on the eiRTCW high-level network model described in clause 4.2. These are expected to be specified in 3GPP TS 26.506 [XX].

This key issue includes:

1)	Functional entities and architecture for eiRTCW

-	Study the functional entity and eiRTCW architecture for collaboration scenario 4 in addition to collaboration scenario 3) based on general WebRTC implementation viewpoint.

2)	Possible interaction with 5GC

-	To realize the QoS control, study the interaction between functional entities in the eiRTCW architecture and those in 5GC.

3)	Media connection model

-	Study the target use cases (i.e., connection model) of user plane (U-Plane) and considerations of QoS enabled over End-to-End Path.

4)	IP addressing

-	Study the considerations on IP addressing related issues and identifies the possible additional enhancements of ICE functionality.

5)-	Alignment and gap analysis between the architectures eiRTCW and RTC.

-	Study the alignment between the eiRTCW architecture derived from 1) – 4) and RTC architecture. This also includes gap analysis of functionalities between the architectures eiRTCW and RTC.

6)	Possible RTC architecture for collaboration scenario 4

-	Study the expected architecture variant for the collaboration scenario 4 and enhancements on the existing RTC generic architecture in RTC stage 2 specification (i.e., 3GPP TS 26.506 [XX]), based on the gap analysis studied in 5). This also includes the clarifications on the focused functions and interfaces in this study.

* * * End of Changes * * * *