3GPP TSG-SA4 Meeting #126	S4-231758

# Chicago, USA, 13 – 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | xxxx | rev |  | Current version: | 0.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP] Number of PDUs in PDU Set within RTP header extension |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In case an RTP sender includes a PDU set size within RTP header extension computed for a given IP version (e.g. IPv4), if there is a NAT46/64 in the path between the RTP sender and the RTP receiver, the signaled PDU Set size will not reflect the actual size of the PDU Set at the receiver side. |  |  |  |  |  |  |  |  |
| Summary of change: |  | XRM application provides the number of PDUs in the PDU Set. UPF can correct the PDU Set size when the IP version of the received packets doesn’t coincide with the one signaled by the AF as per TS 23.501/5.3.7.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN may receive wrong PDU-set size information in case an IP version mismatch occurs due to undetected NAT |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.2.4, 4.4.2.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

### 4.4.2	RTP Header Extension for PDU Set Marking

#### 4.4.2.1	General

The RTP Header Extension for PDU Set marking can be used by an AS (e.g., MRF) or a sender UE that sends media to a receiver UE over RTP.

The RTP Header Extension for PDU Set marking shall support both RTP Header Extension formats (i.e., the one-byte and the two-byte formats) according to RFC 8285.

If the RTP Header Extension for PDU Set marking is the only RTP header extension used, the endpoints shall use the 1-byte header format for maximum savings. If other 2-byte RTP header extension elements are used, then the 2-byte header may be used.

NOTE:	The headers are not shown with padding as this depends on other prospective extension elements in use, as per RFC 8285 alignment specifications.

#### 4.4.2.2	One-byte RTP Header Extension Format

The one-byte RTP Header Extension for the marking of PDU Sets and End of Bursts is defined as follows:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=5   |E| EDB |  PSI  |      PSSN         |     PSN   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     PSSize                    |     NPDS

+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

|

+.+.+.+.+.+.+.+.+

#### 4.4.2.3	Two-byte RTP Header Extension Format

The two-byte RTP Header Extension for the marking of PDU Sets and End of Bursts is defined as follows:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0x100         | appbits |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      ID       |      L=6      |E| EDB |  PSI  |      PSSN

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    PSN    |                   PSSize                      |

+-+-+-+-+-+-+-+-+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

|          NPDS                 |

+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

#### 4.4.2.4	Semantics

The semantics of the fields of the RTP Header Extension for the marking of PDU Set and End of Bursts are defined as follows:

-	End PDU of the PDU Set [E] (1 bit): This field is a flag that shall be set to 1 for the last PDU of the PDU Set and set to 0 for all other PDUs of the PDU Set.

-	End of Data Burst [EDB] (3 bits): The EDB field is 3 bits in length and indicates the end of a Data Burst. The 3 bits encode the End of Data Burst indication as per the encoding and guidelines provided in Clause 4.4.2.6.1.

-	PDU Set Importance [PSI] (4 bits): The PDU Set Importance field indicates the importance of this PDU Set compared to other PDU Sets within the same QoS flow. Lower values shall indicate a higher importance PDU Set with the highest importance PDU Set indicated by 0 and the lowest importance PDU Set indicated by 15.

NOTE 1:	A complete set of guidelines for setting the PSI field for various audio/video codecs are provided in Clause 4.4.2.6.2

Editor’s Note: AS/UE are unaware of QoS flows and so the above text needs to be revised to remove the term. The PSI value needs to be set considering one or more RTP streams (depending on multiplexing and other aspects that are under discussion in reference to the guidelines).  Once the guidelines are finalized, the text above will be revised possibly with the introduction of a new term in place for QoS flow that consists of one or more RTP streams.

-	PDU Set Sequence Number [PSSN] (10 bits): The field encodes the sequence number of the PDU Set to which the current PDU belongs acting as a 10-bit numerical identifier for the PDU Set.

NOTE 2:	This value wraps around at 1023, however, using the RTP packet sequence number and PSSN pair a receiver may uniquely distinguish between any PDU Sets.

-	PDU Sequence Number within a PDU Set [PSN] (6 bits): The sequence number of the current PDU within the PDU Set. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU set in order of transmission from the sender.

NOTE 3:	A receiver may use the RTP packet sequence number together with the PSN to distinguish between PDUs within a PDU Set that contains more than 64 PDUs.

-	PDU Set Size [PSSize] (24 bits): The PDU Set Size indicates the total size of all PDUs of the PDU Set to which this PDU belongs. This field is optional and subject to an SDP signaling offer/answer negotiation, where the Application Server may indicate whether it will be able to provide the size of the PDU Set for that RTP stream. If not enabled, the field should not be present. If enabled, but the Application Server is not able to determine the PDU Set Size for a particular PDU Set, it should set the value to 0 in all PDUs of that PDU Set. The PSSize shall indicate the size of a PDU Set including RTP/UDP/IP header encapsulation overhead of its corresponding PDUs. The PSSize is expressed in bytes. It is recommended to add the PDU Set Size field when the Number of PDUs in the PDU Set field is present.

-	Number of PDUs in the PDU Set [NPDS] (16 bits): The number of PDUs within the PDU Set indicates the total number of PDUs belonging to the same PDU Set. This field is optional and subject to an SDP signaling offer/answer negotiation, where the Application Server may indicate whether it will be able to provide the number of PDUs within the PDU Set for that RTP stream. It is recommended to add the Number of PDUs in the PDU Set field when the PDU Set Size field is present.

NOTE 4:	This field may be optionally present given the signaling of the “pdu-set-size” extension attribute in the SDP offer/answer negotiation as per Clause 4.4.2.5.

NOTE 5:	Guidelines to set the PDU Set Size in bytes by an Application Server are provided in Clause 4.4.2.6.3.

NOTE 6:	In order to handle IP version changes in the path between the sender and the receiver, the receiver should know the original IP version and, upon IP version change detection, should correct the PDU Set Size value. When the receiver discovers that the IP version has changed, it can compute the correct PDU Set size by adding or subtracting the difference between IPv6 and IPv4 header size multiplied by the number of PDUs in the PDU Set.

Alternative Phrasing of NOTE 6

NOTE 6:	When the receiver is aware about the used IP version at the sender, IP version changes in the path (e.g. due to a NAT64) can be handled by the receiver. When the receiver detects an IP version change, the receiver should correct the PDU Set Size value before further processing. The receiver can compute the correct PDU Set size by adding or subtracting the difference between IPv6 and IPv4 header size multiplied by the number of PDUs in the PDU Set.

************************************ END OF CHANGES ****************************************