3GPP TSG-WG SA4 Meeting #126	S4-231761

Chicago, US, Nov 13 – 17, 2023	(revision of S4-231xxx)

Source:	Huawei, HiSilicon

Title:	Discussion on the "Protocol Description"

Document for:	Discussion

Agenda Item:	10.7

Work Item / Release:	5G_RTP / Rel-18

Abstract: This paper provides the discussion and analysis on whether and how the “Protocol Description” is defined as the signalling from the RTP sender and the 5GS.

# 1. Introduction

This paper provides the discussion and analysis on whether and how the “Protocol Description” is defined as the signalling from the RTP sender and the 5GS.

# 2. Background

Before the PDU Set identification or the End of Data Burst identification in the PSA UPF, the corresponding information need to be provisioned to UPF. As defined in SA2 specs, the AF provisions the corresponding information to the PCF/NEF using the N5/N33 interface procedure and NEF/PCF/SMF further provides that to the UPF to understand the used protocol and format type. Following the SA2 functional definition, such information is named as "Protocol Description" as following.

-	Protocol Description: Indicates transport protocol (e.g. RTP, SRTP), transport protocol header extensions (e.g. RTP Header Extension for PDU Set Marking as defined in TS 26.522 [179]), payload type and format (e.g. H.264, H.265), and format parameters (e.g. H.264 profile level and packetization mode) used by the service data flow.

However, the above definition is a bit misleading. The payload type, format parameters are not used for the PDU Set identification based on the Guidelines for PDU Set identification without PDU Set RTP HE in Annex A of TS 26.522.

Besides, in the LS S4-230739, we already mentioned that header extension configuration to be shared with UPF will be provided by SA4.

This allows the AF to receive certain PDU set information and pass it along to the PCF/NEF using the N5/N33 interface procedures. The header extension configuration should be shared with the UPF, and SA4 will provide the relevant configuration information to SA2/CT3 for this purpose.

Also, based on the SA2 specs as shown below, SA4 needs to define the "Protocol Description" in detail.

Table 1 PDU Set Control Information in the PCC rule in TS 23.503

| PDU Set Control Information | Information needed to support the delivery of PDU Sets of a service data flow. |  |  |  |
| --- | --- | --- | --- | --- |
| PDU Set QoS Parameters | See clause 5.7.7 of TS 23.501 [2]. |  | Yes | Added |
| Protocol Description | Indicates the protocol to be used by the PSA UPF to detect PDU Sets received via N6 (See TS 23.501 [2] clause 5.37.5 and TS 26.522 [40]). |  | No | Added |

## 3. Protocol Description

To assist UPF for the PDU Set identification and End of Data Burst identification, the following needed to be known to UPF in advance:

-	The protocol used to carry the media traffic, i.e. RTP.

-	When PDU Set based RTP HE is used, identification on this RTP HE. Besides, additional information should be considered:

-	Whether long RTP header or short RTP header is used.

-	Whether the PDU Set Size is available or not.

-	Whether the End of Data Burst is activated or not.

-	When the PDU Set based RTP HE is absent, additional information should be considered based on the Annex A of TS 26.522:

-	payload format, e.g. H.264/AVC, H.265/HEVC.

# 3. Conclusion and proposal(s)

It’s proposed to take this above into account and add the section 3 into the PD for further consideration.