3GPP TSG-SA WG4 Meeting #126	S4-231818

# Chicago, USA, 13 - 17 November 2023											     revision S4aV230080

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.966 | CR | - | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_HEVC_Profiles] Updates on HEVC Multiview coding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_HEVC_Profiles |  |  |  |  |  | Date: |  |  | 30/10/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Support for MV-HEVC in CMAF was noted as FFS. It was noted that there was no support available for MV-HEVC in CMAF. In the meanwhile, MPEG has recently started working on such an extension for CMAF, and the TR is proposed to be updated to this end. Finally, evaluation methodology was needed to be completed which is provided for MV-HEVC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An overview of the updates to MPEG CMAF on MV-HEVC is provided. Evaluation methodology and existing evaluation is provided for MV-HEVC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor's note with no progress on status of support of MV-HEVC in CMAF and on evaluation methodology will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.3, 5.1.4, 6.2.2.2.3, 6.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Evaluation methodology and evaluation provided for MV-HEVC |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ISO/IEC 14496-10:2022: "Information technology — Coding of audio-visual objects — Part 10: Advanced video coding"

[3]	ISO/IEC 23008-2:2015: "Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 2: High efficiency video coding"

[3]	3GPP TR 26.905: "Mobile stereoscopic 3D video".

[4]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[5]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)".

[6]	3GPP TS 26.214: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[7]	3GPP TS 26.218: "Virtual Reality (VR) profiles for streaming applications"

[8]	3GPP TS 26.347: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs"

[9]	Vetro, Anthony. "Frame compatible formats for 3D video distribution." In 2010 IEEE International Conference on Image Processing, pp. 2405-2408. IEEE, 2010.

[10]	Hannuksela, Miska M., Ye Yan, Xuehui Huang, and Houqiang Li. "Overview of the multiview high efficiency video coding (MV-HEVC) standard." In 2015 IEEE International Conference on Image Processing (ICIP), pp. 2154-2158. IEEE, 2015.

[11]	ISO/IEC JTC1/SC29/WG11 MPEG2011 M22746, "AVC/MVC anchor coding for MFC", November 2011, Geneva, Switzerland.

[12]	ISO/IEC JTC1/SC29/WG11 N16050, "MV-HEVC Verification Test Report", San Diego, US, Feb. 2016.

[13]	ISO/IEC 14496-15:2022, "Information technology — Coding of audio-visual objects — Part 15: Carriage of network abstraction layer (NAL) unit structured video in the ISO base media file format"

[14]	"HTTP Live Streaming (HLS) authoring specification for Apple devices," https://developer.apple.com/documentation/http-live-streaming/hls-authoring-specification-for-apple-devices

[15]	"ISO Base Media File Format and Apple HEVC Stereo Video Format additions," Version 0.9 (Beta) June 21, 2023

[16]	"Apple HEVC Stereo Video," Interoperability Profile Version 0.9 (Beta) June 21, 2023

[17]	Delbracio, Mauricio, Damien Kelly, Michael S. Brown, and Peyman Milanfar. "Mobile computational photography: A tour." Annual Review of Vision Science 7 (2021): 571-604.

[18]	Camera & Imaging Products Association (CIPA) "Production, Shipment of Digital Still Camera January, January-January in 2017," 2016

[19]	"Smartphones vs Cameras: Closing the gap on image quality," https://www.dxomark.com/smartphones-vs-cameras-closing-the-gap-on-image-quality/

[20]	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG JVT-I018, "Color format downconversion for test sequence generation," 2003.

[21]	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG JVT-I019, "Color format upconversion for video display," 2003.

[22]	ISO/IEC 23008-12:2022: "Information technology - MPEG systems technologies - Part 12: Image File Format".

[23]	ISO/IEC 14496-12:2022: "Information technology — Coding of audio-visual objects — Part 12: ISO base media file format".

[24]	"Using HEIF or HEVC media on Apple devices," https://support.apple.com/en-us/HT207022

[25]	"HEIF Imaging," https://source.android.com/docs/core/camera/heif

[26]	ITU-T Recommendation T.81: "Information technology; Digital compression and coding of continuous-tone still images: Requirements and guidelines".

[27]	3GPP TR 26.948: "Study on video enhancements in 3GPP multimedia services"

[28]	HTTP Live Streaming (HLS) Authoring Specification for Apple Devices, https://developer.apple.com/documentation/http_live_streaming/http_live_streaming_hls_authoring_specification_for_apple_devices

[29]	Samira Afzal, Vanessa Testoni, Christian Esteve Rothenberg, Prakash Kolan, Imed Bouazizi, “A holistic survey of multipath wireless video streaming”, Journal of Network and Computer Applications, 212: 103581 (2023)

[30]	ISO/IEC JTC1/SC29/WG11 N16051, "SHVC verification test report", February 2016, San Diego, USA.

[31]	ISO/IEC JTC1/SC29/WG11 N16268, "Supplemental SHVC verification test report", June 2016, Geneva, CH.

[xx]	ISO/IEC 23000-19:2020, "Information technology — Multimedia application format (MPEG-A) — Part 19: Common media application format (CMAF) for segmented media"

[xy]	ISO/IEC JTC1/SC29/WG03 N01026, "Preliminary WD of ISO/IEC 23000-19 AMD New Structural CMAF Brand Profile", October 2023, Hannover, Germany.

[xz]	ITU-R/Study Group 6/Document 6/33-E Draft New Recommendation ITU-R BT. "[EVP]: Subjective assessment of video quality using expert viewing protocol (EVP)", 4 February 2016, Geneva.

* * * Next Change * * * *

### 5.1.3	Evaluation criteria and metrics

The evaluation for the coding performance for stereoscopic 3D content needs to be performed based on the following evaluation criteria.

- Assessment/discussion of hardware impact: there are two possibilities for this:

  - There is existing hardware product-grade support for the tool. In that case, refer to the example hardware.

  - There is no existing hardware support. In this case, a discussion/description with justifications on the expected impact on hardware implementation is provided, or reference to existing demos etc.

- Codec performance evaluation

  - PSNR-based Rate-Distortion (RD) objective performance evaluation, where the RD performance is compared for various solutions with a fixed QP encoding setting to get the plotting data points. A better PSNR-based RD performance is preferred, keeping in view the expected hardware complexity impact.

  - Subjective performance evaluation.

### 5.1.4	Evaluation methodology

#### 5.1.4.1	Objective performance evaluation

#### For objective performance evaluation, suitable source test content is identified that is accepted by video experts as representative content. Some of the important parametes for the content are the resolution, framerate, bit depth, color subsampling, and duration, in addition to the number of views available. Reference software for a specific solution is to be used with fixed QP encoding settings to generate each plotting point on the PSNR RD curves. The encoding settings (e.g. prediction types IPP or IBP etc.) are decided by experts considering the complexity and latency needs for the scenario. The resulting curves can directly be used for comparison by plotting together or by comparing the Bjøntegaard Delta (BD) bitrate.

#### 5.1.4.2	Subjective performance evaluation

The suggested methodology for subjective performance evaluation is to follow “Expert Viewing Protocol” (EVP), that has been used in JVET for this purpose. The EVP visual evaluation protocol is standardized in [xz] with the following main features:

- 9 experts participate as viewers in each EVP session,

- The “unimpaired” Source video Clip (SRC) is shown once, followed by two Processed Video Sequences (PVSs),

- Experts are required to compare the PVS with the SRC, and to rate them separately.

* * * Next Change * * * *

##### 6.2.2.2.3	Support in CMAF

Diverse ecosystems such as the ones targeted by MPEG and 3GPP need to enable CMAF (ISO/IEC 23000-19 [xx]) level signalling to convey the unique parameters for Multiview video encoded formats (e.g. how the views may be organized in switching sets, what possibilities are allowed or disallowed, which addressable units are relevant to application for stereoscopic vs. 2D displays, etc.).

As noted in clause 6.2.2.2.1, carriage of MV-HEVC is specified in ISO/IEC 14496-15 [13] (NAL-video file format) as one of the "Layered HEVC (L-HEVC) extensions", including SHVC, MV-HEVC, and 3D-HEVC. Clause 9 of 14496-15 specifies this L-HEVC elementary stream and sample definitions. Despite this, there is a need to enable CMAF-level functionality noted above. Based on this need, MPEG has started working on an MV-HEVC extension of CMAF in [xy].

Editor's note: The progress on the MPEG CMAF preliminary WD needs to be tracked in the TR.

* * * Next Change * * * *

#### 6.2.3.2 Codec performance evaluation based on existing results

The objective and subjective performance results comparing MVC, Simulcast HEVC (each view is coded independently) with MV-HEVC are documented in [12]. The test sequences used for this evaluation are 1080p 8-bit 4:2:0 either 25 or 30 Hz. IPP encoding is used to generate the results. The objective results demonstrate significant performance improvments achieved by MV-HEVC against bith MVC and simulcast HEVC, demonstrated by the Bjøntegaard Delta (BD) bitrates table reproduced here:

| Test Sequence | BD-rate reduction of MV-HEVC [%] relative to |  |
| --- | --- | --- |
|  | MVC | Simulcast HEVC |
| S03: Undo_Dancer | -45.7 | -38.7 |
| S04: GT_Fly | -52.9 | -41.0 |
| S13: Band06 | -43.3 | -31.7 |
| S14: BMX | -60.6 | -25.6 |
| Average | -50.6 | -34.2 |

Hence at least 30% performance gains were observerd against simulcast HEVC. The corresponding subjective tests using “Expert Viewing Protocol” (EVP) verified the objective gains via MOS for all the sequences above.

Although no formal evaluation exists for the Multiview Main 10 profile of MV-HEVC, consideraing the large gains achieved as noted above, it is expected that it’s performance should be similar to what is demonstrated for 8-bit content.

* * * End of Changes * * * *