3GPP TSG-SA WG4 Meeting #126	S4-231863

# Chicago, USA, 13th November 2023 - 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119 | CR | <CR#> | rev | <Rev#> | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on video capabilities definition and device support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 11-08-2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Defining the video capabilities and their support per device type. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are proposed: - AVC and HEVC decoding and encoding capabilities - Concurrent capabilities definition - Support of those video capabilities per device type |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1, 7.1.2, 7.2.1, 10.2.3, 10.3.3, 10.4.3, 10.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE #1

# 7	Visual functions and capabilities

[Ed note: eg description of video formats and codecs, same for GPU capabilities and formats]

## 7.1	Decoding capabilities

### 7.1.1	Single decoder instance

AVC-FullHD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 4.0 [7] bitstreams.

AVC-UHD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 5.1 [7] bitstreams with the following additional requirements:

-	the maximum VCL Bit Rate is constrained to be 120 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively; and,

-	the bitstream does not contain more than 10 slices per picture.

AVC-8K-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 6.1 [7] bitstreams [TBD: possible additional requirements]

[Editor’s note: not present in TS 26.511].

HEVC-FullHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1 [8] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1 and general_frame_only_constraint_flag equal to 1.

HEVC-UHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 5.1 [8] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1 and general_frame_only_constraint_flag equal to 1.

HEVC-8K-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 6.1 [8] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1 with the following additional requirements:

-	the bitstream does not exceed the maximum luma picture size in samples of 33,554,432; and,

-	the maximum VCL Bit Rate is constrained to be 80 Mbps with CpbVclFactor and CpbNalFactor being fixed to be 1000 and 1100, respectively.

### 7.1.2	Concurrent decoder instances

#### 7.1.2.1	Definition

Concurrent video decoder instance are defined as follows.

For N video bitstreams encoded according to a video codec profile, decoding units flow into the coded picture buffer (CPB) for each stream according to a specified arrival schedule and are delivered by the common Hypothetical Stream Scheduler (HSS) that schedules the N bitstreams for decoding each of the units. For each access unit

- all data associated with an access unit is removed and decoded instantaneously by the instantaneous decoding process at CPB removal time of the access unit.

- Each decoded picture is placed in the Decoded Picture Buffer (DPB) for being referenced by the decoding process of this stream as well as for output and cropping.

- A decoded picture is removed from the DPB at the time that it becomes no longer needed for inter-prediction reference as well as the output time of the access unit is the largest of all decoded pictures remaining in the group of N decoders

Then at any point time,

- each of the individual streams conforms to the signaled profile/level/tier and HRD parameters of the individual stream.

- The sum of the CPB size conforms to common profile/level/tier signaling

- The aggregate decoder processing speed (samples per seconds) conforms to common profile/level/tier signaling.

- The sum of the DPB size conforms to common profile/level/tier signaling

- The common DPB size conforms to common profile/level/tier signaling

A set of N concurrent decoder instances conforms to a given capabilities (defined in clause 7.1.2.2), if a set of up to N bitstreams encoded to be decodable by the HRD above, is decodable within the timing limits.

#### 7.1.2.2	Capabilities

AVC-FullHD-Dec-2: The capability of supporting up to two (N=2) concurrent decoder instances with the aggregate capabilities of AVC-FullHD-Dec.

AVC-UHD-Dec-4: The capability of supporting up to four (N=4) concurrent decoder instances with the aggregate capabilities of AVC-UHD-Dec.

HEVC-UHD-Dec-4: The capability of supporting up to four (N=4) concurrent decoder instances with the aggregate capabilities of HEVC-UHD-Dec.

UHD-Dec-4: The capability supporting up to four (N=4) concurrent decoder instances with either:

-	the aggregate capabilities of AVC-UHD-Dec-4;

-	the aggregate capabilities of HEVC-UHD-Dec-4; or,

-	the capability of decoding up to 4 bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-FullHD-Dec or HEVC-FullHD-Dec.

AVC-8K-Dec-8: The capability of supporting up to eight (N=8) concurrent decoder instances with the aggregate capabilities of AVC-8K-Dec.

HEVC-8K-Dec-8: The capability of supporting up to eight (N=8) concurrent decoder instances with the aggregate capabilities of HEVC-8K-Dec.

8K-Dec-8: The capability supporting up to eight (N=8) concurrent decoder instances with either:

-	the aggregate capabilities of AVC-8K-Dec-8;

-	the aggregate capabilities of HEVC-8K-Dec-8; or,

-	the capability of decoding up to:

-	eight bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-FullHD-Dec or HEVC-FullHD-Dec; or,

-	four bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-UHD-Dec or HEVC-UHD-Dec.

## 7.2	Encoding capabilities

### 7.2.1	Single encoder instance

AVC-FullHD-Enc: the capability to encode a video signal to a bitstream that is decodable by a decoder that is AVC-FullHD-Dec capable as defined in clause 7.1.1.1.

HEVC-FullHD-Enc: the capability to encode a video signal to a bitstream that is decodable by a decoder that is HEVC-FullHD-Dec capable as defined in clause 7.1.1.

HEVC-UHD-Enc: the capability to encode a video signal to a bitstream that is decodable by a decoder that is HEVC-UHD-Dec capable as defined in clause 6.1.

CHANGE #2

## 10.2	Device type 1: Thin AR glasses

[Editor’s note: For each device type, it is expected that rendering capabilities will also be added before completing the TS.]

### 10.2.1	General

The thin AR glasses device type represents a type of device which is considered to be power-constrained and with limited computing power with respect to the other device types. These limitations typically come from the requirement to design a device with a small and lightweight form factor. Regarding rendering capacity, this device type is expected to rely on remote rendering to be able display complex scenes to the user. For example, such device type may run a split rendering session where the split rendering server delivers pre-rendered views of the scene. However, devices in this category can still operate without external support for applications that do not require complex rendering capabilities, for instance, text messaging, 2D video communication, etc. Lastly, the thin AR glasses offers AR experiences to the user via optical see-through display.

### 10.2.2	XR System support

An XR Device complying to the thin AR glasses device has an XR System with at least the following capabilities:

-	orientationTracking is 'true'

-	positionTracking is 'true'

-	Value 'additive' of the enumeration blendMode

-	Values 'monoscopic' and 'stereoscopic' of the enumeration viewConfigurationPrimary

-	Values 'view', 'local' and 'stage' of the enumeration referenceSpace

-	If swapchainSupported is 'true', numberSwapchainImages is equal to 2

-	Values 'projection' and 'quad' of the enumeration compositionLayer

NOTE:	For the definition of those capabilities, please refer to clause 4.1.3.

[Editor’s note: This list of capabilities is a starting point and more can be added after being defined in clause 4.1.3]

### 10.2.3	Video capabilities support

An XR Device complying to the thin AR glasses device shall support at least the following capabilities:

- AVC-FullHD-Dec

- AVC-FullHD-Dec-2

- AVC-FullHD-Enc

An XR Device complying to the thin AR glasses device should support the following capabilities:

- HEVC-FullHD-Dec

- HEVC-FullHD-Enc

CHANGE #3

## 10.3	Device type 2: AR glasses

### 10.3.1	General

The AR glasses device type represents a type of device which is considered to have higher computation power compared to the thin AR glasses device type. As a result, this device type has higher rendering capacities and is generally expected to be capable of rendering scenes without external support, even though remote rendering is not precluded to lower the power consumption on the device or enable the display of scenes beyond the device’s rendering capability. Lastly, the AR glasses offers AR experiences to the user via optical see-through display.

### 10.3.2	XR System support

An XR Device complying to the AR glasses device type offers an XR System with at least the following capabilities:

-	orientationTracking is 'true'

-	positionTracking is 'true'

-	Value 'additive' of the enumeration blendMode

-	Value 'stereoscopic' of the enumeration viewConfigurationPrimary

-	Values 'view', 'local' and 'stage' of the enumeration referenceSpace

-	If swapchainSupported is 'true', numberSwapchainImages is equal to 2

-	Values 'projection' and 'quad' of the enumeration compositionLayer

NOTE:	For the definition of those capabilities, please refer to clause 4.1.3.

[Editor’s note: This list of capabilities is a starting point and more can be added after being defined in clause 4.1.3]

### 10.3.3	Video capabilities support

An XR Device complying to the AR glasses device shall support at least the following capabilities:

- AVC-UHD-Dec

- AVC-UHD-Dec-4

- HEVC-UHD-Dec

- AVC-FullHD-Enc

- HEVC-FullHD-Enc

An XR Device complying to the AR glasses device should support the following capabilities:

- HEVC-UHD-Dec-4

- UHD-Dec-4

CHANGE #4

## 10.4	Device type 3: XR phone

### 10.4.1	General

The XR phone device type represents a type of device which corresponds to a smartphone with capacities and resources sufficient to offer AR experiences. As a result, this device type is capable of rendering scenes without external support. Lastly, the XR phone offers AR experiences to the user via video see-through display.

### 10.4.2	XR System support

An XR Device complying to the XR phone device type offers an XR System with at least the following capabilities:

-	orientationTracking is 'true'

-	positionTracking is 'true'

-	Values 'opaque' and 'alpha_blend' of the enumeration blendMode

-	Values 'monoscopic' and 'stereoscopic' of the enumeration viewConfigurationPrimary

-	Values 'view', 'local' and 'stage' of the enumeration referenceSpace

-	If swapchainSupported is 'true', numberSwapchainImages equal to 2

-	Values 'projection' and 'quad' of the enumeration compositionLayer

NOTE:	For the definition of those capabilities, please refer to clause 4.1.3.

[Editor’s note: This list of capabilities is a starting point and more can be added after being defined in clause 4.1.3]

### 10.4.3	Video capabilities support

An XR Device complying to the XR phone device shall support at least the following capabilities:

- AVC-UHD-Dec

- AVC-UHD-Dec-4

- HEVC-UHD-Dec

- HEVC-UHD-Dec-4

- UHD-Dec-4

- AVC-UHD-Enc

- HEVC-UHD-Enc

An XR Device complying to the XR phone device should support the following capabilities:

- AVC-8K-Dec

- HEVC-8K-Dec

- 8K-Dec-8

CHANGE #5

## 10.5	Device type 4: XR HMD

### 10.5.1	General

The XR HMD device type represents a type of device which corresponds to HMDs capable of offering at least AR experiences but not precluding other types of XR experiences. This device type is expected to be capable of rendering scenes without external support. Lastly, the XR phone offers AR experiences to the user via video see-through display.

### 10.5.2	XR System support

An XR Device complying to the XR HMD device type offers an XR System with at least the following capabilities:

-	orientationTracking is 'true'

-	positionTracking is 'true'

-	Value 'additive' or values 'opaque' and 'alpha_blend' of the enumeration blendMode

-	Values 'monoscopic' and 'stereoscopic' of the enumeration viewConfigurationPrimary

-	Values 'view', 'local' and 'stage' of the enumeration referenceSpace

-	If swapchainSupported is 'true', numberSwapchainImages is equal to 2

-	Values 'projection' and 'quad' of the enumeration compositionLayer

NOTE	For the definition of those capabilities, please refer to clause 4.1.3.

[Editor’s note: This list of capabilities is a starting point and more can be added after being defined in clause 4.1.3]

### 10.5.3	Video capabilities support

An XR Device complying to the XR HMD device shall support at least the following capabilities:

- AVC-UHD-Dec

- AVC-UHD-Dec-4

- HEVC-UHD-Dec

- HEVC-UHD-Dec-4

- UHD-Dec-4

- AVC-UHD-Enc

HEVC-UHD-Enc

An XR Device complying to the XR HMD device should support the following capabilities:

- AVC-8K-Dec

- HEVC-8K-Dec

- 8K-Dec-8

END OF CHANGES