Source:	Orange

Title:	Comments on ATIAS

## Document for:	Discussion

## Agenda Item:	7.6

# Introduction

This contribution aims at triggering some discussion to prepare the finalization of the ATIAS work item. According to [1], ATIAS will produce a new specification in TS 26.259 (requirements) and an update of TS 26.260 (test methods). Ideally preliminary drafting of these outputs should be initiated at SA4#126.

This contribution also brings some considerations on binaural rendering tests.

# Codec operation and structure of requirements

The ATIAS work item depends on the features supported by IVAS. One may note that mono operation is already covered by 3GPP TS 26.131 and 26.132 and this should be made explicit. Furthermore, stereo operation is in the scope of TS 26.131 and 26.132, however the following note was inserted in Rel-12 of both specifications:

- The interfaces are shown for one-channel (mono) operation, interfaces for two-channel (stereo) operation is for further study (see clause 4 of TS 26.131)

- The configurations are shown for one-channel (mono) operation, configurations for two-channel (stereo) operation is for further study (see clause 5 of TS 26.132)

Based on the outcome of ATIAS, this existing text should be updated in these specifications.

One question for ATIAS to discuss is whether all combinations (input/output formats, acoustic and electrical interfaces…) should be tested and if TS 26.259 can be organized in a practical way to match typical terminal tests.

There are dependencies on the IVAS RTP payload format. For instance, it is not yet clear whether a selected input format can be forced at the call setup. Furthermore, some technical characterization of IVAS may be missing to complete the definition of requirements, therefore one may anticipate that at least some requirements may have to be kept in brackets until characterization is completed.

One should discuss the most reasonable way to organize requirements and revisit the structure of TS 26.259. TS 26.131 is organized by bandwidths and test cases with further divisions by interfaces. TS 26.259 should be organized to facilitate terminal testing and usage by “external” test specifications.

# Extension of 3GPP TS 26.260

SA4 will soon consider the transfer of candidate requirements and test methods in [2]  into output ATIAS specifications. Given the existing structure and content of TS 26.260 one should discuss how to extend TS 26.260 with test methods derived from [2].

Note that IVAS is being considered in MeCAR for XR devices, this type of devices should now be described in test setups.

# Considerations on receiver tests

It is essential to define a set of receiver tests that can ensure sufficient terminal audio quality when binaural rendering is configured. Such tests may be based on an acoustic interface (e.g., HATS) when the UE includes a binaural headet or an electrical interface.

The IVAS codec supports a default set of HRIR and BRIR filters for testing (see motivations in [3]). This default set is natively used by IVAS when no filter set is provided through the binaural rendering interface. TS 26.260 should refer to this default set to have more control on test conditions and provide a known quality reference point.

We suggest considering proposals for objective testing from [4] in the scope of ATIAS.

## References

- SP-190040, WID on Terminal Audio quality performance and Test methods for Immersive Audio Services

- S4-231418, ATIAS Pdoc v0.5.0, Source: Editor (Dolby)

- S4-221027, On binaural rendering, Source: Orange

- S4-231117, IVAS Room Acoustics Testing Methodologies, Source: Orange, Philips, Qualcomm