3GPP TSG SA WG4#126	Tdoc S4-231936

Chicago, USA, 13-17 November 2023	revision of S4-231625

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | 0558 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Changes to support Multiparty RTT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CMCC, Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MP_RTT |  |  |  |  |  | Date: |  |  | 2023-10-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | MP_RTT WI is progressing towards completion soon and this CR reflects the solutions captured in the PD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Multiparty RTT solutions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete solution for MP_RTT. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex XX(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[3]	3GPP TS 26.235: "Packet switched conversational multimedia applications; Default codecs".

[4]	3GPP TS 26.236: "Packet switched conversational multimedia applications; Transport protocols".

[5]	3GPP TR 26.914: "Multimedia telephony over IP Multimedia Subsystem (IMS); Optimization opportunities".

[6]	3GPP TR 22.973: "IMS Multimedia Telephony service; and supplementary services".

[7]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[8]	IETF RFC 4566 (2006): "SDP: Session Description Protocol", M. Handley, V. Jacobson and C. Perkins.

[9]	IETF RFC 3550 (2003): "RTP: A Transport Protocol for Real-Time Applications", H. Schulzrinne, S. Casner, R. Frederick and V. Jacobson.

[10]	IETF RFC 3551 (2003): "RTP Profile for Audio and Video Conferences with Minimal Control", H. Schulzrinne and S. Casner.

[11]	3GPP TS 26.071: "Mandatory Speech Codec speech processing functions; AMR Speech CODEC; General description".

[12]	3GPP TS 26.090: "Mandatory Speech Codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Transcoding functions".

[13]	3GPP TS 26.073: "ANSI C code for the Adaptive Multi Rate (AMR) speech codec".

[14]	3GPP TS 26.104: "ANSI-C code for the floating-point Adaptive Multi Rate (AMR) speech codec".

[15]	3GPP TS 26.093: "Mandatory speech codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Source controlled rate operation".

[16]	3GPP TS 26.103: "Speech codec list for GSM and UMTS".

[17]	3GPP TS 26.171: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; General description".

[18]	3GPP TS 26.190: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; Transcoding functions".

[19]	3GPP TS 26.173: "ANCI-C code for the Adaptive Multi Rate - Wideband (AMR-WB) speech codec".

[20]	3GPP TS 26.204: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; ANSI-C code".

[21]	3GPP TS 26.193: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; Source controlled rate operation".

[22]	Void.

[23]	Void.

[24]	Recommendation ITU-T H.264 (04/2017): "Advanced video coding for generic audiovisual services" | ISO/IEC 14496-10:2014: "Information technology – Coding of audio-visual objects – Part 10: Advanced Video Coding".

[25]	IETF RFC 6184 (2011): "RTP Payload Format for H.264 Video", Y.-K. Wang, R. Even, T. Kristensen, R. Jesup.

[26]	ITU-T Recommendation T.140 (02/1998): "Protocol for multimedia application text conversation".

[27]	ITU-T Recommendation T.140 (02/2000): "Protocol for multimedia application text conversation - Addendum 1".

[28]	IETF RFC 4867 (2007): "RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs", J. Sjoberg, M. Westerlund, A. Lakaniemi and Q. Xie.

[29]	Void

[30]	Void.

[31]	IETF RFC 4103 (2005): "RTP Payload for Text Conversation", G. Hellstrom and P. Jones.

[32]	Void.

[33]	3GPP TR 25.993: "Typical examples of Radio Access Bearers (RABs) and Radio Bearers (RBs) supported by Universal Terrestrial Radio Access (UTRA)".

[34]	3GPP TS 22.105: "Services and service capabilities".

[35]	3GPP TS 26.131: "Terminal acoustic characteristics for telephony; Requirements".

[36]	3GPP TS 26.132: "Speech and video telephony terminal acoustic test specification".

[37]	3GPP TS 28.062: "Inband Tandem Free Operation (TFO) of speech codecs; Service description; Stage 3".

[38]	3GPP TS 23.153: "Out of band transcoder control; Stage 2".

[39]	IETF RFC 0768 (1980): "User Datagram Protocol", J. Postel.

[40]	IETF RFC 4585 (2006): "Extended RTP Profile for Real-time Transport Control Protocol (RTCP) - Based Feedback (RTP/AVPF)", J. Ott, S. Wenger, N. Sato, C. Burmeister and J. Rey.

[41]	RTP Tools: http://www.cs.columbia.edu/IRT/software/rtptools/.

[42]	IETF RFC 3556 (2003): "Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth", S. Casner.

[43]	IETF RFC 5104 (2008): "Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF)", S. Wenger, U. Chandra, M. Westerlund and B. Burman.

[44]	Void.

[45]	3GPP TS 26.111: "Codec for circuit switched multimedia telephony service; Modifications to H.324".

[46]	3GPP TS 23.172: "Technical realization of Circuit Switched (CS) multimedia service; UDI/RDI fallback and service modification; Stage 2".

[47]	3GPP TS 23.002: "Network Architecture".

[48]	IETF RFC 3388 (2002): "Grouping of Media Lines in the Session Description Protocol (SDP)", G. Camarillo, G. Eriksson, J. Holler and H. Schulzrinne.

[49]	IETF RFC 4102 (2005): "Registration of the text/red MIME Sub-Type", P. Jones.

[50]	ITU-T H.248 (06/2000): "Packages for text conversation, fax and call discrimination".

[51]	ETSI EG 202 320, v1.2.1 (2005-10): "Human Factors (HF); Duplex Universal Speech and Text (DUST) communications".

[52]	3GPP TS 26.226: "Cellular text telephone modem; General description".

[53]	IETF RFC 4504 (2006): "SIP Telephony Device Requirements and Configuration", H. Sinnreich, Ed., S. Lass and C. Stredicke.

[54]	ITU-T Recommendation V.151 (05/2006): "Procedures for end-to-end connection of analogue PSTN text telephones over an IP network utilizing text relay".

[55]	ITU-T Recommendation V.152 (09/2010): "Procedures for supporting Voice Band Data over IP networks".

[56]	IETF RFC 3448 (2003): "TCP Friendly Rate Control (TFRC): Protocol Specification", M. Handley, S. Floyd, J. Padhye and J. Widmer.

[57]	3GPP TS 24.173: "IMS Multimedia Telephony Communication Service and Supplementary Services".

[58]	IETF RFC 3264 (2002): "An Offer/Answer Model with the Session Description Protocol (SDP)", J. Rosenberg and H. Schulzrinne.

[59]	3GPP TS 26.141: "IP Multimedia System (IMS) Messaging and Presence; Media formats and codecs".

[60]	3GPP TS 26.234: "Transparent end-to-end Packet-switched Streaming Service; Protocols and codecs".

[61]	IETF RFC 4733 (2006): "RTP Payload for DTMF Digits, Telephony Tones, and Telephony Signals", H. Schulzrinne and T.Taylor.

[62]	3GPP TS 23.014: "Support of Dual Tone Multi-Frequency (DTMF) signalling".

[63]	ETSI ES 201 235-2, v1.2.1: "Specification of Dual Tone Multi-Frequency (DTMF); Transmitters and Receivers; Part 2: Transmitters".

[64]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[65]	3GPP TS 29.163: "Interworking between the IP Multimedia (IM) Core Network (CN) subsystem and Circuit Switched (CS) networks".

[66]	Void.

[67]	OMA-ERELD-DM-V1_2-20070209-A: "Enabler Release Definition for OMA Device Management, Approved Version 1.2".

[68]	Void.

[69]	IETF RFC 5939 (2010): "Session Description Protocol (SDP) Capability Negotiation", F. Andreasen.

[70]	Void

[71]	IETF RFC 1952 (May 1996): "GZIP file format specification version 4.3", P. Deutsch.

[72]	IETF RFC 2326 (1998): "Real Time Streaming Protocol (RTSP)".

[73]	IETF RFC 2616 (June 1999): "Hypertext Transfer Protocol -- HTTP/1.1".

[74]	3GPP TS 26.346 "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[75]	Void

[76]	IETF RFC 6236 (2011): "Negotiation of Generic Image Attributes in the Session Description Protocol (SDP)", I. Johansson and K. Jung.

[77]	ITU-T G.711 (11/1988): "Pulse code modulation (PCM) of voice frequencies".

[78]	ITU-T G.722 (09/2012): "7 kHz audio-coding within 64 kbit/s".

[79]	IETF RFC 4821 (2007): "Packetization Layer Path MTU Discovery".

[80]	3GPP TS 23.003: "Numbering, addressing and identification".

[81]	IETF RFC 4796 (2007): "The Session Description Protocol (SDP) Content Attribute", J. Hautakorpi and G. Camarillo.

[82]	3GPP TS 24.247: "Messaging service using the IP Multimedia (IM) Core Network (CN) subsystem".

[83]	IETF RFC 3168 (2001): "The Addition of Explicit Congestion Notification (ECN) to IP", K. Ramakrishnan, S. Floyd and D. Black.

[84]	IETF RFC 6679 (2012): "Explicit Congestion Notification (ECN) for RTP over UDP", M. Westerlund, et. al.

[85]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description".

[86]	Void

[87]	IETF RFC 5506 (2009):  "Support for Reduced-Size Real-Time Transport Control Protocol (RTCP): Opportunities and Consequences".

[88]	IETF RFC 3611 (2003): "RTP Control Protocol Extended Reports (RTCP XR) ", T. Friedman, R. Caceres and A. Clark.

[89]	3GPP TS 25.401: "UTRAN overall description".

[90]	3GPP TS 23.203: "Policy and charging control architecture".

[91]	ITU-T Recommendation T.4 (07/2003): "Standardization of Group 3 facsimile terminals for document transmission".

[92]	ITU-T Recommendation T.30 (09/2005): "Procedures for document facsimile transmission in the general switched telephone network".

[93]	ITU-T Recommendation T.38 (09/2010): "Procedures for real-time Group 3 facsimile communication over IP networks".

[94]	IETF RFC 3362 (2002): "Real-time Facsimile (T.38) - image/t38 MIME Sub-type Registration".

[95]	IETF RFC 5285 (2008): "A General Mechanism for RTP Header Extensions", D. Singer, H. Desineni.

[96]	IETF RFC 5168 (2008): "XML Schema for Media Control", O. Levin, R. Even and P. Hagendorf.

[97]	3GPP2 C.S0055-A, version 1.0: "Packet Switched Video Telephony Service (PSVT/MCS)".

[98]	ETSI TS 181 005, v3.3.1: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Service and Capability Requirements".

[99]	3GPP2 C.S0014-E, version 1.0: "Enhanced Variable Rate Codec (EVRC)".

[100]	ITU-T Recommendation G.729 (06/2012): "Coding of speech at 8 kbit/s using conjugate-structure algebraic-code-excited linear prediction (CS-ACELP)".

[101]	ITU-T Recommendation G.729.1 (05/2006): "G.729-based embedded variable bit-rate coder: An 8-32 kbit/s scalable wideband coder bitstream interoperable with G.729".

[102]	3GPP2 C.S0076, version 1.0: "Discontinuous Transmission (DTX) of Speech in cdma2000 Systems".

[103]	IETF RFC 5188 (2008):"RTP Payload Format for the Enhanced Variable Rate Wideband Codec (EVRC-WB) and the Media Subtype Updates for EVRC-B Codec".

[104]	IETF RFC 4749 (2006): "RTP Payload Format for the G.729.1 Audio Codec".

[105]	IETF RFC 5459 (2009): "G.729.1 RTP Payload Format Update: Discontinuous Transmission (DTX) Support".

[106]	IETF RFC 4788 (2007): "Enhancements to RTP Payload Formats for EVRC Family Codecs".

[107]	IETF RFC 4855 (2007): "Media Type Registration of RTP Payload Formats".

[108]	ITU-T Recommendation P.10 (07/2006): "Vocabulary and effects of transmission parameters on customer opinion of transmission quality".

[109]	ETSI TS 103 737, v1.1.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for narrowband wireless terminals (handset and headset) from a QoS perspective as perceived by the user".

[110]	ETSI TS 103 738, v1.1.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for narrowband wireless terminals (handsfree) from a QoS perspective as perceived by the user".

[111]	ETSI TS 103 739, v1.1.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for wideband wireless terminals (handset and headset) from a QoS perspective as perceived by the user".

[112]	ETSI TS 103 740, v1.1.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for wideband wireless terminals (handsfree) from a QoS perspective as perceived by the user".

[113]	ETSI TS 202 737, v1.3.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for narrowband VoIP terminals (handset and headset) from a QoS perspective as perceived by the user".

[114]	ETSI TS 202 738, v1.3.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for narrowband VoIP loudspeaking and handsfree terminals from a QoS perspective as perceived by the user".

[115]	ETSI TS 202 739, v1.3.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for wideband VoIP terminals (handset and headset) from a QoS perspective as perceived by the user ".

[116]	ETSI TS 202 740, v1.3.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for wideband VoIP loudspeaking and handsfree terminals from a QoS perspective as perceived by the user ".

[117]	ETSI EN 300 175-8, v2.5.1: "Digital Enhanced Cordless Telecommunications (DECT); Common Interface (CI); Part 8: Speech and audio coding and transmission".

[118]	ETSI TS 300 176-2, v2.2.1: "Digital Enhanced Cordless Telecommunications (DECT); Test specification; Part 2: Audio and speech".

[119]	Recommendation ITU-T H.265: "High efficiency video coding" | ISO/IEC 23008-2:2020: "High Efficiency Coding and Media Delivery in Heterogeneous Environments – Part 2: High Efficiency Video Coding". [120]	IETF RFC 7798 (2016): "RTP Payload Format for High Efficiency Video Coding (HEVC)", Y.-K. Wang, Y. Sanchez, T. Schierl, S. Wenger, M. M. Hannuksela.

[121]	3GPP TS 26.441: "Codec for Enhanced Voice Services (EVS); General Overview".

[122]	3GPP TS 26.442: "Codec for Enhanced Voice Services (EVS); ANSI C code (fixed-point)".

[123]	3GPP TS 26.443: "Codec for Enhanced Voice Services (EVS); ANSI C code (floating-point)".

[124]	3GPP TS 26.444: "Codec for Enhanced Voice Services (EVS); Test Sequences".

[125]	3GPP TS 26.445: "Codec for Enhanced Voice Services (EVS); Detailed Algorithmic Description".

[126]	3GPP TS 26.446: "Codec for Enhanced Voice Services (EVS); AMR-WB Backward Compatible Functions".

[127]	3GPP TS 26.447: "Codec for Enhanced Voice Services (EVS); Error Concealment of Lost Packets".

[128]	3GPP TS 26.448: "Codec for Enhanced Voice Services (EVS); Jitter Buffer Management".

[129]	3GPP TS 26.449: "Codec for Enhanced Voice Services (EVS); Comfort Noise Generation (CNG) Aspects".

[130]	3GPP TS 26.450: "Codec for Enhanced Voice Services (EVS); Discontinuous Transmission (DTX)".

[131]	3GPP TS 26.451: "Codec for Enhanced Voice Services (EVS); Voice Activity Detection (VAD)".

[132]	3GPP TS 45.003: "Radio Access Network; Channel coding".

[133]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage2".

[134]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage2".

[135]	ITU-T Recommendation H.224 (01/05): "A real time control protocol for simplex applications using the H.221 LSD/HSD/MLP channels ".

[136]	ITU-T Recommendation H.224 (2005): Corrigendum 1 (08/07).

[137]	ITU-T Recommendation H.281 (11/94): Transmission of non-telephone signals "A far end camera control protocol for videoconferences using H.224".

[138]	ITU-T Recommendation H.323 (12/2009): "Packet-based multimedia communications systems".

[139]	IETF RFC 4573 (2006): "MIME Type Registration for RTP Payload Format for H.224".

[140]	IETF RFC 4588 (2006): "RTP Retransmission Payload Format", J. Rey, D. Leon, A. Miyazaki, V. Varsa and R. Hakenberg.

[141]	IETF RFC 8627 (2019): "RTP Payload Format for Flexible Forward Error Correction (FEC)".

[142]	TR 26.922:  "Video Telephony Robustness Improvements Extensions (VTRI_EXT): Performance Evaluation".

[143]	IETF RFC 5956 (2010): "Forward Error Correction Grouping Semantics in the Session Description Protocol", A. Cengiz.

[144]	3GPP TR 26.924: "Multimedia telephony over IP Multimedia Subsystem (IMS); Study on improved end-to-end Quality of Service (QoS) handling for Multimedia Telephony Service for IMS (MTSI)".

[145]	Void

[146]	Void.

[147]	3GPP TS 24.147: "Conferencing Using IP Multimedia Core Network; Stage 3".

[148]	IETF RFC 4575 (2006): "A Session Initiation Protocol (SIP) Event Package for Conference State".

[149]	IETF RFC 4582 (2006): "The Binary Floor Control Protocol (BFCP)".

[150]	IETF RFC 4583 (2006): "Session Description Protocol (SDP) Format for Binary Floor Control (BFCP) Streams".

[151]	Void.

[152]	3GPP TR 26.980: "Multimedia telephony over IP Multimedia Subsystem (IMS); Media handling aspects of multi-stream multiparty conferencing for Multimedia Telephony Service for IMS (MTSI)".

[153]	IETF RFC 5234 (2008): "Augmented BNF for Syntax Specifications: ABNF", D. Crocker and P. Overell.

[154]	IETF RFC 8853 (2021): "Using Simulcast in Session Description Protocol (SDP) and RTP Sessions"

[155]	IETF RFC 8851 (2021): "RTP Payload Format Restrictions"

[156]	IETF RFC 7728 (2016): "RTP Stream Pause and Resume".

[157]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[158]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol specification".

[159]	"Mobile Location Protocol (MLP)", Open Mobile Alliance, OMA-LIF-MLP-V3_1, Approved Version 3.1 – 20 Sep 2011.

[160]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[161]	3GPP TS 27.007: " Technical Specification Group Core Network and Terminals; AT command set for User Equipment (UE)".

[162]	Void

[163]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[164]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[165]	3GPP TS 26.452: "Codec for Enhanced Voice Services (EVS); ANSI C code; Alternative fixed-point using updated basic operators".

[166]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[167]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[168]	3GPP TR 26.952: "Codec for Enhanced Voice Services (EVS); Performance characterization".

[169]	3GPP TR 26.959: "Study on enhanced Voice over LTE (VoLTE) performance".

[170]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[171]	3GPP TS 37.324: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Service Data Adaptation Protocol (SDAP) specification".

[172]	IETF RFC 8864 (2021): "Negotiation Data Channels Using the Session Description Protocol (SDP)"

[173]	IETF RFC 4960 (2007): "Stream Control Transmission Protocol"

[174]	IETF RFC 8261 (2017): "Datagram Transport Layer Security (DTLS) Encapsulation of SCTP Packets"

[175]	IETF RFC 8831 (2021): "WebRTC Data Channels".

[176]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[177]	IETF RFC 5688 (2010): "A Session Initiation Protocol (SIP) Media Feature Tag for MIME                    Application Subtypes".

[178]	3GPP TS 28.405; "Management of Quality of Experience (QoE) measurement collection; Control and configuration"

[179]	ISO/IEC 23090-2:2019: " Information technology -- Coded representation of immersive media -- Part 2: Omnidirectional media format".

[180]		3GPP TS 26.118: "3GPP Virtual reality profiles for streaming applications".

[181]	ITU-T Recommendation G.1028 (06/2019): "End-to-end quality of service for voice over 4G mobile networks".

[182]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[183]		ISO/IEC 23090-14: Information technology — Coded representation of immersive media — Part 14: Scene Description for MPEG Media.

[184]	IETF RFC 8839 (2021): "Session Description Protocol (SDP) Offer/Answer Procedures for Interactive Connectivity Establishment (ICE)".

[185]	IETF RFC 9071(2021): “RTP-Mixer Formatting of Multiparty Real-Time Text”

Second CHANGE

### 6.2.4	Text

An MTSI client should offer AVP for all media streams containing text. Only in cases where there is an explicit demand for the AVPF RTCP reporting timing or feedback messages AVPF shall be used. If AVPF is offered then RTP profile negotiation shall be done as described in clause 6.2.1a.

Examples of SDP offers for text can be found in clause A.5.

An MTSI client supporting multiparty real-time text shall indicate this by including the "a=rtt-mixer" attribute in SDP.

An MTSI client configured to automatically enable global text telephony (GTT), e.g. because the MTSI client is used by a deaf or hearing-impaired person or a person wanting to communicate with such an impaired person, shall accept an initial INVITE request for a SIP dialogue if the SDP offer does not include real time text media. It shall then send a new SDP offer (e.g. in a SIP UPDATE request during call establishment) adding text media for real time text conversation.

NOTE:	As one example, incoming calls from a PSTN interworked by an MGCF will not contain media for real time text conversation in the initial SDP offer. The new offer adding media for real time text conversation enables the transport of real time text towards the MTSI client.

Third CHANGE

### 7.4.4	Real-time text

The following RTP payload format shall be used:

-	T.140 text conversation RTP payload format according to RFC 4103 [31] including the updates from RFC 9071 [185] when the negotiation for support of multiparty real-time text is successful.

Real-time text shall be the only payload type in its RTP stream because the RTP sequence numbers are used for loss detection and recovery. The redundant transmission format shall be used for keeping the effect of packet loss low.

Media type signalling for usage in SDP is specified in section 10 of RFC 4103 [31], section 3 of RFC 4102 [49] and section 2.3 of RFC 9071 [185].

Negotiation of support for mixing real-time text for multiparty-aware MTSI clients shall be done by using "a=rtt-mixer" to SDP attribute specified in RFC 9071. When the negotiation fails in a multiparty call, mixing for multiparty unaware endpoints shall be done by a mixer capable of handling multiparty mixing of real-time text as specified in RFC 9071 [185].

Note: multiparty unaware endpoints is define in RFC9071[185].

FOURTH CHANGE

#### 7.5.2.3	Text

Real-time text is intended for human conversation applications. Text shall not be transferred with higher rate than 30 characters per second (as defined for cps in section 6 of RFC 4103 [31]). A text-capable MTSI client shall be able to receive text with cps set up to 30 from an MTSI client in terminal. A text-capable MTSI client may be able to receive text with cps set up to 90 in order to be prepared to handle text received in a multiparty session, with multiple parties sending at a rate of 30 characters per second. This shall be indicated by the cps parameter in the sdp fmtp attribute for the "t140" format.

FIFTH CHANGE

## 9.4	Text

Redundant transmission provided by the RTP payload format as described in RFC 4103 [31] shall be supported. When the negotiation for support of multiparty real-time text is successful, redundant transmission provided by the RTP payload format as described in IETF RFC 9071 [185] shall also be supported. The transmitting application may use up to 200 % redundancy, i.e. a T140block transported in one RTP packet may be repeated once or twice in subsequent RTP packets. 200 % redundancy shall be used when the conditions along the call path are not known to be free of loss. However, the result of media negotiation shall be followed, and transmission without redundancy used if one of the parties does not show capability for redundancy.

The sampling time shall be 300 ms as a minimum (in order to keep the bandwidth down) and should not be longer than 500 ms. New text after an idle period shall be sent as soon as possible. The first packet after an idle-period shall have the M-bit set.

The procedure described in section 5 of RFC 4103 [31], or a procedure with equivalent or better performance, shall be used for packet-loss handling in the receiving MTSI client in terminal.

SIXTH CHANGE

# A.5	SDP offers and answers for text

## A.5.1	T.140 with and without redundancy

An offer to use T.140 real-time text may be realized by using SDP according to the following example in session setup or for addition of real-time text during a session.

Table A.5.1: Example SDP offer for T.140 real-time text

SDP offer

m=text 53490 RTP/AVP 100 98 b=AS:2 b=RS:0 b=RR:500 a=rtpmap:100 red/1000/1 a=rtpmap:98 t140/1000/1 a=fmtp:100 98/98/98 a=rtt-mixer

The example in table A.5.1 shows that RTP payload type 98 is used for sending text without redundancy, whereas RTP payload type 100 is used for sending text with 200 % redundancy. IPv4 addressing is assumed in the computation of bandwidth values. The "a=rtt-mixer" attribute indicates multiparty support.

An answer from a device supporting multiparty capability could provide the following SDP:

Table A.5.2: Example SDP answer for T.140 real-time text with multiparty capability

SDP answer from multiparty capable device

m=text 14400 RTP/AVP 100 98 b=AS:2 b=RS:0 b=RR:500 a=rtpmap:100 red/1000/1 a=rtpmap:98 t140/1000/1 cps=90 a=fmtp:100 98/98/98 a=rtt-mixer

The example in table A.5.2 shows an answer with RTP payload type 100 text with 200% redundancy, RTP payload type 98 is used for declaring the "t140" format to be carried with redundancy in the "red" format. Successful multiparty support negotiation is indicated by the "a=rtt-mixer" attribute. Note that the format can be used also for point-to-point sessions.

An answer from a device without multiparty capability could provide the following SDP:

Table A.5.3: Example SDP answer for T.140 real-time text without multiparty capability

SDP answer from multiparty uncapable device

m=text 14400 RTP/AVP 100 98 b=AS:2 b=RS:0 b=RR:500 a=rtpmap:100 red/1000/1 a=rtpmap:98 t140/1000/1 cps=90 a=fmtp:100 98/98/98

The example in table A.5.3 shows an answer with RTP payload type 100 included by a device for receiving text with 200% redundancy. RTP payload type 98 is used for declaring the "t140" format to be carried with redundancy in the "red" format. IPv4 addressing is assumed in the computation of bandwidth values. Note that a mixer may send multiparty text to a device without multiparty capability by formatting text for presentation for a multiparty view with some functional limitations.

SEVENTH CHANGE

# E.4	Bi-directional real-time text (3 kbps, IPv4 or IPv6, RTCP and MBR=GBR bearer)

Bi-directional text at 3 kbps all inclusive (text, IP overhead, RTCP).

Note: For multiparty call scenarios, "Guaranteed bitrate for downlink (kbps)" and "Maximum bitrate for downlink (kbps)" should be a higher value than 3 kbps, e.g., 6kbps would be a suitable value.

Table E.3: QoS mapping for bi-directional real-time text (3 kbps, IPv4, RTCP and MBR=GBR bearer) when using a conversational class bearer

| Traffic class | Conversational class | Notes |
| --- | --- | --- |
| Delivery order | No | The application should handle packet reordering. |
| Maximum SDU size (octets) | 1400 | Maximum size of IP packets |
| Delivery of erroneous SDUs | No |  |
| Residual BER | 10-5 | Reflects the desire to have a medium level of protection to achieve an acceptable compromise between packet loss rate and speech transport delay and delay variation. |
| SDU error ratio | 1*10-3 | Text should have a higher level of protection than voice and video. |
| Transfer delay (ms) | 130 ms | Indicates maximum delay for 95th percentile of the distribution of delay for all delivered SDUs between the UE and the PS domain during the lifetime of a bearer service. Permits the derivation of the RAN part of the total transfer delay for the radio access bearer. This attribute allows RAN to set transport formats and H-ARQ/ARQ parameters such as the discard timer. |
| Guaranteed bitrate for uplink (kbps) | 3.0 | An assumed total bit-rate of a real-time text service including headers and RTCP. |
| Maximum bitrate for uplink (kbps) | 3.0 | The same as the guaranteed bitrate. |
| Guaranteed bitrate for downlink (kbps) | 3.0 | An assumed total bit-rate of a real-time text service including headers and RTCP. |
| Maximum bitrate for downlink (kbps) | 3.0 | The same as the guaranteed bitrate. |
| Allocation/Retention priority | Subscribed value | Indicates the relative importance to other radio access bearers. It should be a lower value to the priority of a Conversational bearer with source statistics descriptor ‘speech'. |
| Source statistics descriptor | ‘unknown' |  |

Using a conversational class bearer means that resources are reserved throughout the session. Depending on the intended usage of real-time text, it might not be the most resource efficient choice to use a conversational class bearer, especially if it is foreseen that the sessions will be long-lived while the actual text conversations will be rare and bursty. Table E.4 therefore shows an example with QoS mapping for using an interactive class bearer. It is recommended to use QCI 6, 8, or 9 [90] for T.140 real-time text unless the service policy decides to assign different QCI types.

Table E.4: QoS mapping for bi-directional real-time text (3 kbps, IPv4, RTCP) when using an interactive bearer

| Traffic class | Interactive class | Notes |
| --- | --- | --- |
| Delivery order | No | In sequence delivery is not required |
| Maximum SDU size (octets) | 1400 | Maximum size of IP packets |
| Delivery of erroneous SDUs | No |  |
| Residual BER | 10-5 | Reflects the desire to have a medium level of protection to achieve an acceptable compromise between packet loss rate and voice transport delay and delay variation. |
| SDU error ratio | 10-4 | Text should have a higher level of protection than voice and video. |
| Maximum bitrate (kbps) | [Depending on UE category] | Should be set as high as the UE category can handle |
| Allocation/Retention priority | Subscribed value | Indicates the relative importance to otherradio access bearers. It should be a lower value to the priority of a Conversational bearer with source statistics descriptor ‘speech'. |
| Traffic handling priority | 3 |  |
| Signalling indication | ‘No’ |  |

Table E.5: QoS mapping for bi-directional real-time text (3 kbps, IPv6, RTCP and MBR=GBR bearer) when using a conversational class bearer

| Traffic class | Conversational class | Notes |
| --- | --- | --- |
| Delivery order | No | The application should handle packet reordering. |
| Maximum SDU size (octets) | 1400 | Maximum size of IP packets |
| Delivery of erroneous SDUs | No |  |
| Residual BER | 10-5 | Reflects the desire to have a medium level of protection to achieve an acceptable compromise between packet loss rate and speech transport delay and delay variation. |
| SDU error ratio | 1*10-3 | Text should have a higher level of protection than voice and video. |
| Transfer delay (ms) | 130 ms | Indicates maximum delay for 95th percentile of the distribution of delay for all delivered SDUs between the UE and the PS domain during the lifetime of a bearer service. Permits the derivation of the RAN part of the total transfer delay for the radio access bearer. This attribute allows RAN to set transport formats and H-ARQ/ARQ parameters such as the discard timer. |
| Guaranteed bitrate for uplink (kbps) | 4.0 | An assumed total bit-rate of a real-time text service including headers and RTCP. |
| Maximum bitrate for uplink (kbps) | 4.0 | The same as the guaranteed bitrate. |
| Guaranteed bitrate for downlink (kbps) | 4.0 | An assumed total bit-rate of a real-time text service including headers and RTCP. |
| Maximum bitrate for downlink (kbps) | 4.0 | The same as the guaranteed bitrate. |
| Allocation/Retention priority | Subscribed value | Indicates the relative importance to other radio access bearers. It should be a lower value to the priority of a Conversational bearer with source statistics descriptor ‘speech'. |
| Source statistics descriptor | ‘unknown' |  |

END OF CHANGES