3GPP TSG SA WG4#126	S4-232011

Chicago, United States, 13 – 17 November 2023 	revision of S4-231682

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.565 | CR | pseudo | rev | - | Current version: | 0.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_MSE] Updates to Media Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  |  | Date: |  | 06/11/2023 |
| Category: | B |  |  |  |  |  |  | Release: |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Progress on work in MeCAR |  |  |  |  |  |  |  |  |
| Summary of change: |  | All good |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR ... CR 26.119 (S4-231672, S4-231694) |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | It may be considered to define a split rendering profile for Media in TS 26.511. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 26.857: "5G Media Service Enablers".

[2]	3GPP TS 26.119: "Media Capabilities for Augmented Reality".

=====  CHANGE  =====

Annex C (normative):
Split Rendering Profiles

## C.1 	Introduction

This Annex defines split rendering profiles to define requirements for SRC and SRS for different scenarios. At this stage the following two profiles are defined:

- 2D Pixel Streaming Profile in clause C.2 to support split rendering to 2D screens, devices of type 3 in TS 26.119 [2]

- 3D Pixel Streaming Profile in clause C.3 to support split rendering to devices of type 1, 2, and 4 in TS 26.119 [2]

## C.2 	2D Pixel Streaming Profile

### C.2.1	Introduction

This profile defines required capabilities for UE-based SRC functionalities as network-side SRS capabilities to support split rendering to 2D screens.

### C.2.2	SRC Capabilities

#### C.2.2.1	Overview

Requirements for UE-based SRC functionalities for following functions are defined in this clause:

-	Media Decoding

-	Media Encoding

-	Metadata Formats

Editor’s Note: Additional Media Capabilities are for further study

The capabilities of the receiving UE are shared with the split rendering server prior to the start of the split rendering session.

Editor’s Note: Signaling of capabilities and configurations are for further study. For example, it would indicate that the output device is an HMD that supports 2 views and stereo audio.

#### C.2.2.2 	Media Capabilities

#### The SRC shall support the media capabilities of a device type 3 as defined in TS 26.119 [2], clause 10.4.

#### C.2.2.3 	Metadata Formats

XR-Pose-Cap 1: the SRC shall be able to retrieve one or more pose predictions for each view and for every frame to be rendered. The pose predication shall be formatted according to clause 8.2.2.2.

XR-Pose-Cap 2: the SRC shall be able to retrieve and collect the user actions that occurred during an identified time interval. The action information shall be formatted according to clause 8.2.2.3.

### C.2.3	SRS Capabilities

#### C.2.3.1	Overview

Requirements for network-based SRS functionalities for following functions are defined in this clause:

-	Media Encoding

-	Media Decoding

-	Metadata Formats

Editor’s Note: Additional Media Capabilities are for further study

The capabilities of the SRC are shared with the SRC prior to the start of the split rendering session.

Editor’s Note: Signaling of capabilities and configurations are for further study.

#### C.2.3.2	Video encoding

Editor’s Note: Video Encoding capabilities is for further study to match the SRC capabilities.

#### C.2.3.3	Audio and Speech encoding

Editor’s Note: Audio and Speech Encoding capabilities is for further study to match the SRC capabilities.

#### C.2.3.4	Video decoding

Editor’s Note: Video Decoding Capabilities are for further study

#### C.2.3.5	Audio and Speech decoding

Editor’s Note: Audio and Video Decoding Capabilities are for further study

#### C.2.3.6 	Metadata Formats

Editor’s Note: Metadata capabilities are for further study to match the SRC capabilities.

## C.3 	3D Pixel Streaming Profile

### C.3.1	Introduction

This profile defines required capabilities for UE-based SRC functionalities as network-side SRS capabilities to support MeCAR devices.

### C.3.2	SRC Capabilities

#### C.3.2.1	Overview

Requirements for UE-based SRC functionalities for following functions are defined in this clause:

-	Media Decoding

-	Media Encoding

-	Metadata Formats

Editor’s Note: Additional Media Capabilities are for further study

The capabilities of the receiving UE are shared with the split rendering server prior to the start of the split rendering session.

Editor’s Note: Signaling of capabilities and configurations are for further study. For example, it would indicate that the output device is an HMD that supports 2 views and stereo audio.

#### C.2.2.2 	Media Capabilities

The SRC shall support the media capabilities of a device type 1 as defined in TS 26.119 [2], clause 10.2.

#### If the device is a device type 2 as defined in TS 26.119 [2], clause 10.4, it shall also support the media capabilities of a device type 2 as defined in TS 26.119 [2], clause 10.3.

#### If the device is a device type 4 as defined in TS 26.119 [2], clause 10.5, it shall also support the media capabilities of a device type 2 as defined in TS 26.119 [2], clause 10.5.

#### C.3.2.3	Metadata Formats

XR-Pose-Cap 1: the SRC shall be able to retrieve one or more pose predictions for each view and for every frame to be rendered. The pose predication shall be formatted according to clause 8.2.2.2.

XR-Pose-Cap 2: the SRC shall be able to retrieve and collect the user actions that occurred during an identified time interval. The action information shall be formatted according to clause 8.2.2.3.

### C.3.3	SRS Capabilities

#### C.3.3.1	Overview

Requirements for network-based SRS functionalities for following functions are defined in this clause:

-	Media Encoding

-	Media Decoding

-	Metadata Formats

Editor’s Note: Additional Media Capabilities are for further study

The capabilities of the SRC are shared with the SRC prior to the start of the split rendering session.

Editor’s Note: Signaling of capabilities and configurations are for further study.

#### C.3.3.2	Video encoding

Editor’s Note: Video Encoding capabilities is for further study to match the SRC capabilities.

#### C.3.3.3	Audio and Speech encoding

Editor’s Note: Audio and Speech Encoding capabilities is for further study to match the SRC capabilities.

#### C.3.3.4	Video decoding

Editor’s Note: Video Decoding Capabilities are for further study

#### C.3.3.5	Audio and Speech decoding

Editor’s Note: Audio and Speech Decoding Capabilities are for further study

#### C.3.3.6 	Metadata Formats

Editor’s Note: Metadata capabilities are for further study to match the SRC capabilities.