Source:		Qualcomm Incorporated (Rapporteur)

Title:	Permanent Document: Commonalities and Differences between 
TS 26.501 and TS 26.506

Version:		0.1.1

Agenda Item:		8.8

Document for:		Agreement

Work Item information:

| Work Item Name | 5G Media Streaming Protocols Phase 2 |
| --- | --- |
| Acronym | 5GMS_Pro_Ph2 |
| Unique ID | 1000018 |
| Target release | 18 |

Source details:

| Rapporteur and Co-Editor | Name | Thomas Stockhammer |
| --- | --- | --- |
|  | Company | Qualcomm Incorporated |
|  | Email | tsto@qti.qualcomm.com |
| Co-Editor | Name | Hakju Ryan Lee (iRTCw rapporteur) |
|  | Company | Samsung |
|  | Email | hakju00.lee@SAMSUNG.COM |

Revision history:

| Version | Date | Meeting | Subject/Comment |
| --- | --- | --- | --- |
| 0.0.0 | 2023-08-14 | SA4#125 | Initial Submission |
| 0.1.0 | 2023-08-24 | SA4#125 | Initial Version agreed during SA4#125 (S4-231502) |
| 0.1.1 | 2023-11-06 | SA4#126 | Proposed updates based on Telco agreements (S4aI230164) S4aI230146; [5GMS_Ph2] Discussion on media reference architecture; BBC; Richard Bradbury. Presenter: Richard Bradbury Online Discussion: - Richard: we postponed CRs for this at SA4#148. There are errors in the existing architecture (multiple reference points with the same name, etc.) I started with Thomas’s contribution as a baseline and removed duplicates, etc. I used the SA2 convention that they assign interface names and then assign reference points to the interface name. I can’t get an answer whether it’s possible to do peer-to-peer connections in RTC, but if that’s possible (and the protocols do support peer-to-peer), we need a new reference point for that. M8 is entirely private. If we can converge on this, we can align the CRs to use the agreed reference architecture - Thomas: I think this is very good. It implements exactly what we discussed about separating reference points with APIs. I am supportive of this - Spencer (in chat): I'm not going to use call time to say this, but I really like the use of functional names distinct from reference points, AND this will make spec writing easier. - Yoshihiro: Question for clarification - Richard: The proposal impacts both the 26501 and 26506. - Yoshihiro: I think we need to consider RTC aspects to see if they are considered in the proposed architecture. I will comment in meeting minutes. - Clause 2.4 looks re-defining the interface. Should we simply refer to the reference point definition in 5GMS and RTC specification? (to avoid misalignment) - Iraj: If this goes to 501 and 506, do you see any impact in architecture in 512? - Richard: Have additional proposal text coming in. - Thorsten: This M10 interface, in public networks is not allowed, it needs to go via media AS. However, in this case, one interface is missing e.g., between two WebRTC Media AS - Richard: Maybe we can add interface between Media AS to Media AS - Maybe with respect to physical layer, it will help to clarify what this M10 is - Thorsten: Thinking about WebRTC. Quite often we have STUN and TURN. Is the TURN server a new Media AS, or is it a separate entity - Imed: TURN server is not considered a Media AS - Yoshihiro: Does the generalized 5G Media Delivery architecture intend to show only common aspects? Decision: - Noted and will be revised, and will go into the permanent document S4aI230146 is noted. |

# References

- [1]	3GPP TS 26.501: "5G Media Streaming architecture", Release 18.

- [2]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)", Release 18.

# 1.	Background

At SA4#124, it was agreed to pursue the development of a new common stage 3 specification for Media Delivery in Release 18, designated TS 26.510, that harmonises media session handling procedures and APIs for both 5G Media Streaming (5GMS) and Real-Time Communication (RTC) in TS 26.506 [2]. The stage 2 architectures for 5GMS and RTC defined respectively in TS 26.501 [1] and TS 26.506 [2] will remain separate in Release 18. However, it was agreed in principle that both of these specifications will instead be enhanced to (redundantly) define a generalised Media Delivery architecture. Each of these stage-2 specifications will also map its respective reference model onto the common Media Delivery reference architecture.

Change requests to that effect were contributed to SA4#125, but there was insufficient time to agree them:

- TS 26.501 CR0074 [S4-231196] was postponed.

- TS 26.506 CR0001 [S4-231197] was postponed.

In reviewing the generalised Media Delivery reference model, some long-standing deficiencies were noted by the author of this paper and a revision to the first of the above TDocs was proposed. Specifically, the existing 5G Media Streaming architecture model in TS 26.501 [1] violates the following basic tenet of a reference model:

- A named reference point uniquely identifies exactly one pair of system actors.

As can be seen in figure 1 overleaf, the 5GMS Client function in the UE violates this tenet in two different ways:

- Reference points M6 and M7 are not unique: they each appear twice, with different pairs of system actors in each case:

  - M6 between 5GMS-Aware Application and Media Session Handler.

  - M6 between Media Stream Handler and Media Session Handler.

and:

  - M7 between 5GMS-Aware Application and Media Stream Handler.

  - M7 between Media Session Handler and Media Stream Handler.

- There is more than one reference point between the Media Session Handler and the Media Stream Handler, namely:

  - M6

  - M7

NOTE:	The 5GMS Client in the UE is depicted in the form of Media Session Handler and Media Stream Handler constituent functions which expose APIs to one another in the same way that those APIs are exposed to 5GMS-Aware Applications. This UE architecture is not applicable generally; it is just as valid to implement a 5GMS Client that does not expose interfaces M6 and M7 within the 5GMS Client. It is also valid for a 5GMS Client inside a UE to be completely self-contained, such that all functionality typically implemented in the 5GMS-Aware Application is embedded in the UE and thus interfaces M6 and M7 are not exposed at all.

Figure 1: 5G Media Streaming general architecture
(reproducing figure 4.1-2 from TS 26.501)

The root of the problem is the conflation of two distinct architectural concepts:

- Reference point.

- Interface (a.k.a. “exposed API”).

This discussion paper proposes a refinement of the generalised Media Delivery reference model that attempts to disentangle the two above architectural concepts in order to address the problem. The aim is to permit a straightforward mapping from both the extant 5GMS and RTC models with minimal specification text. An additional aim is to define meaningful – but reasonably short – names for the exposed APIs so that referring to them in specification text is not overly cumbersome.

In addition, while the stage-2 specifications are being examined and enhanced, it would be useful to consider whether additional reference points should be added to the generalised reference model to describe the possibility of:

- Chaining of Application Functions to allow its logical responsibilities to be distributed.

- Peer-to-peer Media Delivery from one UE to another without mediation through an Application Server.

NOTE:	Adding reference points to the reference architecture does not imply that they need to be further defined. At the same time as defining them, they can be declared out of scope in the present release (deferring the details until later) or just beyond the scope of 3GPP standardisation.

# 2.	Generalized Media Delivery architecture

## 2.1	Generalized Media Delivery in the 5G System

Due to the similarity of the 5GMS architecture (as defined in the present document) to the architecture for Real-Time Communication (RTC) defined in TS 26.506 [X], the RTC functions and 5GMS functions may share or may make use of many common functionalities for both media session handling and media delivery. While a full integration of 5GMS and RTC is not addressed in the present document, a generalized 5G media architecture that integrates 5GMS and RTC can be represented as shown in figure 4.1.2.1-1.

Figure 4.1.2.1-1: Generalized media support within the 5G System

In this representation:

-	The 5GMS AF is one possible realisation of the general Media AF.

-	the 5GMS AS is one possible realisation of the general Media AS.

-	The 5GMS Client is part of the general Media Client.

## 2.2	Reference architecture for Media Delivery

A functional description with additional details as well as reference points is provided below, as illustrated in figure 4.1.2.2-2.

NOTE 1:	Exposed APIs are named in italics.

NOTE 2:	If the Media Client is deployed as a monolithic functional block, it may choose not to expose interfaces externally at reference point M11.

Figure 4.1.2.2-2: Generalized 5G Media Delivery architecture

## 2.3	Network Functions and UE entities

Functional definitions may be generalized as follows:

-	Media AF: An Application Function similar to that defined in clause 6.2.10 of TS 23.501 [11] dedicated to Media Delivery.

-	Media AS: An Application Server dedicated to Media Delivery.

-	Media Client: A UE internal function dedicated to Media Delivery comprising:

-	Media Session Handler: An entity on the UE that communicates with the Media AF in order to establish, control and support the delivery of a media session.

-	Media Access Function: An entity on the UE that communicates with the Media AS in order to access and deliver media content. The media access function for example may be further sub-divided into content delivery protocols, codecs, media types and metadata representation.

-	Media-aware Application: An application entity on the UE that makes use of 3GPP-defined APIs to invoke the Media Session Handler and/or the Media Access Function in order to support Media Delivery.

NOTE:	An application (e.g., a web browser application) that does not invoke either the Media Session Handler or the Media Access Function using 3GPP-defined APIs is not considered a Media-aware Application and is not mapped into the generalized Media Delivery reference architecture.

Table 4.1.2.3-1 Mapping of 5GMS functions to generalized Media Delivery architecture

| Generalized media architecture function |  | 5GMSd function | 5GMSu function |
| --- | --- | --- | --- |
| Media AF |  | 5GMSd AF | 5GMSu AF |
| Media AS |  | 5GMSd AS | 5GMSu AS |
| Media Client |  | 5GMSd Client | 5GMSu Client |
|  | Media Session Handler | Media Session Handler |  |
|  | Media Access Function | Media Player |  |
| 5GMS Application Provider |  | Media Application Provider |  |
| 5GMS-Aware Application |  | Media-aware Application |  |

## 2.4	Reference points

The following reference points are defined for Media Delivery:

M1:	Reference point between the Media Application Provider and the Media AF for the provisioning of Media Delivery.

M2:	Reference point between the Media Application Provider and the Media AS for the purposes of ingesting media into the Media AS or egesting media from the Media AS.

M3:	Reference point between the Media AF and the Media AS for the purposes of Media AS configuration and/or for media session handling in relation to Media Delivery.

NOTE:	This reference point is out of scope for the RTC architecture in this release.

M4:	Reference point between the Media AS and the Media Access Function in the UE for the purpose of downlink transport of media from the Media AS to the Media Access Function ("content distribution") or uplink transport of media from the Media Access Function to the Media AS ("content contribution").

M5:	Reference point between the Media AF and the Media Session Handler in the Media Client for the purpose of media session handling in relation to Media Delivery.

M6:	Reference point between the Media-aware Application and the Media Session Handler for the purpose of configuring the Media Session Handler.

M7:	Reference point between the Media-aware Application and the Media Access Function for the purpose of media access control.

M8:	Reference point between the Media-aware Application and the Media Application Provider.

NOTE 1:	This reference point is private and therefore beyond the scope of standardisation.

M9:	Reference point between one instance of the Media AF and another for the purpose of Media AF instance chaining.

NOTE 2:	The details of this reference point are for future study and are not further defined in this release.

M10:	Reference point between one instance of the Media AS and another for the purpose of peer-to-peer media transport between different Media Clients.

NOTE:	This reference point is out of scope for the RTC architecture in this release.

M11:	Reference point between the Media Session Handler and the Media Access Function (both in the Media Client) for the purpose of configuring the Media Session Handler and/or media access control.

Table 4.1.2-1 Mapping of 5GMS reference points to generalized 5G Media Delivery architecture

| Generalized media architecture reference point | 5GMSd reference point | 5GMSu reference point |
| --- | --- | --- |
| M1 | M1d | M1u |
| M2 | M2d | M2u |
| M3 | M3d | M3u |
| M4 | M4d | M4u |
| M5 | M5d | M5u |
| M6 | M6d | M6u |
| M7 | M7d | M7u |
| M8 | M8d | M8u |
| M9 | N/A | N/A |
| M10 | N/A | N/A |
| M11 | M6d, M7d | M6u, M7u |

## 2.2	Interfaces and APIs

## 2.2.1	Interfaces and APIs supporting media session handling

The Media AF exposes the following network service interfaces for media session handling:

-	Provisioning API (Maf_Provisioning): External API, exposed to the Media Application Provider by the Media AF at reference point M1 to provision the usage of the Media Delivery and to obtain feedback.

-	Media Session Handling API (Maf_SessionHandling) exposed by a Media AF to the Media Session Handler at reference point M5 and/or to the Media AS at reference point M3 for media session handling, control, reporting and assistance that also include appropriate security mechanisms, e.g. authorization and authentication.

The Media Session Handler exposes the following UE APIs for media session handling:

-	Media Session Handling Client API: exposed by the Media Session Handler to the Media-aware Application at reference point M6 and to the Media Access Function at reference point M11, for configuring media session handling, including service launch.

## 2.2.2	Interfaces and APIs supporting media transport

The Media AS exposes the following network service interfaces to support media transport:

-	Media Application Server Configuration API (Mas_Configuration) used by the Media AF at reference point M3 to configure the Media AS.

The Media AS exposes the following media transport interfaces:

-	Northbound media transport interface between the Media AS and the Media Application Provider, used to exchange media data using a media transport protocol at reference point M2.

-	Southbound media transport interface between the Media Access Function and the Media AS, used to exchange media data using a media transport protocol at reference point M4.

The Media Access Client exposes the following UE APIs for media access control:

-	Media Access Control API exposed by the Media Access Function to the Media-aware Application at reference point M7 and to the Media Session Handler at reference point M11, in order to configure and communicate with the Media Access Function.

## 2.2.2	Interfaces and APIs supporting application functionality

The Media Application Provider exposes the following network service interfaces to support application functionality:

-	Application-private API used for information exchange between the Media-aware Application and the Media Application Provider at reference point M8.

# Contents

Contents	2

1	Introduction	3

2	Background	3

# 1	Introduction

It was decided to run permanent document on the side of the above referred work item to investigate the commonality and differences between TS 26.501 and TS 26.506 especially for the functionalities, interfaces, procedures, and resources associated and document them in TS 26.506 using the progress and findings on a common stage-3 specification.

# 2	Background

Commonalities across different work items are summarized in the following. For Media Delivery, extensions to the 5G System architecture were developed to address media delivery. This includes different delivery systems including download, streaming, real-time communication, etc. Key in the media delivery is that the media is time-continuous. Streaming points to the fact that the media is predominantly sent only in a single direction and consumed as it is received. Real-time communication refers more to bi-directional traffic for which media is delivered in both directions.

Generalized Media Support within the 5G System is shown in Figure 1 and Figure 2 below.

Figure 1 – Generalized Media Support within the 5G System

Figure 2 Generalized 5G Media Delivery Architecture

Functional definitions may be generalized as follows:

-	Media AF: An Application Function as defined in TS 23.501, clause 6.2.10, dedicated to Media Delivery.

-	Media AS: An Application Server dedicated to Media Delivery.

-	Media Client: A UE internal function dedicated to Media Delivery.

-	Media Session Handler: A function on the UE that communicates with the Media AF in order to establish, control and support the delivery of a media session.

-	Media Access Function: A UE internal function A function on the UE that communicates with the Media AS in order to access and deliver media content. The media access function for example may be further sub-divided into content delivery protocols, codecs, media types and metadata representation.

The following interfaces and APIs may be defined for Media Delivery:

-	M1 (Provisioning API): External API, exposed by the Media AF which enables the Media Application Provider to provision the usage of the 5G Media Delivery and to obtain feedback.

-	M2 (User Plane interface): External interface provided by the Media AS and used when the Media AS in the trusted DN to exchange data media data with the application service provider.

-	M3: (Server Configuration API): API used to exchange information between Media AF and Media AS for configuration purposes.

-	M4 (Media Delivery Interface): Interface and reference point between media access function and Media AS in order to exchange media content.

-	M5 (Session Handling API): APIs exposed by a Media AF to the Media Session Handler for media session handling, control, reporting and assistance that also include appropriate security mechanisms, e.g. authorization and authentication.

-	M6 (Client Configuration APIs): APIs exposed by a Media Session Handler to the Application and media access function for client-internal communication.

-	M7 (Media Access APIs): APIs exposed by a Media Access function to configure and communicate with the Media access function.

-	M8 (Application reference point): application interface used for information exchange between the Media Application and the Media Application Provider.

While on architecture and interface level, commonalities are pretty obvious, questions may arise what is common on the next level. For this, the core functions are further divided into

-	Media Delivery Functions including codecs, content delivery protocol, encapsulation

-	Media Session Handling functions

What has been of lower priority until now, but is getting more importance and never found a real home, are aspects around HTTP versions, HTTP headers in use, the usage and configuration for HTTP and QUIC for media user plane services, secure media delivery, and so on. While some of those aspects may be dealt specifically for a stack, some layering is needed, and also components, extensions and profiling of these stacks is needed.

A summary what may be useful for either of the frameworks is provided in Table 1.

Table 1 Overview of media and media handling functions in different service scenarios

| Feature | Downlink media streaming | Uplink media streaming | Real-time communication |
| --- | --- | --- | --- |
| Media Delivery Functions and Protocols |  |  |  |
| CMAF | Yes | yes | No |
| DASH/HLS | Yes | No | No |
| MP4 | Yes | No | No |
| RTP/AVP | Not until now | Not until now | No |
| webRTC | Not until now | Not until now | yes |
| HTTP/1.1 and TCP/IP | Yes | Yes | No |
| HTTP/2 | Yes | Yes | No |
| HTTP/3 and QUIC | Yes | Yes | No |
| UDP/IP | Yes | Yes | yes |
| Media Handling Functions |  |  |  |
| Service Access | Yes | Yes | Yes In form of configuration information |
| Content hosting | Yes | No | ??? |
| Content publishing | No | Yes | ??? |
| Content preparation | Yes | No | ??? |
| Network assistance | Yes | Yes | Yes |
| Dynamic policies | Yes | Yes | Yes |
| Remote control | No | Yes | ??? |
| Consumption reporting | Yes | No | Yes |
| QoE metrics reporting | Yes | ??? | ??? |
| Service URL Handling | Yes | Yes | Yes |
| Edge Computing | Yes | Yes | Yes |
| eMBMS delivery | Yes | No | ??? |
| Data Collection | Yes | ??? | ??? |
| MBS delivery | Yes | No | ??? |
| Event exposure | Yes | Yes | ??? |

At the end, many of the features collected above may be considered a toolbox which are applicable to different service scenarios. Differences for the different service scenarios in the content delivery protocols as well as in the applicable tools.

# 3	Common Features

Editor’s Note: collects the common features on stage 2, and identifies the commonalities and differences.