Source:	HEAD acoustics GmbH

Title:	UE classification and test structure for ATIAS

Document for:	Agreement

# Introduction

The inputs made so far for the work item ATIAS mainly focused on proposals for new test methods and requirements for immersive communication terminals. Even though these technical discussions and preliminary agreed test methods in ATIAS-1 [1] are highly relevant for the update of TS 26.260 [2] and the upcoming new TS 26.261 [3], supplementary information and several definitions are still missing. In addition, most of the test methods collected in ATIAS-1 focus on testing a certain IVAS codec mode rather than a certain UE type. The present document suggests several introductory clauses for ATIAS-1 (and/or an upcoming CR to TS 26.260) that define UE types and may help to group and better structure the existing, proposed and possibly new test methods.

# Definition of UE types

## Introduction

The definition of UE types for immersive communication terminals is not as clear and trivial as in the "traditional" test specifications 3GPP TS 26.131 [4] and TS 26.132 [5]. Due to the variety of new applications that are enabled by IVAS codec, it is expected that the capture and playback audio format might not necessarily be the same in send and receive direction. Thus, the classification of UE types should be defined as follows:

-	The UE type is composed of "SND-UE-type" and "RCV-UE-type".

-	The SND-UE-type is defined as the combination of a certain audio capturing mode (acoustic or electric) and an IVAS input format.

-	The RCV-UE-type is defined as the combination of an IVAS output format and a certain audio playback mode (acoustic or electric).

-	Each audio capturing/playback mode corresponds to a specific physical test arrangement.

While the IVAS input/output formats are already clearly defined, capturing/playback modes and corresponding interfaces were not yet discussed. The following subclauses suggest several UE type definitions, which might be applicable for SND and/or RCV.

## Handset Mode (Send + Receive)

Not applicable for immersive communication due to the following reasons:

-	A handset device is typically held close to the user's head, i.e., mouth and (a single) ear.

-	RCV: Monaural listening cannot provide any spatial/immersive audio.

-	SND: Even if the device provides multiple input microphones, it is always positioned close to the user's mouth, which does not allow to encode any spatial information into the uplink signal (for e.g., object-based audio format).

The EVS-Interop mono mode of IVAS should be tested according to 3GPP TS 26.131 [4] and TS 26.132 [5].

## Headset Mode (Send + Receive)

The test setup for headset UE for send and receive directions is shown in Figure 1. It applies to all head-worn terminals, which are typically connected via wired or wireless link (e.g., analogue jack, Bluetooth, or USB) to a device with sufficient computation power (e.g., mobile phone). Optionally, the device might provide head tracking data that can be used for rendering audio in the receive direction.

Figure 1: Headset UE and test equipment

The following IVAS formats apply for Headset UE in send direction:

-	Stereo

-	MASA

-	Object-based [TBD?]

-	Scene-based [TBD?]

The following IVAS formats apply for Headset UE in receive direction:

-	Stereo

-	MASA + optional headtracking support

-	Object-based + optional headtracking support

-	Scene-based + optional headtracking support

## Handheld Mode (Send + Receive)

The test setup for handheld hands-free UE for send and receive directions is shown in Figure 2. It applies to all devices that can be held in front of the user.

Figure 2: Handheld hands-free UE and test equipment

The distance D = 42 cm between HATS reference point (HRP) and center of the UE is used in TS 26.132 [5], but different geometries of this setup could also be considered (for e.g., multi-talker scenarios or speech from certain angles).

The following IVAS output formats can be used in conjunction with handheld hands-free UE in send direction:

-	Stereo

-	MASA

-	Object-based [TBD]

-	Scene-based [TBD]

The following IVAS input formats can be used in conjunction with handheld hands-free UE in receive direction:

-	Stereo

-	MASA [TBD: + optional support of device rotation?]

-	Object-based [TBD: + optional support of device rotation?]

-	Scene-based [TBD: + optional support of device rotation?]

## Table-mounted Mode (Send + Receive)

The test setup for table-mounted hands-free UE for send and receive directions is shown in Figure 3. It applies to all hands-free devices that are intended for usage on tables (like e.g., conference devices). In contrast to handheld UE, the reflections of the table are explicitly included in the test setup.

Figure 3: Table-mounted hands-free UE and test equipment

Figure 3 shows an example with a distance of D = 40 cm between front of the UE and lip reference plane, which corresponds to the desktop hands-free setup as specified in Recommendation ITU-T P.341 [6], which is also referenced in 3GPP TS 26.132 (width W = 40 cm, height H = 30 cm). In general, multiple sub-setups could be considered for this UE type, like e.g., the "group audio terminal" position (clause 4.2.4 of P.341 [6]) or the softphone/laptop-based setups 3GPP TS 26.132 [5].

NOTE:	The term "table-mounted hands-free" is suggested here instead of "desktop hands-free", as used in e.g., TS 26.132. The intention for this is to explicitly address also different/larger setups like e.g., conferencing scenarios with multiple microphones and loudspeaker arrays.

The following IVAS output formats can be used in conjunction with table-mounted hands-free UE in send direction:

-	Stereo

-	MASA

-	Object-based [TBD]

-	Scene-based [TBD]

The following IVAS input formats can be used in conjunction with table-mounted hands-free UE in receive direction:

-	Stereo

-	MASA

-	Object-based [TBD?]

-	Scene-based [TBD?]

## Loudspeaker Mode (Receive)

The test setup for loudspeaker hands-free UE for receive direction is shown in Figure 4. It applies to advanced (mostly larger) playback systems and/or in case the previous RCV-UE-types are not applicable to a certain device.

Figure 4: Loudspeaker hands-free UE and test equipment

The following IVAS input formats can be used in conjunction with table-mounted hands-free UE in receive direction:

-	Stereo

-	MASA

-	Object-based

-	Channel-based

-	Scene-based

## Electrical interface (Send + Receive)

The test setup for electrical interface UE for send and receive directions is shown in Figure 5. It applies to all devices that do not provide integrated or associated equipment for capturing and/or reproduction of immersive audio. Note that the interface is not necessarily digital (Bluetooth, USB, or digital audio interfaces), it could also be an analogue jack plug, which provides up to two channels in receive and also send direction (see Recommendation ITU-T P.382 [7]). Optionally, the device might provide an additional input for head tracking data that can be used for rendering the receive direction (format/hardware interface for further study…?).

Figure 5: Electrical interface UE and test equipment

The following IVAS output formats can be used in conjunction with handheld hands-free UE in send direction:

-	Stereo

-	MASA

-	Object-based

-	Channel-based

-	Scene-based

The following IVAS input formats can be used in conjunction with table-mounted hands-free UE in receive direction:

-	Stereo

-	MASA

-	Object-based

-	Channel-based

-	Scene-based

## Summary

The suggested UE types and their applicability are summarized in Table 1. Even though some of the acoustic configurations are composed of two or even more parts (like e.g., headset and mobile phone, loudspeaker system and mobile phone), these should always be tested jointly, if all parts are considered as associated by the manufacturer.

Table 1: Overview of UE configurations in send and receive direction

| Name | SND | RCV | Comment |
| --- | --- | --- | --- |
| Handset |  |  | No reasonable use case? |
| Headset |  |  | Optional: Headtracking in RCV |
| Handheld Hands-free |  |  | possibly define multiple distances for testing? |
| Table-mounted Hands-free |  |  | Table reflections relevant; possibly define multiple distances for testing? |
| Loudspeakers |  |  | For advanced playback systems or as a generic mode if handheld or table-mounted do not apply. |
| Electrical interface |  |  | Suitable for all IVAS formats; analogue (max. 2 channels), USB, Bluetooth, …? Optional: Headtracking in RCV |

# Proposed structure for TS 26.260

For the finalization of the work item ATIAS, the content of ATIAS-1 (and possibly the proposed definitions of the present document) have to be integrated into the existing specification TS 26.260 [2]. This was originally an output of the work item LiQuImAS [8], where test methods for immersive audio systems were developed in a rather generic way and are in general also applicable for communication terminals in the scope of the ATIAS work item. However, the IVAS codec and its corresponding formats, rendering capabilities, etc. were not known at that time, which are important test parameters that should be defined. On the other hand, it seems rather difficult to modify the document in a way that …

a)	the clause numbering remains consistent (in particular for adding new clauses on interfaces, definitions, codec modes, test setups, etc.),

b)	the existing test methods remain valid for the device types considered in LiQuImAS , and

c)	the clauses on new test methods for IVAS-based terminals are integrated seamlessly in the existing structure.

The source proposes the following for the upcoming structure of TS 26.260, which minimizes text duplication and redundancies:

-	All existing test methods in clause 4 remain unchanged.

-	Clause 4 should be renamed to: "Objective Test Methodologies for [Generic] Immersive Audio Systems".

-	For IVAS-based terminals, a new clause 5 should be added: "Objective Test Methodologies for Immersive [Communication/Audio] Systems [IVAS-based]".

-	In case a test method of clause 4 is found to be suitable for a test suite in clause 5, it can just be referenced ("test method according to clause 4.x.y") and complemented with IVAS-/ATIAS-specific definitions.

-	One or more introductory clauses regarding test configurations/conditions/definitions, etc., are added under subclause 5.1, 5.2, etc. (such a subclause is currently missing in clause 4).

-	After the introductory subclauses, test methods should be separated into send and receive direction (same as now in clause 4 and in ATIAS-1) and then grouped for each IVAS format (stereo, scene-/object-/channel-based, MASA). Each of this group then defines a test suite that can be run for a certain UE type (see section 2), if applicable.

NOTE:	Detailed wording of terms marked in red are for discussion.

Example of proposed new structure in TS 26.260:

5	Objective Test Methodologies for Immersive Communication Systems (IVAS-based)

5.1	Test setups for terminals

5.2	Test conditions

5.3	[other general definitions?]

5.4	Test methods in the sending direction

5.4.1	Stereo

5.4.1.1	Stereo Test Method 1

[Description of test method]

The test method applies to UE configurations Headset, Handheld-hands-free, Table-mounted.

5.4.1.2	Stereo Test Method 2

[Description of test method]

The test method applies to UE configuration electrical interface only.

…

5.4.2	Object-based Audio

5.4.2.1	…

5.4.3	Scene-based Audio

5.4.3.1	…

5.4.4	Channel-based Audio

5.4.4.1	…

5.4.5	MASA

5.4.5.1	…

5.5	Test methods in the receiving direction

5.5.1	Stereo

5.5.1.1	…

5.5.2	Object-based Audio

5.5.2.1	…

5.5.3	Scene-based Audio

5.5.3.1	…

5.5.4	Channel-based Audio

5.5.4.1	…

5.5.5	MASA

5.5.5.1	…

# Conclusion

The source suggests considering the input of the present document for the next update of the permanent document ATIAS-1 [1] and/or the upcoming CR for TS 26.260 [2].

# References

| [1] | 3GPP S4-231418, „Permanent Document ATIAS-1:“. |
| --- | --- |
| [2] | 3GPP TS 26.260, „Objective test methodologies for the evaluation of immersive audio systems,“ Release-18. |
| [3] | 3GPP TS 26.261, „Terminal audio quality performance requirements for immersive audio services,“ Release-18. |
| [4] | 3GPP TS 26.131, „Terminal acoustic characteristics for telephony; Requirements,“ Release-17.1. |
| [5] | 3GPP TS 26.132, „Speech and video telephony terminal acoustic test specification,“ Release-17.1. |
| [6] | Recommendation ITU-T P.341, „Transmission characteristics for wideband digital loudspeaking and hands-free telephony terminals,“ 03/2011. |
| [7] | Recommendation ITU-T P.382, „Technical requirements and test methods for analogue wired multi-microphone headsets or headphones and corresponding universal interface of terminals,“ 03/2023. |
| [8] | 3GPP SP-170609, „New WID on Test Methodologies for the Evaluation of Perceived Listening Quality in Immersive Audio Systems (LiQuImAS),“ SA WG4, 2017. |