Agenda item: 	8.8

Source: 	Qualcomm Inc.

Title: 	[5GMS_Pro_Ph2] Notification channel for MSH

Document for	Discussion and Agreement

# Introduction

The specification of dynamic policy and network assistance procedures in 5GMS assumes that the 5GMS AF will subscribe to notifications related to the session with the PCF, NEF, and SMF. The introduction of BDT also requires the 5GMS AF to receive notifications related to the state of the BDT resources usage during a BDT time window.

Currently, TS26.512 defines the usage of MQTT for notifications between the 5GMS AF and MSH that are limited to sending updates to the M5QoSSpecification as part of the network assistance procedure. This procedure is unfortunately under-specified and restricted. TS26.512 even lacks a reference to the specification and version number of the MQTT protocol and how to use it properly.

# Requirements

The following requirements apply to the notification mechanism:

- A notification channel will allow to asynchronously send notifications from the 5GMS AF to the MSH.

- Notification messages should allow the carriage of a wide range of notification types,

- Notification messages should uniquely identify the session they pertain to,

- Receiving and processing notifications should be efficient and should not significantly increase network, processing, memory, and power resource consumption on the UE,

- Receiving notifications should be secure and timely.

# Proposal

We propose to agree the above requirements and consider the associated CR to generalize the usage of the notification channel.