3GPP TSG-SA WG4 Meeting #126	S4-231798

# Chicago, US, 13 – 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.565 | pCR | ???? | rev | ?? | Current version: | 0.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_MSE] Pose correction parameters for split-rendering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_MSE |  |  |  |  |  | Date: |  |  | 2023-??-?? |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Pose correction is a commonly used technique in split-rendering scenario, which aims at mitigating pose error introduced by the transmission delay. However, pose-correction shall be applied carefully as the rendered content often contains layers of different areas having different sensitivity to pose correction. For example, the user-interface (UI) elements typically don’t need to be corrected, and applying the pose-correction to it would substantially drop QoE for the viewer. The current specification does not enable different layers of the rendered frame to be treated separately depending on the pose-correction sensitivity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating call flow for split-rendering setup - Include aspects about pose-correction parameters. - Document how the parameters should be used, forwarded, and processed by the different entities. - Addition of pose-correction parameter in split-rendering configuration - Session-level metadata signalling how the different layers can be derived by the UE. - Updated guidelines on policy template - Including of alpha channel |  |  |  |  |  |  |  |  |
| tr |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Pose-correction may fail and reduce the QoE in some cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 8.4.2.2, 7.2.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | N/A |  |  |  |  |  |  |  |  |

===== CHANGE #1 =====

### 5.2.2 	Call flow for Split Rendering session setup

The split rendering operation can be described by the as depicted in the call flow in the Figure 5.2.2-1.

Figure 5.2.2- 1 High-level call flow for split rendering session setup and operation

The steps are:

- The Presentation Engine discovers the split rendering server and sets up a connection to it. It provides information about its rendering capabilities and the XR runtime configuration, e.g the OpenXR configuration may be used for this purpose.

- In response, the split rendering server creates a description of the split rendering output and the input it expects to receive from the UE. Output may include multiple video tracks, auxiliary channels and metadata (e.g. SEI messages) to drive composition and rendering in the UE.

- The Presentation Engine requests the buffer streams from the MAF, which in turn establishes a connection to the split rendering server to stream pose and retrieve split rendering buffers and metadata.

- The Source Manager retrieves pose and user input from the XR runtime.

- The Source Manager shares the pose predictions and user input actions with the split rendering server.

- The split rendering server uses that information to render the frame. The split rendering server generates optimal pose-correction parameters.

- The rendered frame is encoded and streamed down to the MAF, together with the required auxiliary channels and metadata.

- The media access function demultiplex and decodes the received frames.

- The decoded raw frames are passed to the scene manager and forwarded to the XR runtime for display.

- Optionally, and if pose-correction is applied, the pose-correction parameters are passed to the scene manager and forwarded to the XR runtime to be used by the pose-correction algorithm (e.g., ATW).

- The XR runtime composes and render the frame for the final display, possibly including pose-correction.

===== CHANGE #2 =====

### 8.4.2 	Split Rendering Configuration Format

#### 8.4.2.1	Introduction

The Split Rendering client establishes an XR session locally based on the device configuration and user selection. The SR client defines the view configuration (e.g. mono or stereo views), the projection format (such as projection, equirectangular, quad, or cubemap), the swap chain image configuration, etc.

In addition, XR space and action configurations are negotiated between the SR client and server. This includes defining common XR spaces and defining and selecting actions and action sets.

The format is extensible to support the exchange of additional/future configuration information.

#### 8.4.2.2	 Split Rendering Configuration Format

The session configuration information shall be in JSON format. It shall have the following format:

| Name | Type |  |  |  | Cardinality |  |  |  |  | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| renderingFlags | Array(SR_CONFIG_FLAGS) |  |  |  |  |  | 0..1 |  |  |  |  | Provides a set of flags to activate/deactivate selected rendering functions. The defined SR_CONFIG_FLAGS are: - FLAG_ALPHA_BLENDING - FLAG_DEPTH_COMPOSITION - FLAG_EYE_GAZE_TRACKING |  |  |
| spaceConfiguration | Object |  |  |  | 0..1 |  |  |  |  | The space configuration is typically sent by the split rendering server to the split rendering client. Upon reception of this information, the SR client uses this information to create the reference and action spaces as well as to agree on common identifiers for the XR spaces. |  |  |  |  |
| referenceSpaces | Array |  |  |  | 0..1 |  |  |  |  | An array of reference spaces and their identifiers. |  |  |  |  |
| id | number |  |  |  | 1..1 |  |  |  |  | A unique identifier of the XR space in the context of the split rendering session. |  |  |  |  |
| refSpace | enum |  |  |  | 1..1 |  |  |  |  | One of the defined reference spaces in OpenXR. These may be: XR_REFERENCE_SPACE_TYPE_VIEW, XR_REFERENCE_SPACE_TYPE_LOCAL, or XR_REFERENCE_SPACE_TYPE_STAGE. |  |  |  |  |
| actionSpaces | Array |  |  |  | 0..1 |  |  |  |  | An array of action spaces that need to be defined by the split rendering client in the XR session. |  |  |  |  |
| id | number |  |  |  | 1..1 |  |  |  |  | A unique identifier of the XR space in the context of the split rendering session. |  |  |  |  |
| actionId | number |  |  |  | 1..1 |  |  |  |  | Provides the unique identifier of the action. |  |  |  |  |
| subactionPath | string |  |  |  | 1..1 |  |  |  |  | The subaction path identifies the action, which can then be mapped by the XR runtime to user input modalities. |  |  |  |  |
| initialPose | Pose |  |  |  | 0..1 |  |  |  |  | Provides the initial pose of the new XR space’s origin. |  |  |  |  |
| viewConfiguration | Object |  |  |  | 0..1 |  |  |  |  | Conveys the view configuration that is configured for the XR session. |  |  |  |  |
| type | Enum |  |  |  | 1..1 |  |  |  |  | The type indicates the view configuration. Defined values are MONO and STEREO. Other values may be added. |  |  |  |  |
| width | number |  |  |  | 1..1 |  |  |  |  | The recommended width of the swapchain image. |  |  |  |  |
| height | number |  |  |  | 1..1 |  |  |  |  | The recommended height of the swapchain image. |  |  |  |  |
| compositionLayer | string |  |  |  | 1..1 |  |  |  |  | An identifier of the selected composition layer. |  |  |  |  |
| environmentBlendMode | enum |  |  |  | 1..1 |  |  |  |  | The type indicates the environment blend mode configuration. Defined values are OPAQUE, ADDITIVE and ALPHA_BLEND. Other values may be added. |  |  |  |  |
| actionConfiguration | Array |  |  |  | 0..1 |  |  |  |  | This contains a list of the actions that are to be defined by the SR client. |  |  |  |  |
| action | Object |  |  |  | 1..n |  |  |  |  | A definition of a single action object. |  |  |  |  |
| id | number |  |  |  | 1..1 |  |  |  |  | A unique identifier of the action. |  |  |  |  |
| actionType | enum |  |  |  | 1..1 |  |  |  |  | The type of the action state. This can be a Boolean, float, vector2, pose, vibration output, etc. |  |  |  |  |
| subactionPaths | string |  |  |  | 1..n |  |  |  |  | An array of subaction paths associated with this action. The split rendering client will provide the state of all defined sub-action paths. |  |  |  |  |
| pcpConfiguration |  | Object |  |  |  | 1..1 |  |  |  |  | Provides information about the pose-correction parameters (pcp) that should be applied to the rendered video frame to minimize QoE drops |  |  |  |
| pcpReferenceChannel |  | enum |  |  |  | 1..1 |  |  |  |  | The type indicates which channel is used as reference for pose correction intervals description. Defined values are { FLAG_DEPTH, FLAG_ALPHA, FLAG_PIXEL} |  |  |  |
| pcpThresholds |  |  | Array |  |  |  |  | 1..n |  |  |  |  | This contains the list of thresholds enabling to identify pixels belonging to different layers through the selected reference channel |  |  |
| pcpSensitivity | Array | 1..n |  | This contains the list of pose correction intensity that should be applied to every layer. 0 means no pose-correction shall be applied. |  |
| extraConfigurations | Object |  |  |  | 0..1 |  |  |  |  | A placeholder for addition configuration information. |  |  |  |  |

===== CHANGE #3 =====

### 7.2.2.4	Configuration Guidelines for Split Rendering

#### 7.2.2.4.1	Guidelines on Provisioning Session

The ProvisioningSessionType shall be set to “BIDIRECTIONAL”.

The aspId shall be configured and shall be a unique identifier for the Application Service Provider that offers split rendering.

The externalApplicationId shall be a URN that uniquely identifies the application and shall be terminated by the sub-string “+3gpp-sr”. An examples is as follows: “urn:com:example:game+3gpp-sr”.

#### 7.2.2.4.2	Guidelines on Edge Resource Configuration

A split rendering application may use the procedures defined in TS26.512 clause 7.10 to define an edge resource configuration to be used for split-rendering  session. In this case:

- The eligibilityCriteria shall be present and shall have appRequest set to true.

- The easRequirements shall indicate “SR” as the easType and shall include “3gpp-sr” among the easFeatures. The serviceKpi shall be present and indicate the SRS processing and networking capabilities and requirements.

- The easRelocationRequirements shall indicate “RELOCATION_INTOLERANT” in the tolerance field.

#### 7.2.2.4.3	Guidelines on Policy Template

A policy template for the split rendering shall be associated with the split rendering configuration. The QoS specification should include at least the following sub-streams:

- 2 configurations for left and right eye buffer streams

- 1 optional configuration for a depth buffer stream

- 1 optional configuration for an alpha buffer stream

- 1 configuration for an audio stream

===== END OF CHANGES =====