3GPP TSG-S4 Meeting # 126	S4-231802

Chicago, US,  13th - 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.565 | CR |  | rev |  | Current version: | 0.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 7th November 2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on previous discussions, this pCR introduces the glTF-based description of the output format for the split rendering and uses 3GPP extensions to describe the different components of the rendered media. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 26.857: "5G Media Service Enablers".

[2]		ISO/IEC 12113:2022, Information technology, Runtime 3D asset delivery format, Khronos glTF 2.0

[3]		ISO/IEC 23090-14: Information technology — Coded representation of immersive media — Part 14: Scene Description for MPEG Media.

2nd Change

## 8.5	 	Description of the Rendering Format

8.5.1 General

In response to the Split Rendering Configuration message, the SRS shall reply with a description of the rendering format.

The rendering format description shall be a compliant glTF 2.0 [1] file. The file may include references to the buffer streams that contain the components of the rendered media.

Both SRS and SRC shall comply with the SD-Rendering-Ext1 capability as defined in TS26.119.

In addition, both SRS and SRC shall support for referencing WebRTC RTP streams and data channels as described in [2].

An SRC that complies with the 3D Pixel Streaming profile shall support the 3GPP_node_prerendered extension as defined in 8.5.2.

8.5.2	3D Pixel Streaming Profile-specific Extensions

The 3GPP_node_prerendered extension is an extension at the node level to describe that the corresponding node is accessible as a prerendered content. The 3GPP_node_prerendered extension should be associated with the root node of the scene. It constitutes an alternative representation of the node and all its children. As such, if present, if the client decides to use the pre-rendered representation, it shall completely ignore the mesh description of the node and its children nodes.

The 3GPP_node_prerendered supports multiple 2D video textures and audio sources that correspond to the rendered views and audio content.

The semantics of the 3GPP_node_prerendered are provided by the following table:

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| visual | Object | O | N/A | An object that describes the rendered visual components of the content. |
| audio | Object | O | N/A | An object that describes the rendered audio components of the content. |

The description of the visual object is provided in the following table:

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| visual_configuration | enum | O | VIEW_STEREO | An indication of the view configuration for the pre-rendered media. It can either be VIEW_MONO or VIEW_MONO. |
| Views | array(Object) | M |  | An array that describes the views of the prerendered content. |
| eye_visibility | enum | M |  | The visibility of the current view. This can take one of the following values: “EYE_LEFT”, EYE_RIGHT”, “EYE_BOTH”, or “EYE_NONE”. EYE_NONE is used for depth and transparency components. |
| composition_layers | array(number) | M |  | An array of accessors identifiers that each corresponds to a composition layer of the parent view. |
| composition_layer_type | array(enum) | M |  | For each of the composition layers of the parent view, this indicates the type of that composition layer. The values should be provided in the same order as the composition_layers. The allowed values are: “COMPOSITION_LAYER_PROJECTION”, “COMPOSITION_LAYER_QUAD”, “COMPOSITION_LAYER_EQUIRECTANGULAR”, “COMPOSITION_LAYER_CUBEMAP”, “COMPOSITION_LAYER_DEPTH”, and “COMPOSITION_LAYER_OCCUPANCY”. |

The description of the audio object in the prerendered media extension is provided in the following table:

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | enum | O | AUDIO_STEREO | describes the format of the prerendered audio content. The type can take one of the following values: “AUDIO_MONO”, “AUDIO_STEREO”, and “AUDIO_HOA”. |
| Components | array(number) | M |  | provides a list of the accessors that point to the media streams associated with rendered audio content. |

The JSON scheme for the 3GPP_node_prerendered is as follows:

{ "$schema" : "http://json-schema.org/draft-07/schema", "title" : "3GPP_node_rendered", "type" : "object", "description": "glTF extension to described pre-rendered content", "allOf": [ { "$ref": "glTFProperty.schema.json"} ], "properties" : { "visual": { "$ref": "3GPP_node_rendered.visual.schema.json", "description": "visual streamed buffers" }, "audio": { "$ref": "3GPP_node_rendered.audio.schema.json", "description": "audio streamed buffers" }, "extensions": {}, "extras": {} }, "required": ["visual"] } { "$schema" : "http://json-schema.org/draft-07/schema", "title" : "3GPP_node_rendered.visual", "type" : "object", "description": "Object representing the visual rendered media", "allOf": [ { "$ref": "glTFProperty.schema.json"} ], "properties" : { "view_configuration": { "type": "string", "description": "the view configuration used for the session", "gltf_detailedDescription": "the view configuration used for the session", "enum": ["VIEW_MONO", "VIEW_STEREO"] }, "views": { "type": "array", "description": "array of layer view objects", "gltf_detailedDescription": "", "items": { "$ref": "3GPP_node_rendered.visual.view.schema.json" }, "minItems": 1 }, "extensions": {}, "extras": {} }, "required": ["views"] } { "$schema" : "http://json-schema.org/draft-07/schema", "title" : "3GPP_node_rendered.visual.view", "type" : "object", "description": "A representation of a rendered view", "allOf": [ { "$ref": "glTFProperty.schema.json"} ], "properties" : { "eye_visibility": { "type": "string", "description": "the visibility of the current view", "enum": ["EYE_LEFT", "EYE_RIGHT", "EYE_BOTH", "EYE_NONE"] }, "composition_layers": { "type": "array", "description": "array of timed accessors that carry the streamed buffers for each composition layer of the view", "items": { "type": "integer" }, "minItems": 1 }, "composition_layer_type": { "type": "array", "items": { "type": "string", "description": "the type of composition layer in the array of composition layers with the same array index", "gltf_detailedDescription": "the type of composition layer in the array of composition layers with the same array index", "enum": ["COMPOSITION_LAYER_PROJECTION", "COMPOSITION_LAYER_QUAD", "COMPOSITION_LAYER_EQUIRECTANGULAR", "COMPOSITION_LAYER_CUBEMAP", "COMPOSITION_LAYER_DEPTH", "COMPOSITION_LAYER_OCCUPANCY"] }, "minItems": 1 }, "extensions": {}, "extras": {} }, "required": ["views"] } { "$schema" : "http://json-schema.org/draft-07/schema", "title" : "3GPP_node_rendered.audio", "type" : "object", "description": "Object representing the audio rendered media", "allOf": [ { "$ref": "glTFProperty.schema.json"} ], "properties" : { "type": { "type": "string", "description": "the type of the rendered audio", "gltf_detailedDescription": "the type of the rendered audio", "enum": ["AUDIO_MONO", "AUDIO_STEREO", "AUDIO_HOA"], "default": "AUDIO_STEREO" }, "components": { "type": "array", "description": "array of timed accessors to audio component buffers", "items": { "type": "integer" }, "minItems": 1 }, "extensions": {}, "extras": {} }, "required": ["components"] }