SA4#126 RTC SWG	S4-231836

Chicago, US, 13 November 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | pseudo | rev | - | Current version: | 0.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP] Adopting the “Absolute Sender Time” RTP header extension in 3GPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP |  |  |  |  |  | Date: |  |  | 13/11/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Google developed the “RTP Header Extension for Absolute Sender Time” in WebRTC which can be useful for delay measurement purposes. However, the RTP header extension has not been registered with IANA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add the above RTP header extension to 3GPP TS26.522. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RTP header extension can not be leveraged for delay measurements in 3GPP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Background

RTP header extensions, each carrying a single timestamp, have been developed to measure one-way delays. In RFC 6051 [1], two such RTP header extensions are specified, with a 56-bits timestamp and a 64-bit timestamp, respectively.

There is also the “RTP Header Extension for Absolute Sender Time” RTP header extension [2] that uses only 24 bits, with 6 LSB’s of the ‘integer’ portion and 18 MSB’s of the ‘fractional’ portion. The communication overhead is less than those in RFC 6051. This RTP header extension was developed for WebRTC by Google but has not been registered with IANA.

It will be beneficial to reuse this RTP header extension in 3GPP. It is recognized that guidelines in RFC 8285 recommend registering an RTP header extension with IANA using a URN-based extension URI. However, the “Absolute Sender Time” header extension as implemented in WebRTC uses an URL (or http) based extension URI. We propose to keep using the URL-based extension URI for potential IANA registration. Additionally, we propose to replicate the definition of the header extension in TS26.522 and put TS26.522 as the “reference” in the IANA registration. This eliminates the risk of using an URL-based extension URI.

As an alternative, which is not preferable, an URN-based extension URI may be defined.

# Proposed changes

In the Clause 2 References, add the following reference:

# * * * * 1st change * * * *

[10] RTP Header Extension for Absolute Sender Time                     https://webrtc.googlesource.com/src/+/refs/heads/main/docs/native-code/rtp-hdrext/abs-send-time

[11] IETF RFC 5905, Network Time Protocol Version 4: Protocol and Algorithms Specification, June 2010.

# * * * * End of 1st  change * * * *

In Clause 4, add the following:

# * * * * 2nd change * * * *

### 4.4.3	RTP Header Extension for in-band end-to-end delay measurement

#### 4.4.3.1	Absolute Sender Time RTP header extension

The “RTP Header Extension for Absolute Sender Time” is defined for the one-byte format [10]. We describe it here and then extend it to the two-byte format.

#### 4.4.3.2	One-byte RTP Header Extension Format

The RTP header extension element for the RTP packet that carries ony one timestamp T1 is shown below. This is the same as the “RTP Header Extension for Absolute Sender Time” [10].

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=2   |     Originate Timestamp or T1 (24 bits)       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

#### 4.4.3.3	Two-byte RTP Header Extension Format

The RTP header extension element for the RTP packet that carries one timestamp T1 is shown below.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0x100           |appbits|           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID           | L=3           | Originate Timestamp or T1    …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|

+-+-+-+-+-+-+-+-+

#### 4.4.3.4	Syntax

T1: consists of 24 bits, taken from the 6 LSB bits of the integer part and the 18 MSB bits of the fractional part of the NTP timestamp format defined in RFC 5905 [11].

#### 4.4.3.4	Semantics

T1: Originate Timestamp. It represents the time when a sender transmits the RTP packet toward the receiver.

#### 4.4.3.5	SDP signaling

The signaling of the delay measurement RTP header extensions shall follow the SDP signaling design and the syntax and semantics of the "extmap" attribute as outlined in RFC8285.The header extension identifiers shall be registered with IANA.

For IANA registration, the “reference” field in the registry is 3GPP TS 26.522.

The ABNF syntax for the “extmap” attribute is as follows:

extmap-attr="a=extmap:" header-ext-ID SP http://www.webrtc.org/experiments/rtp-hdrext/abs-send-time [SP format]

header-ext-ID =1*3DIGIT

format = "short"/ "long"

If the format field is absent, the RTP header extension follows the one-byte format, i.e., the “short” format. If the format field is “short”, the RTP header extension follows the one-byte format. If the format field is “long”, the RTP header extension follows the two-byte format.

NOTE: http://www.webrtc.org/experiments/rtp-hdrext/abs-send-time is the extension URI and is currently implemented in WebRTC. This extension URI, instead of URN-based ones, allows for support from WebRTC without any change to the WebRTC implementation.

NOTE: Google gave a written permission to register the RTP header extension with IANA. The “reference” entry in the IANA registry will be “3GPP TS 26.522 [v18.x.x.x]”.

# * * * * End of 2nd change * * * *

# References

[1] IETF RFC6051, Rapid Synchronisation of RTP Flows, 2010.

[2] RTP Header Extension for Absolute Send Time, http://www.webrtc.org/experiments/rtp-hdrext/abs-send-time