Source:	Nokia Corporation

Title:	Spatial capture for smartphones and other form factors

Document for:	Discussion & Agreement

Agenda Item:	7.8 – FS_DaCED

# 1. Introduction

Providing immersive audio capability in end-user devices is becoming more and more common and practicable. With the development of various 4G/5G technologies, including the recent selection of the 3GPP IVAS standard, voice and audio communications are also on a clear path to become immersive.

Due to various physical constraints relating to end-user device shapes and sizes, these devices are generally configured with varying number and arrangement of integrated microphones, which leads to different audio capturing capabilities in terms of level of possible spatiality/immersion as well as general audio quality. In addition, devices can, e.g., connect to external microphones.

# 2. Background

Draft TR 26.933 [1] documents trends in smartphone length, width, and depth. All these dimensions have been increasing over the past 10 years or so. A typical phone length in 2022 is reported at slightly below 17cm, width at slightly above 7cm, and depth at slightly below 1cm. Concerning possible microphone placement and resulting distances, we note that the device length for a typical smartphone is roughly that of the distance between human ears. We further note that the depth of a typical smartphone is small. On the other hand, foldable devices, which are not considered in above averages, could in some configurations provide greater depth and other dimensions that rival those of at least smaller tablets.

In [2], it was provided further device measurements including tablets, laptops, earbuds, watches, AR glasses, and cars. For example, tablets, like smartphones, come in many sizes, and the study indicates a typical size where the length is somewhat greater than the distance between human ears and the width often comes close or even reaches this distance.

Smartphone and tablet form factors have very similar shapes, where the possible microphone arrays are varied and irregular, allowing good separation between channels along two axes and a very limited separation along the third. This is challenging for many immersive audio capture approaches that depend on a well-understood array. In practice, various forms of parametric capture analysis have therefore been developed for and applied in multi-microphone devices for more than 10 years. In this context, interoperability has often been lacking.

Recently, a parametric immersive audio format, metadata-assisted spatial audio (MASA), described in Annex A of [3], has been specified as a means to directly interface with the Immersive Voice and Audio (IVAS) codec standard.

Several other device categories and form factors including laptops, AR glasses, and cars are also of interest in this context. However, it may not be possible to make similarly generalizing statements about their respective microphone arrays as for slim, large-displayed devices like current smartphones and tablets.

# 3. Discussion

3.1 General

Several device form factors require spatial audio capture processing that is carefully designed and tuned for the specific multi-microphone array. In practice, a suitable parametric spatial audio capture analysis and processing is often implied for such devices.

3.2 Parametric spatial audio capture

3.2.1 Principle of parametric spatial audio representation

Spatial, or immersive, audio representations generally enable reproduction of audio scenes where the listener experience can optimally correspond with the real recorded situations and environments. This means, e.g., that a listener can hear audio sources around them in their original directions relative to the listening position and orientation. Faithful reproduction is generally possible via headphones, including head-tracked binaural rendering, or a loudspeaker setup that provides sufficient spatial capability.

Parametric spatial audio describes a spatial sound field according to a parametric representation. In a typical solution, a multi-microphone capture is compressed into a lower number of audio channels and associated spatial parameters. The parameters define the perceptually relevant properties of the sound field (e.g., directional information, how directional or diffuse the sound field is, etc.), while the audio channels define the actual energetic representation of the captured sound field.

The analyzed parameters and audio channels can be further utilized for the synthesis of an accurate spatial audio representation in a desired output configuration. For efficient and realistic synthesis, the directional parameters are mapped into perceptual spatial cues, which are further applied for the transport audio channels. To enhance the quality of the reproduction, the parameters can be analysed based on the desired auditory frequency bands.

Typical processing flow of the parametric spatial audio capture is illustrated in the Figure 1.

Figure 1 Overview of the parametric spatial audio capture

A prominent example of a parametric spatial audio representation is the Metadata-assisted spatial audio (MASA) format defined in Annex A of [3]. Specifically, MASA comprises one (mono) or two (stereo) transport audio signals and metadata.

3.2.2 Characteristics of parametric spatial audio capture and representation

Parametric spatial audio capture is typically purpose-fit for the device form factor that utilizes it. There can also be different representations, or a specific capture algorithm can utilize only a subset of parameters that another capture algorithm uses. Therefore, two substantially similar devices can have different capture algorithms or at least different tunings.

Parametric spatial audio capture is generally used when the microphone array is not suitable, e.g., for capture of Ambisonics. Parametric spatial audio capture thus makes it possible to obtain a spatial audio representation on such device.

Synthesizing an Ambisonics signal from a parametric spatial audio representation infers an additional computational cost and typically results in some loss of fidelity (e.g., in terms of perceptual quality and processing delay).

Parametric spatial audio capture and representation typically enables relatively low computational complexity for capture processing and encoding, largely because the number of channels in the representation can be lower than the number of originally captured channels.

3.2.3 Factors that affect parametric spatial audio capture

Multiple factors may affect the quality and accuracy of the parametric spatial audio capture. A few factors are:

- Dimensions of the capture device (distance and the placement of the microphones)

- Number and the characteristics of the microphones (directivity and frequency response)

- Applied signal processing techniques for the captured multi-microphone signals, e.g., noise suppression and filtering

- Quality of the spatial parameter analysis algorithm and the device specific tuning

Furthermore, due to the above factors, the quality and the accuracy of the parametric spatial capture is not necessarily similar for all the capture directions. This is highly dependent, e.g., on the specific device form factor and its microphone placement/spacing.

3.2.4 Multi-microphone configurations in parametric spatial audio

The number and the configuration of the microphones can be arbitrary, but in principle the lowest number of microphones for accurate 2D planar representation is typically 2 for 180° (frontal) capture, and 3 for 360° capture. Furthermore, by increasing the number of the microphones, the whole 3D sound field can be captured accurately. Typically, minimum number for microphones in this case is 4.

The associated direction parameters can be obtained, e.g., by assessing the inter-channel properties of the captured multi-microphone signals. Such properties could be inter-channel time-difference, coherence, and/or level difference. The analysis can be based on suitable frequency bands.

# 4. Summary

This input discusses parametric spatial audio capture and representation for various UE form factors, e.g., smartphones.

It is proposed to document relevant parts of the discussion (in section 3) in TR 26.933.

# References

[1]	S4-231496: TR 26.933, v0.2.0, Study on Diverse Audio Capturing system (Release 19)

[2]	S4aA230111: “Proposal for more UE categories determination”, Beijing Xiaomi Mobile Software Co., Ltd

[3]	3GPP TS 26.258: "Codec for Immersive Voice and Audio Services (IVAS); C code (floating point)"