33GPP TSG-WG SA4 #127 meeting	S4-240037

# Sophia Antipolis, FR, 28 January – 2 February 2024					Revision of S4-xxxxxx

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.113 | CR |  | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [iRTCW] Clarification of relationship between RTC-1 API and M1 API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | iRTCW |  |  |  |  |  | Date: |  |  | 2024-01-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 6.1 descrives the relevant APIs for RTC sessions in comparison with those in TS 26.512. However the comparision in Technical Specificaiton shoud be described for TS 26.510 since these APIs are specified in TS 26.510, not TS 26.512 at the end of Release-18 work. Also, the intention of categories in Table Headers (i.e., "Inherited", "Extended", "Not Relevant" and "New") should be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description for the comparison between RTC specification and the generalized specification is modified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear description in Technicl Specification causes implementation errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The reference number [xxx] for 3GPP TS 26.510 need to be assigned when this pCR is implemented. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x2]	W3C Recommendation: "WebRTC 1.0: Real-Time Communication Between Browsers".

[x3]	IETF RFC 7478 (2015): "Web Real-Time Communication Use Cases and Requirements".

[x4]	3GPP TS 26.119: "Media Capabilities for Augmented Reality"

[x5]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)"

[x6]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[x7]	3GPP TS 26.114: " IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[x8]	IETF RFC 8825 (2021): "Overview: Real-Time Protocols for Browser-Based Applications".

[x9]	IETF RFC 8835 (2021): "Transports for WebRTC".

[x10]	ITU-R Recommendation BT.601-7 (03/2011): "Studio encoding parameters of digital television for standard 4:3 and wide screen 16:9 aspect ratios".

[x11]	Microsoft: "Microphone Array Geometry Descriptor Format".

[x12]	Apple: "Getting Raw Accelerometer Events".

[x13]	Google: "Sensor Coordinate System".

[x14]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[x15]	3GPP TS 37.324: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Service Data Adaptation Protocol (SDAP) specification".

[x16]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[x17]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[x18]	3GPP TS 23.003: "Numbering, addressing and identification".

[x19]	IETF RFC 8829 (2021): "JavaScript Session Establishment Protocol (JSEP)".

[xxx]	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling".

* * * Next Change * * * *

6.1	General

This clause defines provisioning API used by the Application Provider to provision resources for their real-time communication sessions. The Provisioning API is an extension of the Provisioning API as defined in 3GPP TS 26.510 [xxx] clause 8.

Table 6.1-1 specifies the relevant APIs for RTC sessions in comparison with those in 3GPP TS 26.510 [xxx].

The relationship is categorized as follows:

Common:
The API is supported on RTC-1 interface. The procedure, resource structure and data models for this API comply with the corresponding M1 API specified in 3GPP TS 26.510 [xxx] is implemented on RTC-1.

Extended:
The API is supported on RTC-1 interface. The procedure, resource structure and/or data models for this API has extension to the corresponding M1 API specified in 3GPP TS 26.510 [xxx]. The extensions for RTC-1 API are specified in this specification.

Not Applicable:
The API is not supported on RTC-1 interface.

NOTE:	The APIs categorized as "Not Applicable" (e.g., Server Certificates Provisioning API and Event Data Processing Provisioning API) can be changed into "Inherited" or "Extended"in future releases.

Table 6.1-1: List of APIs relevant to RTC-1

| API | Common | Extended | Not Applicable |
| --- | --- | --- | --- |
| Provisioning Sessions API |  | O |  |
| Server Certificates Provisioning API |  |  | O |
| Content Preparation Templates API |  |  | O |
| Content Protocols Discovery API |  |  | O |
| Content Hosting Provisioning API |  |  | O |
| Consumption Reporting Provisioning API | O |  |  |
| Metrics Reporting Provisioning API |  | O |  |
| Policy Templates Provisioning API |  | O |  |
| Edge Resources Provisioning API | O |  |  |
| Event Data Processing Provisioning API |  |  | O |
| Configuration Provisioning API | O |  |  |

Editor's note: Configuration Provisioning API needs to be defined as RTC specific API in 3GPP TS 26.510.

* * * End of Changes * * * *