3GPP TSG SA WG4 Meeting #127	S4-240045

# Sophia Antipolis, FR, 29 January -  2nd February 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.511 | CR | 0009 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5GMS3] Correction on IMSC 1.1, AVC and HEVC signaling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | January 3 2024 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections on the Description of ISO BMFF signaling for IMSC1.1, AVC and HEVC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial change for HEVC and AVC tracks to use correct terminology. Im2t is not defined in 14496-30 rephrase and reference correct sections of 23000-19 for IMSC 1.1 tracks. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect text, inconsistent interpretation and implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.3.1 AVC-HD sample entry referencing 4.2.1.3.2 AVC-FullHD sample entry referencing 4.2.1.3.3 AVC-UHD Sample entry referencing 4.2.2.3.1 HEVC-HD Sample entry referencing 4.2.2.3.2 HEVC-FullHD Sample entry referencing 4.5.1.2 ISO BMFF File Format 5.2.8.4 Subtitle media profiles |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

##### 4.2.1.3.1	AVC-HD

4.2.1.3.1.1	ISO BMFF File Format

If AVC-HD media is provided in a bitstream that is decodable by a decoder capable of the AVC-HD-Dec decoding capabilities as defined in clause 4.2.1.1 and the media is encapsulated in an ISO BMFF Track [14], then the file format track shall conform with the requirements as defined in ISO/IEC 14496-15 [15] clause 5, and shall contain a sample entry with name ‘avc1’ or ‘avc3’ as defined in ISO/IEC 14496-15.

Second Change

##### 4.2.1.3.2	AVC-FullHD

4.2.1.3.2.1	ISO BMFF File Format

If AVC-FullHD media is provided in a bitstream that is decodable by a decoder capable of the AVC-FullHD-Dec decoding capabilities as defined in clause 4.2.1.1 and the media is encapsulated in an ISO BMFF Track [14], then the file format track shall conform with the requirements as defined in ISO/IEC 14496-15 [15] clause 5, and shall contain a sample entry with name ‘avc1’ or ‘avc3’ as defined in ISO/IEC 14496-15.

Third Change

##### 4.2.1.3.3	AVC-UHD

4.2.1.3.3.1	ISO BMFF File Format

If AVC-UHD media is provided in a bitstream that is decodable by a decoder capable of the AVC-UHD-Dec decoding capabilities as defined in clause 4.2.1.1 and the media is encapsulated in an ISO BMFF Track [14], then the file format track shall conform with the requirements  as defined in ISO/IEC 14496-15 [15] clause 5, and shall contain a sample entry with name ‘avc1’ or `avc3` as defined in ISO/IEC 14496-15.

Fourth Change

##### 4.2.2.3.1	HEVC-HD

4.2.2.3.1.1	ISO BMFF File Format

If HEVC-HD media is provided in a bitstream that is decodable by a decoder capable of the HEVC-HD-Dec decoding capabilities as defined in clause 4.2.1.1 and the media is encapsulated in an ISO BMFF Track [14], then the file format track shall conform with the requirements as defined in ISO/IEC 14496-15 [15] clause 8, and shall contain a sample entry with name ‘hvc1’ or `hev1` as defined in ISO/IEC 14496-15 clause 8.

Fifth Change

##### 4.2.2.3.2	HEVC-FullHD

4.2.2.3.2.1	ISO BMFF File Format

If HEVC-FullHD media is provided in a bitstream that is decodable by a decoder capable of the HEVC-FullHD-Dec decoding capabilities as defined in clause 4.2.1.1 and the media is encapsulated in an ISO BMFF Track [14], then the file format track shall conform with the requirements as defined in ISO/IEC 14496-15 [15] clause 8, and shall contain a sample entry with name  'hvc1' or 'hev1' as defined in ISO/IEC 14496-15 clause 8.

Sixth Change

#### 4.5.1.2	ISO BMFF File Format

If an IMSC1.1 Text Track is provided in a bitstream that is decodable by a decoder capable of the IMSC1.1-TEXT-DEC decoding capabilities as defined in clause 4.5.1.1 and the media is encapsulated in an ISO BMFF Track [14], then the file format track shall conform with the requirements as defined in ISO/IEC 14496-30 [17] clause 5.

The XMLSubtitleSampleEntry shall contain a MIMEBox as specified in ISO/IEC 14496-12 and its content_type field shall be constrained as follows.

—	The type shall be "application".

—	The subtype shall be "ttml+xml".

The codecs parameter of the content_type field of the MIME box within the XMLSubtitleSampleEntry box shall contain the value "im2t", which signals that an IMSC1.1 text processor is required, as specified in the W3C, TTML Media Type Definition and Profile Registry.

Seventh Change

#### 5.2.8.4	Subtitle media profiles

For IMSC1.1 Text Tracks:

-	the <profiles> parameter is defined in ISO/IEC 23000-19 [7], clause 11.3.2 as application/mp4

-	the <codecs> parameter is defined in ISO/IEC 23000-19 [7], clause L.2 as 'stpp.ttml.im2t'.