3GPP TSG-SA WG4 Meeting #127	S4-240001

# Sophia-Antipolis, France, 29 January - 2 February 2024

3GPP TSG SA4-e RTC SWG post 126	TDoc S4aR230168

e-Meeting, 10 January 2024

Title:	LS on PDU Set marking configuration within the Policy APIs

Response to:	—

Release:	Rel-18

Work Item:	iRTCW

Source:	3GPP SA4

To:	3GPP CT3, 3GPP CT4

Cc:

Contact person:	Hakju Ryan Lee, Samsung (hakju00 dot lee at Samsung dot com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA4 has completed the stage-2 work on the Real-Time media Communication (RTC) architecture (TS 26.506) and is currently working on the stage-3 APIs (TS 26.113). When defining the policy related API, SA4 is considering to define the PDU Set marking configuration inside the data model of the PolicyTemplate resource to be specified in TS 26.510.

In the case that the AS provides the PDU set information via the header extension of PDU set marking for RTP or SRTP, the PDU Set marking configuration contains the description of PDU Set configuration for the session, defined in Table 1. Note that this information is acquired by the Media AF (e.g. from the Media Session Handler in the UE) and is expected to be passed to the UPF through the PCF/NEF as part of the request for a specific policy treatment to a particular PDU Set data flow.

Table 1: PDU Set marking configuration

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| serviceDataFlowDescription | ServiceDataFlowDescription | 1..1 |  | The 5-Tuple that identifies the service data flow for which the QoS dynamic policy is requested. |
| headerExtensionVersion | Integer | 1..1 |  | The RTP header extension version is set to 18 |
| localIdentifier | Integer | 1..1 |  | A unique identifier of the RTP header extension in the scope of the media session. |
| format | Boolean | 0..1 |  | Indicates if a short or a long header extension format is used. When set to false, a short 1-byte header extension format is being used. |
| pduSetSizeActive | Boolean | 0..1 |  | A flag to indicate if the PDU Set size in bytes is present in the RTP header extension. |

SA4 would like to ask CT3/CT4 to take the above information into account as part of the protocol description and to provide a response, particularly whether Policy APIs/N4 rules are able to accommodate relaying the proposed PDU Set marking configuration parameters to the UPF via PCF/SMF.

# 2	Actions

To CT3, CT4

ACTION: 	SA4 kindly asks CT3/CT4 to consider the above information and to provide a response on SA4's proposal.

# 3	Dates of next TSG SA WG 4 meetings

SA4 #127	29st January – 2nd February 2024		Sophia-Antipolis, France

SA4 #127-bis-e	08th–12th April 2024			e-Meeting