Source:	HEAD acoustics GmbH

Title:	Updated SWB Measurement results for eUET (update 3)

Document for:	Discussion

# Introduction

The eUET work item [1] was approved at SA#96 with the objectives, among other things, to update several clauses in 3GPP TS 26.131 [2] on sensitivity/frequency characteristics in sending and receiving for Headset UE, Desktop UE and handheld hands-free UE. Several missing frequency masks in SWB will be defined, including a possible review of the related test methods in 3GPP TS 26.132 [3].

A test series was conducted for headset UE, desktop hands-free UE, and handheld hands-free UE in SWB mode to investigate the performance that can be achieved by such devices [4]. The present document provides new test results for handheld hands-free UE. The measurement results can be used to define tolerance masks for the frequency responses that are under study in this work item.

# Test setups

## Headset UE

The test setup for the measurement series according to clause 5.1.2 of TS 26.132 [3] (HATS equipped with two type 3.3 ears as specified in Recommendation ITU-T P.57 [5]) was used.

## Desktop Hands-free UE

The test setup for the measurement series according to clause 5.1.3.2 of TS 26.132 [3] was used, which is mainly provides references to the measurement specifications ETSI TS 103 738/740 [6] for hands-free terminals. In contrast to other types of UE with acoustic interface, a test table is introduced into the setup and affects the acoustic paths. Figure 1 depicts the positioning of HATS, terminal, and table.

| Figure 1: Setup for desktop hands-free from side (left) and top (right) view (from [6]) |  |
| --- | --- |

## Handheld Hands-free UE

The test setup for the measurement series according to clause 5.1.3.3 of TS 26.132 [3] was used (HATS equipped with two type 3.3 ears and free-field equalization, as specified in Recommendation ITU-T P.57 [5]), which is illustrated in Figure 2.

Figure 2: Setup for handheld hands-free UE (from [3])

# Devices

## Mobile phones

The mobile phones according to Table 1 were used for the VoLTE connection to the network simulator with EVS-SWB at 24.4 kbit/s. These were either used as stand-alone devices (see desktop hands-free) or in combination with additional audio equipment (headset or desktop hands-free device).

Table 1: Mobile phones evaluated in test series

| Mobile Phone ID | Year of release |
| --- | --- |
| 1 | 2014 |
| 2 | 2021 |
| 3 | 2019 |
| 4 | 2020 |
| 5 | 2014 |
| 6 | 2022 |
| 7 | 2023 |

## Headset

The headset (HE) UE is typically assumed as the combination of a mobile phone and a bundled headset (analogue or digital). In the test series, some of such units are included as well, as indicated in the list of headsets in Table 2.

Table 2: Headsets evaluated in test series

| HE ID | El. Interface | Bundled with mobile phone ID |
| --- | --- | --- |
| 1 | Analogue | 5 |
| 2 | USB | - |
| 3 | Analogue | - |
| 4 | USB | 4 |
| 5 | Analogue | - |
| 6 | Analogue | - |

NOTE 1:	Since neither headsets nor mobile phones with SWB-capable LC3-Codec were available, Bluetooth was not considered as digital headset interface in the evaluation.

NOTE 2:	All headsets were wired binaural in-ear devices with "dangling" microphones.

Each combination of mobile phone and headset represents a separate "sub-DUT", which is denoted with a unique ID as:

DUT[mobile phone ID]-H[headset ID]

With:

[mobile phone ID]				Mobile phone ID according to Table 1.

[headset ID]	Headset ID according to Table 2.

This leads to identifiers like e.g., DUT1-H1, DUT2-H3, etc.; note that not all possible combinations could be tested due to time constraints.

NOTE 3:	The insertion in-ear headsets into the (round) ear canal entry of type 3.3 ears with a sufficient sealing is typically hard to achieve. The work around usually is to reposition and measure the headset multiple times. Due to time constraints, this could not be realized; instead, each headset positioning was manually optimized for the lowest possible leakage. However, it cannot be guaranteed that the headset tip remains stable all over the duration of the measurement.

## Desktop Hands-free

Desktop hands-free (DHF) UE is not a very common use case. When introducing this type of UE, it was most likely intended to address stand-alone hands-free/group audio terminals, which had direct network access to voice services, i.e., needed a dedicated SIM card in the device. At least today, this class of devices is very rare (or do not even exist), but there are two different options that are used more often:

-	Mobile phones are configured in hands-free mode, but in contrast to handheld hands-free mode, it is placed on a table/desktop.

-	External devices providing microphone(s) and loudspeaker(s) are connected via electrical interface of a mobile phone. This is related to headset UE, where the combination of a mobile phone and external equipment are regarded as a dedicated type of UE.

Both configurations were evaluated in the measurement series. As device under test (DUT), each mobile phone was either used stand-alone in speakerphone mode or with externally connected hands-free equipment (EQ) provided in Table 3.

Table 3: Hands-free equipment evaluated in test series

| EQ ID | El. Interface | Form factor | Playback |
| --- | --- | --- | --- |
| 1 | USB | Small (10 cm x 10 cm) | Mono |
| 2 | USB | Medium (19 cm x 19 cm) | Mono |
| 3 | USB | Medium/triangular shape (2 x 19 cm + 23 cm) | Stereo |
| 4 | USB | Large (36 cm x 18 cm) | Stereo |

Each combination of mobile phone and hands-free equipment represents a separate "sub-DUT", which is denoted with an unique identifier (ID) as:

DUT[mobile phone ID]-E[hands-free equipment ID]

With:

[mobile phone ID]				Mobile phone ID according to Table 1.

[hands-free equipment ID]		Hands-free equipment ID according to Table 3. If mobile phone is used in hands-free mode without external equipment, this ID is set to 0.

This leads to identifiers like e.g., DUT1-E0, DUT2-E1, etc.; note that not all possible combinations could be tested due to time constraints. All mobile phones and external equipment used are commercially available devices, but none of the combinations were explicitly intended to be used with each other as a bundle (as it is common for headsets, which are often included with mobile phones). All connected audio equipment devices should in general be send/receive fullband speech signals, as no limitation was provided in the data sheets and all devices can operate at multiple sampling rates up to 48 kHz.

In addition, two reference DUTs according to Table 4 were included in the evaluation as well. These are simply passive analogue loudspeakers, which were equalized and calibrated at 10 cm and in the absence of the table setup. These "devices" are only used in receiving directions and should illustrate the impact of the table surface on the frequency response. The fixed level was calibrated to meet the requirement range of RLR (5 +/- 4 dB) as per clause 7.2.3 of [2].

Table 4: Reference DUTs used in test series

| DUT ID | Description |
| --- | --- |
| REF-LS1 | Medium analogue loudspeaker |
| REF-LS2 | Small analogue loudspeaker |

## Handheld Hands-free

The five mobile phones according to Table 1 were used in loudspeaker mode for the test series with Handheld Hands-free (HH) UE.

# Tolerance Masks for SWB

## Method to derive tolerance masks

Initial tolerance masks were already proposed in a (draft) CR in S4-242049 [7], which were derived as follows:

-	In the lower frequency range (up to ~6-8 kHz), the tolerance mask to be defined for SWB should be the same as for WB.

-	The upper limits of the WB tolerance masks are used and extended from 8 kHz to 16 kHz for SWB mode.

-	To address bandpass characteristic, the lower limits of the WB tolerance masks are typically flat until 5 kHz, followed by a "roll-off" (of ~5 dB) until 6.3 kHz, which is one 1/3rd octave below maximum frequency of 8 kHz, where no lower limit applies. To extended to lower limit to SWB, the roll-off is moved from 6.3 kHz to one 1/3rd octave below maximum frequency of 16 kHz, i.e., 12.5 kHz.

The following subclauses provide the corresponding resulting tolerance masks based on the values shown in S4-242049 [7]. If ranges are provided as provisional values, the limits used in the following are marked in bold.

## Headset UE

Table 5 shows the suggested performance requirements for Headset UE in receiving, which are derived from the method described in clause 4.1.

Table 5: Headset receiving sensitivity/frequency requirement mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | 6 |  |
| 200 | 6 | -10 |
| 250 | 6 | -6 |
| 1000 | 6 | -6 |
| 2000 | 8 | -6 |
| 8000 | 8 | -6 |
| 12500 | 8 | -12 |
| 16000 | 8 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

## Desktop Hands-free UE

Table 6 and Table 7 show the suggested performance requirements for Desktop Hands-free UE in sending and receiving, which are derived from the method described in clause 4.1.

Table 6: Desktop hands-free sending sensitivity/frequency requirement mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | 5 |  |
| 200 | 5 | -5 |
| 8000 | 5 | -5 |
| 12500 | 5 | -10 |
| 16000 | 5 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Table 7: Desktop hands-free receiving sensitivity/frequency requirement mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 125 | 8 |  |
| 200 | 8 | -12 |
| 250 | 8 | -9 |
| 315 | 7 | -6 |
| 400 | 6 | -6 |
| 8 000 | 6 | -6 |
| 12 500 | 6 | -11 |
| 16 000 | 6 |  |
| NOTE: The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) - logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

## Handheld Hands-free UE

Table 8 and Table 9 show the suggested performance requirements for Handheld Hands-free UE in sending and receiving, which are derived from the method described in clause 4.1.

Table 8: Hand-held hands-free sending sensitivity/frequency requirement mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | 5 |  |
| 200 | 5 | -5 |
| 8000 | 5 | -5 |
| 12500 | 5 | -10 |
| 16000 | 5 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale |  |  |

Table 9: Hand-held hands-free receiving sensitivity/frequency requirement mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | 6 |  |
| 630 | 6 | -12 |
| 800 | 6 | -6 |
| 8 000 | 6 | -6 |
| 12 500 | 6 | -12 |
| 16 000 | 6 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

# Measurement Results

## Headset

### SND

The measurement results for SFR and UE type HE are provided in Figure 3 to Figure 8 for each headset ID and across the different mobile phones used. For headset UE in SWB, performance requirements for send frequency response already exist and are the same as for handset UE. For this reason, the tolerance mask as specified in clause 7.4.1 is shown in each figure. Note that results in sending are only provided for reference, since performance requirements are already defined.

Figure 3: SFR for mobile phones used in HE mode with additional headset H1

Figure 4: SFR for mobile phones used in HE mode with additional headset H2

Figure 5: SFR for mobile phones used in HE mode with additional headset H3

Figure 6: SFR for mobile phones used in HE mode with additional headset H4

Figure 7: SFR for mobile phones used in HE mode with additional headset H5

Figure 8: SFR for mobile phones used in HE mode with additional headset H6

For reference and to supplement the SFRs, measurement results for sending loudness rating (SLR) are provided in Table 10 for each evaluated combination of phone and EQ. The requirement range of SLR as per clause 7.2.5 of [2] is 8 dB +/- 3 dB.

Table 10: SLR (in dB) for combinations of mobile phone and headset

| EQ | H1 | H2 | H3 | H4 | H6 |
| --- | --- | --- | --- | --- | --- |
| Phone |  |  |  |  |  |
| DUT1 |  |  | 7.1 |  |  |
| DUT2 |  | 10.9 | 4.4 | 10.4 | 15.6 |
| DUT3 |  |  | 6.2 |  |  |
| DUT4 |  | 21.5 | 11.8 | 15.6 |  |
| DUT5 | 27.1 | 20.7 | 3.7 | 4.4 |  |
| DUT6 | 18.9 | 10.9 | 8.8 | 15.2 | 19.8 |

It can be noticed that DUT4 and DUT5 (except for H3 and H5) seem to expect a much higher sensitivity at the microphone input. On the other hand, H3 provides such high sensitivity in general across all mobile phones.

### RCV

The measurement results for RFR and UE type HE are provided in Figure 9 to Figure 14 for each headset ID and across the different mobile phones used. The proposed tolerance mask according to clause 4.2 is shown in the following figures.

Figure 9: RFR for mobile phones used in HE mode with additional headset H1

Figure 10: RFR for mobile phones used in HE mode with additional headset H2

Figure 11: RFR for mobile phones used in HE mode with additional headset H3

Figure 12: RFR for mobile phones used in HE mode with additional headset H4

Figure 13: RFR for mobile phones used in HE mode with additional headset H5

Figure 14: RFR for mobile phones used in HE mode with additional headset H6

For reference and to supplement the RFRs, measurement results for receive loudness rating (RLR) are provided in Table 11. For each combination of phone and headset, the nominal volume control setting closest to the target RLR of 8 dB (binaural) was selected.

It turned out that in some cases (DUT5-H2, DUT5-H4), the playback levels were quite high and independent of the volume setting. This can be explained by the digital USB-headsets used with DUT5, which was not able to handle the volume control commands via USB.

Table 11: RLR (in dB) at nominal volume control setting for combinations of mobile phone and headsets

| EQ | H1 | H2 | H3 | H4 | H5 | H6 |
| --- | --- | --- | --- | --- | --- | --- |
| Phone |  |  |  |  |  |  |
| DUT1 |  |  | 3.2 |  |  |  |
| DUT2 |  | 6.4 | 9.0 | 7.7 |  | 5.5 |
| DUT3 |  |  | 4.6 |  |  |  |
| DUT4 | 6.1 | 10.1 | 7.0 | 8.3 |  | 7,9 |
| DUT5 | 7.0 | -1.1 | 7.3 | -7.9 | 8.5 |  |
| DUT6 |  | 8.8 | 8.0 | 5.4 |  | 7.8 |

## Desktop Hands-free

### SND

The measurement results for sending frequency response (SFR) and UE type DHF are provided in Figure 15 to Figure 19 for each EQ and across the different mobile phones used. The proposed tolerance mask according to clause 4.3 is shown in the following figures.

Figure 15: SFR for mobile phones used in DHF mode without additional equipment (E0)

Figure 16: SFR for mobile phones used in DHF mode in combination with E1

Figure 17: SFR for mobile phones used in DHF mode in combination with E2

Figure 18: SFR for mobile phones used in DHF mode in combination with E3

Figure 19: SFR for mobile phones used in DHF mode in combination with E4

For reference and to supplement the SFRs, measurement results for sending loudness rating (SLR) are provided in Table 12 for each evaluated combination of phone and EQ. The requirement range of SLR as per clause 7.2.3 of [2] is 13 dB +/- 4 dB.

| EQ | E0 | E1 | E2 | E3 | E4 |
| --- | --- | --- | --- | --- | --- |
| Phone |  |  |  |  |  |
| DUT1 | 9.1 | 4.6 | 23.8 |  |  |
| DUT2 | 11.1 | -0.7 | 6 | -2 | -0.5 |
| DUT3 | 9.8 | 12 | 5 |  | -1.2 |
| DUT4 | 10.28 |  |  |  |  |
| DUT6 | 8.39 | -1.02 | 10.90 | 0.12 | -0.57 |
| DUT7 | 9.27 |  |  |  | 2.39 |

Table 12: SLR (in dB) for combinations of mobile phone and audio equipment

### RCV

The measurement results for receiving frequency response (RFR) and UE type DHF at nominal volume setting are provided in Figure 20 to Figure 24 for each HFE and across the different mobile phones used. The proposed tolerance mask according to clause 4.3 is shown in the following figures.

Figure 20: RFR for mobile phones used in DHF mode without additional HFE (E0)

Figure 21: RFR for mobile phones used in DHF mode in combination with E1

Figure 22: RFR for mobile phones used in DHF mode in combination with E2

Figure 23: RFR for mobile phones used in DHF mode in combination with E3

Figure 24: RFR for mobile phones used in DHF mode in combination with E4

Figure 25: RFR for reference loudspeakers

Two findings can be drawn from the RFR results of the reference loudspeakers (Figure 25):

- It seems in general possible that devices with loudspeakers of a certain degree of (medium-to-high) quality can pass the suggested SWB tolerance mask (which might be the basis for the extension to SWB).

- As no signal processing or coding is present, only the (mostly linear) acoustics transfer functions apply here. Since the loudspeakers were equalized in advance to a flat frequency response, RFR curves show approximately the difference between free-field conditions and the table setup.
For both loudspeakers, a level increase at 1250 Hz can be observed, which causes violations of the frequency mask. Other peaks and dips are visible as well; it should be investigated further if these are linear (and can thus be compensated with an additional corresponding equalization) – or if these occur due to resonances, from e.g., the table setup (which cannot be compensated).

For reference and to supplement the RFRs, measurement results for receive loudness rating (RLR) are provided in Table 13. For each combination of phone and EQ (or speakerphone mode), the nominal volume control setting closest to the target RLR of 5 dB was selected. However, it turned out that in some cases the playback levels were not sufficient to achieve this target (RLR > 5 dB), even at maximum volume setting.

Table 13: RLR (in dB) at nominal volume control setting for combinations of mobile phone and EQ

| EQ | E0 | E1 | E2 | E3 | E4 | LS1 | LS2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Phone |  |  |  |  |  |  |  |
| DUT1 | 8.7 | 7.8 | 5.7 |  |  |  |  |
| DUT2 | 5.3 | 5.4 | 6.3 | 7.1 | 6.7 |  |  |
| DUT3 | 4.3 | 5.4 | 4.5 |  | 4.8 |  |  |
| DUT4 | 5.43 |  |  |  |  |  |  |
| DUT6 | 6.27 | 5.75 | 5.72 | 7.66 | 8.22 |  |  |
| DUT7 | 6.25 |  |  |  | 8.84 |  |  |
| REF |  |  |  |  |  | 5.5 | 5.7 |

### RCV (speakerphone mode)

When UEs are used in speakerphone mode in table-mounted test setups, i.e., without any companion device, they are operating in speakerphone mode, which is the same mode as for handheld hands-free UE. While frequency masks in SND are the same for HH and DHF, it might be reasonable to consider the frequency mask in RCV from handheld hands-free UE (see clause 4.4), as form factor / loudspeaker size is expected to be much smaller than for mobile phones. The results with HH masks (denoted as "desktop hands-free speakerphone", DesktopHFSP) are shown in Figure 26.

Figure 26: RFR for mobile phones used in DHF mode without additional HFE (E0), with HHHF frequency mask

## Handheld Hands-free

### SND

The measurement results for SFR and UE type HH are provided in Figure 27 for each mobile phone used. The proposed tolerance mask according to clause 4.4 is shown in the following figures.

Figure 27: SFR for mobile phones used in HH mode

For reference and to supplement the SFRs, measurement results for sending loudness rating (SLR) are provided in Table 14 for each evaluated combination of phone and EQ. The requirement range of SLR as per clause 7.2.4 is 13 ± 4 dB, which is met by all devices.

Table 14: SLR (in dB) for each mobile phone

| Phone | SLR |
| --- | --- |
| DUT1 | 12.8 |
| DUT2 | 13.7 |
| DUT3 | 13.1 |
| DUT4 | 10.9 |
| DUT5 | 15.1 |
| DUT6 | 9.74 |
| DUT7 | 8.78 |

### RCV

The measurement results for RFR and UE type HH are provided in Figure 28 for each mobile phone used. The proposed tolerance mask according to clause 4.4 is shown in the following figures.

Figure 28: RFR for mobile phones used in HH mode

For reference and to supplement the RFRs, measurement results for receive loudness rating (RLR) are provided in Table 15. For each combination of phone and EQ (or speakerphone mode), the nominal volume control setting closest to the target RLR of 10 dB was selected. The requirement range of RLR as per clause 7.2.4 is 10 ± 9 dB, which is met by all devices.

Table 15: RLR (in dB) at nominal volume control setting for each mobile phone

| Phone | RLR |
| --- | --- |
| DUT1 | 10.2 |
| DUT2 | 8.4 |
| DUT3 | 11.2 |
| DUT4 | 11.4 |
| DUT5 | 10.0 |
| DUT6 | 9.23 |
| DUT7 | 6.04 |

# Conclusion

For the determination of suitable performance requirements for sending and receiving frequency responses, an additional measurement series was conducted with Handheld Hands-free UE, Headset UE, and Desktop hands-free UE. Compared to previous work [4], outlier results for Headset UE were reviewed and new measurements were conducted.

Headset UE:

Results for HE also shows varying results across the combinations of mobile phones and connected headsets. The performance of the two bundled DUT4-H4 and DUT5-H1 seems to be the same level as for non-matching combinations. Results for SFR and RFR show the performance is often comparable for a single headset that is connected to multiple phones.

Desktop hands-free UE:

The measurement data showed that performance varies highly across the DUTs, which is not a surprising result: the different combinations of mobile phone and hands-free equipment investigated were not intended to jointly work together. Standalone mobile phones performed surprisingly well in sending (see Figure 15), but clearly fail in receive direction (see Figure 20). This case might be considered by applying performance requirements and objectives of handheld hands-free UE instead.

In general, it is still unclear which device types are addressed with DHF:

- Similar to the test series in the present document, additional/external desktop-/table-mounted devices (connected via e.g., USB or Bluetooth to UEs) could be considered. However, test are usually as additional external equipment.

- Bundles of mobile phone and companion device: currently unlikely to be available on the market.

- The same applies for desktop devices with direct network access (i.e., which including a SIM card).

- Using a notebook with softphone (as indicated in TS 26.132):

Several clarifications could be added to clause 4.3 ("Acoustical interfaces") of TS 26.131/132.

Handheld Hands-free UE:

Results for SLR/RLR (all values within requirements) and in general more consistent SFR/RFR curves indicate that HH seems to be a use case that is more optimized by the manufacturers than the other two – even though the existing (for WB) and derived performance requirements (for SWB) indicate that limits are in general achievable.

Proposed tolerance masks:

For all considered UE types and directions, the suggested extension of tolerance masks from WB to SWB seem to be a suitable way forward to find performance requirements. In most cases, at least one device passes the proposed limits or in case of a failure, results are often rather close to the tolerance mask.

# References

| [1] | 3GPP SP-220610, „New WID on Enhancements to UE Testing,“ Orange, HEAD acoustics GmbH, Fraunhofer IIS, ROHDE & SCHWARZ, Amazon, 2022. |
| --- | --- |
| [2] | 3GPP TS 26.131, „Terminal acoustic characteristics for telephony; Requirements,“ Release-17.1. |
| [3] | 3GPP TS 26.132, „Speech and video telephony terminal acoustic test specification,“ Release-17.1. |
| [4] | 3GPP S4aA230120, „Updated SWB Measurement results for eUET (update 2),“ HEAD acoustics GmbH. |
| [5] | Recommendation ITU-T P.57, „Artificial Ears,“ 06/2021. |
| [6] | ETSI TS 103740 v1.4.1, „Transmission requirements for wideband mobile wireless terminals (hands-free) from a QoS perspective as perceived by the user,“ 10/2021. |
| [7] | 3GPP S4-240049, „SWB frequency masks for headset UE, desktop hands-free UE, and handheld hands-free UE (draft CR),“ HEAD acoustics GmbH. |
| [8] | 3GPP S4-221019, „Initial measurement results for eUET,“ HEAD acoustics GmbH, 08/2022. |
| [9] | 3GPP S4-231974, „SWB frequency masks for headset UE, desktop hands-free UE, and handheld hands-free UE (draft CR),“ HEAD acoustics GmbH. |