Title:	Proposal on Microphone Array Signal Processing

Source:	Beijing Xiaomi Mobile Software Co., Ltd

Document for:	Discussion & Agreement

Agenda Item:	7.8

Introduction

We have summarized the principle of acoustic design of immersive audio in chapter 6 of TR 26.933[1]. This proposal is to start the discussion of the microphone array signal processing in chapter 7 as clause 7.2. The subsequent updates will follow the current structure.

Content

…

[

Microphone Array Signal Processing

According to previous investigations, Microphone Array Signal Processing (MASP) is an essential step for stereo capture, parametric spatial audio capture and non-parametric spatial audio capture, the basic processing is to transform the raw microphone signals into an expected audio representation, and enhancement could be done if necessary.

MASP for Channel-based

MASP for Stereo

In Clause 4.1, we investigated mobile phones with dimensions of approximately 15cm in length and 7cm in width. Furthermore, a minimum of two microphones is required for stereo audio capture. Consequently, it is logical to use the most basic microphone array configuration as an example: two microphones positioned at a distance of less than 15cm or 7cm, depending on the orientation of mobile phones. The next step is to investigate how to complete microphone array signal processing for producing stereo signals based on such microphone array.

MASP for Binaural

According to clause 6.2.1, binaural capture on UE appears to be the sole format capable of directly obtaining audio from the raw microphone signals via the earbuds' microphones, enhancement processing could be needed for better performance.

MASP for Scene-based

FOA

FOA signal model is very clear, which consists of four coincident signals: W, X, Y and Z, where W is an omnidirectional signal, while X, Y and Z are figure 8 direction signals aligned with the cartesian coordinate axes. So, the aim of microphone array signal processing for FOA is very clear, which is to generate standard four coincident signals. [2]

Matrix on current FOA microphones

The current FOA microphones utilize a tetrahedral configuration, comprising four cardioid microphones oriented in the directions of Front-Left-Up (FLU), Front-Right-Down (FRD), Back-Left-Down (BLD), and Back-Right-Up (BRU). The W, X, Y, Z component is achieved through the encoding of the four cardioid signals through the following matrix:

WXYX=121111311-1-11-11-11-1-11*FLUFRDBLDBRU

HOA

HOA is for further study.

MASP for MASA

MASA format signals consist of audio signals and metadata. The metadata refer to IVAS-4 P-doc are derived from analysis of microphone raw signals, so microphone array signal processing is an essential module for producing MASA signal.

MASP for Object-based

According to IVAS-4[3], Object-based audio consists of 1-4 individual mono object streams with associated metadata. Many existing technologies can be used to obtain object-based audio.

Mono object stream

The mono object stream requires audio of high quality and sufficient SNR, characteristics that closely match the existing mono audio solution. Consequently, we can derive the mono object stream from the current mono audio solution provided by UE.

Associated Object Metadata

A minimal set of object metadata associated is the object position in the polar coordinate system described using azimuth [-180°,180°] and elevation [-90°, 90°] angel.

The estimated direction of arrival (DOA), commonly utilized in current audio services, allows these services to determine the direction of the audio that needs to be enhanced. That direction also can be used as an associated object metadata to describe the position of the audio object.

]

…

Conclusion

It is proposed to include section 2 into TR 26.933 as the chapter 7.2 Microphone Array Signal Processing.

## References

- S4-231944: [FS_DaCED] TR 26.933 v0.3.0

- Franz Zotter, Matthias Frank, “Ambisonics: A Practical 3D Audio Theory for Recording, Studio Production, Sound Reinforcement, and Virtual Reality”, Springer Nature, 2019

- S4-231031 IVAS Design Constraints (IVAS-4)