3GPP TSG-WG SA4 Meeting #127	S4-240155

Sophia-Antipolis, France, 29th Jan – 2nd Feb, 2024	(revision of S4-23xxxx)

Source:	Huawei, HiSilicon

Title:	Corrections on the policy template resources on PDU Set marking

Document for:	Approval

Agenda Item:	10.5

Work Item / Release:	iRTCW / Rel-18

Introduction

Since SA4#126, the PolicyTemplate resource definition is agreed and captured in the PD. The RTCQoSSpecification object is defined to indicate the QoS handling requirements targeted to a specific service data flow, including the PDU Set marking configuration.

However, there is another term defined in SA2 on this, named as “protocol description”. As agreed in SP-231763, the protocol description is used to indicate the transport protocol used by the service data flow (e.g. RTP, SRTP) and information, e.g. following:

-	Protocol Description: Indicates the transport protocol used by the service data flow (e.g. RTP, SRTP) and information, e.g. the following:

-	RTP [185] or SRTP [186];

-	RTP or SRTP with RTP Header Extensions, including:

-	RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179];

-	Other RTP Header Extensions as defined RFC 8285 [189];

-	RTP or SRTP without RTP Header Extensions, but together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]);

-	RTP or SRTP with RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179], and together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]);

-	RTP or SRTP with other RTP Header Extensions following RFC 8285 [189], and together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]).

Therefore, the PDU Set marking configuration in the policy template need to be aligned with SA2 and CT3/4. Besides, the PDU Set QoS parameters are also needed in the RTCQoSSpecification object.

Text proposal

# * * * * First change * * * *

## 10.2	PolicyTemplate resource

The data model for the PolicyTemplate resource is specified in table 10.2-1 below:

Table 10.2-1: Definition of PolicyTemplate resource

| Property | Type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| policyTemplateId | ResourceId | 1..1 | C: RO R: RO U: RO | Identifier of this Policy Template assigned by the 5GMS AF that is unique within the scope of the Provisioning Session. |
| state | string enum | 1..1 | C: RO R: RO U: RO | A Policy Template may be in the PENDING, INVALID, READY, or SUSPENDED state. Only a Policy Template in the READY state may be instantiated as a Dynamic Policy Instance and applied to media streaming sessions. |
| stateReason | Problem‌Details | 1..1 | C: RO R: RO U: – | Additional details about the current state of this Policy Template exposed to the 5GMS Application Provider by the 5GMS AF. The instance sub-property shall be present and shall indicate the URL of this Policy Template resource. The title sub-property shall be present and shall indicate a human-readable representation of the state property specified above, e.g. "Policy Template ready for use" or "Policy Template invalid". The detail sub-property shall be present and shall indicate a human-readable status/error message. All other properties shall be omitted. |
| externalReference | string | 1..1 | C: RW R: RO U: RW | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the media streaming session. |
| qoSSpecification | M1‌QoS‌Specification | 0..1 | C: RW R: RO U: RW | Specifies the network quality of service to be applied to media streaming sessions at this Policy Template. |
| rtcQosSpecification | array(RTCQoSSpecification) | 0..1 | C: RW R: RO U: RW | Specifies the network quality of service to be applied to the different media streams of the RTC session. |
| application‌Session‌Context | Object | 1..1 |  | Specifies information about the application session context to which this Policy Template can be applied. |
| sliceInfo | Snssai | 0..1 | C: RW R: RW U: RW | As defined in clause 5.4.4.2 of TS 29.571 [12]. |
| dnn | Dnn | 0..1 | C: RW R: RW U: RW | As defined in clause 5.3.2 of TS 29.571 [12]. |
| charging‌Specification | Charging‌Specification | 0..1 | C: RW R: RW U: RW | Provides information about the charging policy to be used for this Policy Template. |

The RTCQoSSpecification object is defined in table 10.2-2 below:

Table 10.2-2: Definition of RTCQoSSpecification object

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| serviceDataFlowDescription | ServiceDataFlowDescription | 1..1 |  | The 5-Tuple that identifies the service data flow for which the QoS dynamic policy is requested. |
| mediaIdentifier | String | 1..1 |  | Provides an identifier for the media stream to associate with the corresponding service component in the QoS Policy. |
| marBwDlBitRate | BitRate | 1..1 |  | Maximum requested bit rate for the Downlink. |
| marBwUlBitRate | BitRate | 1..1 |  | Maximum requested bit rate for the Uplink. |
| minDesBwDlBitRate | BitRate | 0..1 |  | Minimum desired bit rate for the Downlink. |
| minDesBwUlBitRate | BitRate | 0..1 |  | Minimum desired bit rate for the Uplink. |
| mirBwDlBitRate | BitRate | 1..1 |  | Minimum requested bit rate for the Downlink. |
| mirBwUlBitRate | BitRate | 1..1 |  | Minimum requested bandwidth for the Uplink. |
| desLatency | Integer | 0..1 |  | Desired Latency. |
| desLoss | Integer | 0..1 |  | Desired Loss Rate. |
| desPduSetLatency | Integer | 0..1 |  | Desired PDU Set latency. |
| desPduSetLoss | Integer | 0..1 |  | Desired PDU Set Loss Rate. |
| PduSetIntegratedInfo | Boolean | 0..1 |  | Indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side. |
| ProtocolDescription | ProtocolDescription | 0..1 |  | A description of the protocol(s) used for the service data flow, which is used for the PDU Set and End of Burst marking configuration and identification as defined in clause 5.5.4.13 of TS 29.571 [y]. |

The ProtocolDescription type defined in TS 29.244 [x] and TS 29.571 [y] includes the following parts:

-	Transport Protocol: It indicates the transport protocol used for the media flow.

-	RTP HE Information: It contains information on the RTP header extension that can be used for PDU Set and/or End of Data Burst identification if RTP/SRTP is used.

-	RTP Payload Information: It contains the RTP payload information for the RTP stream which can be used to derive the PDU Set information and/or the End of Data Burst.

When the RTP Header Extensions for PDU Set Marking is activated for the media flow, the RTP HE Information shall indicate the PDU Set marking configuration. The corresponding PDUSetMarking object is defined in table 10.2-3 below:

Table 10.2-3: Definition of PDUSetMarking object

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| headerExtensionVersion | Integer | 1..1 |  | The RTP header extension version. |
| localIdentifier | Integer | 1..1 |  | A unique identifier of the RTP header extension in the scope of the media session. |
| format | Boolean | 0..1 |  | Indicates if a short or a long header extension format is used. When set to false, a short 1-byte header extension format is being used. |
| pduSetSizeActive | Boolean | 0..1 |  | A flag to indicate if the PDU Set size in bytes is present in the RTP header extension. |

Editor’s Note:	How to link the PDUSetMarking object and the RTP HE Information in the ProtocolDescription depends on further progress in CT3/4 WGs.

# * * * * Second change * * * *

References

[1] Azure Kinect DK hardware specifications, Microsoft

[2] Intel RealSense D400 series product family datasheet, Intel

[3] Camera calibration parameters, MathWorks

[4] intrinsicMatrix, Apple

[5] CameraIntrinsics, Google

[6] S4aR220046, 3D video types for iRTC client in the terminal, Meta

[7] Quick measure, Samsung

[8] Measure app, Apple

[9] S4aR220014, 3D video capture description for iRTC client in the terminal, Meta

[10] Windows microphone array geometry descriptor format, Microsoft

[11] VRIF volumetric video guidelines v0.95, https://www.vr-if.org/guidelines/

[12] 3GPP TSG SA WG4 S4-221150, “MeCAR Permanent Document v0.3.0”, August 2022

[13] 3GPP TSG SA WG4 S4-221209, “5G_RTP Permanent Document v0.0.2”, August 2022

[14] IETF RFC 4566, “SDP: Session Description Protocol”, July 2008

[15] IETF RFC 8831, "WebRTC Data Channels", January 2021

[16] IETF RFC 4960, “Stream Control Transmission Protocol”, September 2007

[17] IETF RFC 8837, “Differentiated Services Code Point (DSCP) Packet Markings for WebRTC QoS"

[18] IETF RFC 8864, “Negotiation Data Channels Using the Session Description Protocol (SDP)”, January 2021

[19] IETF RFC 8832, “WebRTC Data Channel Establishment Protocol”, January 2021

[20] 3GPP TS 26.928, “Extended Reality (XR) in 5G”, April 2022

[21] ISO/IEC 23090-5:2021: “Information technology — Coded representation of immersive media — Part 5: Visual volumetric video-based coding (V3C) and video-based point cloud compression (V-PCC)”

[22] C. Cao, T. Simon, J. Kim, G. Schwartz, M. Zollhoefer, S. Saito, S. Lombardi, S. Wei, D. Belko,

S. Yu, Y. Sheikh, J. Saragih, Authentic volumetric avatars from a phone scan, ACM Transactions on Graphics, Volume 41, Issue 4, July 2022

[23] Multiface dataset, Meta

[24] S. Ma, T. Simon, J. Saragih, D. Wang, Y. Li, F. De La Torre, and Y. Sheikh, Pixel codec avatars, IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), June 2021

[25] https://www.youtube.com/watch?v=t7_TMD7v0Xs

[26] S4-230073, An implementation of real-time V3C streaming for conversational scenario

[27] IETF RFC 8445, “Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal”

[28] IETF RFC 8656, “Traversal Using Relays around NAT (TURN): Relay Extensions to Session Traversal Utilities for NAT (STUN)

[29] IETF RFC 8489, “Session Traversal Utilities for NAT (STUN)”

[x] 3GPP TS 29.244, "Interface between the Control Plane and the User Plane Nodes", Dec 2023

[y] 3GPP TS 29.571, "5G System; Common Data Types for Service Based Interfaces", Dec 2023

# * * * * End of change * * * *

Proposal

It’s proposed to agree and capture the changes in Clause 2 into the permanent document.