| 3GPP TR 26.966 V1.0.1 (2024-01) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Evaluation of new HEVC coding tools (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.2	Abbreviations	10

4	Background	10

5	Scenarios	10

5.1	Scenario #1.1: Streaming of stereoscopic 3D content	10

5.1.1	Overview	10

5.1.2	Review of previous work	10

5.1.3	Evaluation criteria and metrics	11

5.1.4	Evaluation methodology	11

5.2	Scenario #1.2: Low delay applications of stereoscopic 3D video	12

5.2.1	Overview	12

5.2.2	Review of previous work	12

5.2.3	Evaluation criteria and metrics	12

5.2.4	Evaluation methodology	12

5.3	Scenario #2: High quality photography	13

5.3.1	Overview	13

5.3.2	Review of previous work	13

5.3.3	Evaluation criteria and metrics	13

5.4	Scenario #3: Optimising multi-bitrate delivery	14

5.4.1	Overview	14

5.4.2	Review of previous work	14

5.4.3	Evaluation criteria and methodology	14

5.5	Scenario #4: Pose correction optimisation	14

5.5.1	Overview	14

5.5.2	Review of previous work	15

5.5.3	Evaluation criteria and methodology	15

6	Solutions	15

6.0	Mapping of Solutions to Scenarios	15

6.1	Solution #1.1: HEVC simulcast	15

6.1.1	Introduction	15

6.1.2	High-level Description	16

6.1.3	Evaluation	16

6.2	Solution #1.2: Multiview HEVC coding	16

6.2.1	Introduction	16

6.2.2	High-level Description	16

6.2.3	Evaluation	17

6.3	Solution #2.1: HEVC 4:2:0 coding	19

6.3.1	Introduction	19

6.3.2	High-level Description	19

6.3.3	Evaluation	19

6.4	Solution #2.2: HEVC 4:2:2 coding	20

6.4.1	Introduction	20

6.4.2	High-level Description	20

6.4.3	Evaluation	20

6.5	Solution #2.3: Native 4:4:4 coding - HEVC Main 4:4:4 profiles	20

6.5.1	Introduction	20

6.5.2	High-level Description	20

6.6	Solution #2.4: Derived 4:4:4 coding- Layered use of HEVC 4:2:0 profiles	21

6.6.1	Introduction	21

6.6.2	High-level Description	21

6.6.3	Evaluation	22

6.7	Solution #3.1: Scalable HEVC coding	23

6.7.1	Introduction	23

6.7.2	High-level Description	23

6.7.3	Evaluation	24

6.8	Solution #4.1: MV-HEVC with auxiliary depth/alpha channels	25

6.8.1	Introduction	25

6.8.2	High-level Description	25

6.8.3	Evaluation	26

7	Conclusions	27

7.1	Conclusions for scenario #1:	27

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

This Technical Report gathers the opportunities for improving HEVC-based services. This includes documentation of motivating use cases and scenarios. Specifically, potential of improving on the following use cases are identified: the compression performance for stereoscopic 3D content, the network performance related to exploding adaptive streaming traffic, and the demands for very high-quality image/video prosumer applications and gaming/screen content sharing.  HEVC based solutions to address each opportunity are identified: HEVC Multiview profiles, HEVC Scalable profiles, and HEVC 4:4:4 (up to 10 bits) capable profiles. Methodologies to investigate and document the pros and cons of the proposed solutions for each use case are documented. Finally, conclusions are drawn on the relevancy of solutions and if any new normative specification work is to be done.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ISO/IEC 14496-10:2022: "Information technology — Coding of audio-visual objects — Part 10: Advanced video coding"

[3]	ISO/IEC 23008-2:2015: "Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 2: High efficiency video coding"

[3]	3GPP TR 26.905: "Mobile stereoscopic 3D video".

[4]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[5]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)".

[6]	3GPP TS 26.214: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[7]	3GPP TS 26.218: "Virtual Reality (VR) profiles for streaming applications"

[8]	3GPP TS 26.347: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs"

[9]	Vetro, Anthony. "Frame compatible formats for 3D video distribution." In 2010 IEEE International Conference on Image Processing, pp. 2405-2408. IEEE, 2010.

[10]	Hannuksela, Miska M., Ye Yan, Xuehui Huang, and Houqiang Li. "Overview of the multiview high efficiency video coding (MV-HEVC) standard." In 2015 IEEE International Conference on Image Processing (ICIP), pp. 2154-2158. IEEE, 2015.

[11]	ISO/IEC JTC1/SC29/WG11 MPEG2011 M22746, "AVC/MVC anchor coding for MFC", November 2011, Geneva, Switzerland.

[12]	ISO/IEC JTC1/SC29/WG11 N16050, "MV-HEVC Verification Test Report", San Diego, US, Feb. 2016.

[13]	ISO/IEC 14496-15:2022, "Information technology — Coding of audio-visual objects — Part 15: Carriage of network abstraction layer (NAL) unit structured video in the ISO base media file format"

[14]	"HTTP Live Streaming (HLS) authoring specification for Apple devices," https://developer.apple.com/documentation/http-live-streaming/hls-authoring-specification-for-apple-devices

[15]	"ISO Base Media File Format and Apple HEVC Stereo Video Format additions," Version 0.9 (Beta) June 21, 2023

[16]	"Apple HEVC Stereo Video," Interoperability Profile Version 0.9 (Beta) June 21, 2023

[17]	Delbracio, Mauricio, Damien Kelly, Michael S. Brown, and Peyman Milanfar. "Mobile computational photography: A tour." Annual Review of Vision Science 7 (2021): 571-604.

[18]	Camera & Imaging Products Association (CIPA) "Production, Shipment of Digital Still Camera January, January-January in 2017," 2016

[19]	"Smartphones vs Cameras: Closing the gap on image quality," https://www.dxomark.com/smartphones-vs-cameras-closing-the-gap-on-image-quality/

[20]	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG JVT-I018, "Color format downconversion for test sequence generation," 2003.

[21]	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG JVT-I019, "Color format upconversion for video display," 2003.

[22]	ISO/IEC 23008-12:2022: "Information technology - MPEG systems technologies - Part 12: Image File Format".

[23]	ISO/IEC 14496-12:2022: "Information technology — Coding of audio-visual objects — Part 12: ISO base media file format".

[24]	"Using HEIF or HEVC media on Apple devices," https://support.apple.com/en-us/HT207022

[25]	"HEIF Imaging," https://source.android.com/docs/core/camera/heif

[26]	ITU-T Recommendation T.81: "Information technology; Digital compression and coding of continuous-tone still images: Requirements and guidelines".

[27]	3GPP TR 26.948: "Study on video enhancements in 3GPP multimedia services"

[28]	HTTP Live Streaming (HLS) Authoring Specification for Apple Devices, https://developer.apple.com/documentation/http_live_streaming/http_live_streaming_hls_authoring_specification_for_apple_devices

[29]	Samira Afzal, Vanessa Testoni, Christian Esteve Rothenberg, Prakash Kolan, Imed Bouazizi, “A holistic survey of multipath wireless video streaming”, Journal of Network and Computer Applications, 212: 103581 (2023)

[30]	ISO/IEC JTC1/SC29/WG11 N16051, "SHVC verification test report", February 2016, San Diego, USA.

[31]	ISO/IEC JTC1/SC29/WG11 N16268, "Supplemental SHVC verification test report", June 2016, Geneva, CH.

[32]	3GPP TR 26.955: "Video codec characteristics for 5G-based services and applications"

[33]	ISO/IEC 23000-19:2020, "Information technology — Multimedia application format (MPEG-A) — Part 19: Common media application format (CMAF) for segmented media"

[34]	ISO/IEC JTC1/SC29/WG03 N01026, "Preliminary WD of ISO/IEC 23000-19 AMD New Structural CMAF Brand Profile", October 2023, Hannover, Germany.

[35]	Recommendation ITU-R BT.2095-1 "Subjective assessment of video quality using expert viewing protocol (2016-2017) ", 06/2017.

[36]	ISO/IEC JTC1/SC29/WG03 N01033, "Technology under consideration on CMAF", October 2023, Hannover, Germany.

[37]	3GPP TR 26.928: "Extended Reality (XR) in 5G"

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Background

The video codec characteristics for 5G services are documented in TR 26.955 [32], and they demonstrate that the HEVC coding standard provides satisfactory performance to fulfil the needs of video service studied in the TR. It also recommended to consider upgrading specifications to support profiles, levels, and possibly features available in HEVC, including features that may include better support for screen content and computer-generated content, XR/AR type of services, as well as low and very low latency services. On the one hand, there is interest in the distribution, including streaming, of 3D movie content, and on the other hand, there are applications that could benefit from the distribution of 4:4:4 video, such as screen sharing, gaming, and even for new immersive applications. Finally, the use of scalability could further enhance multi-bitrate systems such as video conferencing, or adaptive streaming, but may also provide additional benefits to end user devices, such as power adaptation. HEVC may be suitable to cater and enable such applications. This specification outlines these emerging applications for video coding, gather evidence whether specific new tools can provide advantage for specific services and applications, and conclude if normative specification work is needed on these aspects.

# 5	Scenarios

## 5.1	Scenario #1.1: Streaming of stereoscopic 3D content

### 5.1.1	Overview

There has been renewed interest in the distribution, including streaming, of 3D movie content, as evident by media coverage of recent 3D movie releases. Consumption of stereoscopic 3D video content is expected to rapidly grow given new AR related products beings launched.

### 5.1.2	Review of previous work

Evaluation of AVC based stereoscopic 3D coding techniques has been done in TR 26.905 [3] and its normative support has been added for 3GPP DASH in TS 26.247 [3], the 3GPP file format in TS 26.244 [5], IMS in TS 26.114 [6], VR profiles in TS 26.118 [7], and MBMS in TS 26.347 [8]. The work done in TR 26.905 [3] for Rel-11 focused mostly on stereoscopic viewing on TVs, while today's applications have grown far beyond these, given especially advancements in AR devices. Also, today's requirements on quality are much higher owing to higher quality displays and the available channel capacities. Frame packing is not sufficient because of the detrimental impact on quality, as noted in TR 26.905 [3]: using frame sequential or side by side packed full resolution is not as efficient as multiview based coding.

With the established support for MV-AVC in 3GPP SA4 specifications, an assessment needs to be done to upgrade the support for multiview coding using MV-HEVC with its superior coding performance.

### 5.1.3	Evaluation criteria and metrics

The evaluation for the coding performance for stereoscopic 3D content needs to be performed based on the following evaluation criteria.

1. Assessment/discussion of hardware impact: there are two possibilities for this:

a)	There is existing hardware product-grade support for the tool. In that case, refer to the example hardware.

b)	There is no existing hardware support. In this case, a discussion/description with justifications on the expected impact on hardware implementation is provided, or reference to existing demos etc.

2. Codec performance evaluation

a)	PSNR-based Rate-Distortion (RD) objective performance evaluation, where the RD performance is compared for various solutions with a fixed QP encoding setting to get the plotting data points. A better PSNR-based RD performance is preferred, keeping in view the expected hardware complexity impact.

b)	Subjective performance evaluation.

### 5.1.4	Evaluation methodology

#### 5.1.4.1	Objective performance evaluation

For objective performance evaluation, suitable source test content is identified that is accepted by video experts as representative content. Some of the important parameters for the content are the resolution, framerate, bit depth, color subsampling, and duration, in addition to the number of views available. Reference software for a specific solution is to be used with fixed QP encoding settings to generate each plotting point on the PSNR RD curves. The encoding settings (e.g. prediction types IPP or IBP etc.) are decided by experts considering the complexity and latency needs for the scenario. The resulting curves can directly be used for comparison by plotting together or by comparing the Bjøntegaard Delta (BD) bitrate.

#### 5.1.4.2	Subjective performance evaluation

Recommendation ITU-R BT.2095-1 Subjective assessment of video quality using expert viewing protocol [35] describes the method to subjectively assess video quality by means of the expert viewing protocol (EVP), with the participation of a reduced number of viewers, all selected among experts in the relevant video processing area.  This methodology has been used in JVET for the assessment of multiview video codec performance. The EVP visual evaluation protocol is specified in detail in [35] with the following main features:

- 9 experts participate as viewers in each EVP session,

- The “unimpaired” Source video Clip (SRC) is shown once, followed by two Processed Video Sequences (PVSs),

- Experts are required to compare the PVS with the SRC, and to rate them separately.

## 5.2	Scenario #1.2: Low delay applications of stereoscopic 3D video

### 5.2.1	Overview

While scenario #1.1 focuses the use case of streaming of stereoscopic 3D content, there are several other use cases for such content where the latency requirements are stricter compared to the lax latency requirements of the streaming use case. For example, with the advent of modern-era XR devices, video conversational applications exchange stereoscopic 3D content. Some of the other use cases may include the stereoscopic content exchange for split rendering over edge where a (partially) rendered stereoscopic view may be exchanged between the edge cloud server and the device. Such low latency applications will demand different source formats (resolutions, framerates etc.), coding settings, as well as transport considerations to cater for this lower latency requirement.

### 5.2.2	Review of previous work

The evaluation of AVC based stereoscopic 3D coding techniques done in TR 26.905 [3] was primarily focused on download and streaming scenarios. Similarly, most other normative aspects specified had been for download or streaming use cases e.g. in 3GPP DASH in TS 26.247 [3], the 3GPP file format in TS 26.244 [5], IMS in TS 26.114 [6], VR profiles in TS 26.118 [7], and MBMS in TS 26.347 [8]. [Reduced resolution frame packing is not sufficient because of the detrimental impact on quality due to resampling, as noted in TR 26.905 [3]. On the other hand, using full resolution frame packing on is not as efficient as multiview based coding because of either the complete loss or limitations of using inter-layer prediction.]

TR 26.928 [37] (study on Extended Reality (XR) in 5G) has documented a video resolution of 2k x 1k per eye at 50/60 fps, 4-10 Mbps (viewport-dependent) in context of quality and bitrate considerations for omnidirectional visual formats, similarly in clause 6.2.8 (XR conversational application). Further traffic characteristics were not documented (noted as FFS).

Hence in addition to a study on the streaming applications of stereoscopic 3D video content, realtime delivery aspects also need to be studied.

### 5.2.3	Evaluation criteria and metrics

The evaluation for the performance of coding stereoscopic 3D content for low delay applications can be done in alignment with the evaluation for streaming applications. However, low delay configurations instead of random access ones, would need to be considered. Additional criteria include:

- Assessment/discussion of hardware impact; there are two possibilities for this:

  - There is existing hardware product-grade support for the tool. In that case, refer to the example hardware.

  - There is no existing hardware support. In this case, a discussion/description with justifications on the expected impact on hardware implementations is provided, or reference to existing demos etc.

- Codec performance evaluation

  - PSNR-based Rate-Distortion (RD) objective performance evaluation, where the RD performance is compared for various solutions with a fixed QP encoding setting to get the plotting data points. A better PSNR-based RD performance is preferred, keeping in view the expected hardware complexity impact.

  - Subjective performance evaluation.

### 5.2.4	Evaluation methodology

Editor's note: Harmonizing this clause with clause 5.1.4 for streaming case is FFS.

Editor's note: Using existing study results e.g., for coding configuration, from TR 26.955 and TR 26.928 is FFS.

#### 5.2.4.1	Objective performance evaluation

For an objective performance evaluation, suitable source test content should be identified that is accepted by video experts as representative content. Some of the important parameters for the content include the resolution, framerate, bit depth, color subsampling, and the duration, of the content in addition to the number of views available. Reference software for a specific solution is to be used with fixed QP encoding settings to generate each plotting point on the PSNR RD curves. The encoding settings (e.g. prediction types IPP or IBB etc.) are to be decided by experts, considering the complexity and latency needs for the scenario. The resulting curves can directly be used for comparison by plotting them together with an anchor, i.e. simulcast encoding of both views, or by computing the Bjøntegaard Delta (BD) rate metric compared to the anchor.

#### 5.2.4.2	Subjective performance evaluation

Same considerations are made as in clause 5.1.4.2, i.e. relying on previous strategy adopted by JVET for assessment of multiview video codec performance by using EVP [35].

## 5.3	Scenario #2: High quality photography

### 5.3.1	Overview

The demand for high quality photography has been and continues to stay a dominating factor in cell phone market growth [17]. Reports such as [18] (processed and published by [19]) have shown in the past that smartphone shipments have been devouring not just point-and-shoot but also high-end DSLR cameras, by closing the gap in image quality. Additional encoding tools are needed to progress further in achieving even higher image quality.

### 5.3.2	Review of previous work

JPEG-based still image [26] support is provided in SA4 specifications, and suitable extensions to attain an even higher quality are explored in this scenario.

### 5.3.3	Evaluation criteria and metrics

The evaluation for high quality image encoding tools shall be done based on the following evaluation criteria.

- Assessment/discussion of hardware impact: there are two possibilities for this:

  - There is existing hardware product-grade support for the tool. In that case, refer to the example hardware.

  - There is no existing hardware support. In this case, a discussion/description with justifications on the expected impact on hardware implementation is provided, or reference to existing demos etc.

- Codec performance evaluation

  - Objective performance evaluation: e.g. PSNR-based Rate-Distortion (RD) performance evaluation, where the RD performance is compared for various solutions. A better PSNR-based RD performance is preferred, keeping in view the expected hardware complexity impact.

### 5.3.4	Evaluation methodology

Editor's note: How evaluation is performed to assess the evaluation criteria is FFS and concluded by SA4#127.

## 5.4	Scenario #3: Optimising multi-bitrate delivery

### 5.4.1	Overview

New video codecs have potential to assist further in optimising multi-bitrate delivery applications such as video conferencing, or adaptive streaming, and may also provide additional benefits to end user devices, such as power adaptation. One specific target of optimization is the storage space savings achieved by employing scalable video.

### 5.4.2	Review of previous work

SA4 has studied SHVC in TR 26.948 [27] in 2015, there are however possibility of exploring new scenarios since that time that will be pursued here.

### 5.4.3	Evaluation criteria and methodology

- Assessment/discussion of hardware impact: there are two possibilities for this:

  - There is existing hardware product-grade support for the tool. In that case, refer to the example hardware.

  - There is no existing hardware support. In this case, a discussion/description with justifications on the expected impact on hardware implementation is provided, or reference to existing demos etc.

- Codec performance evaluation

  - The performance evaluation of positive impact on streaming will be determined by the savings of storage space w.r.t. conventional streaming with similar quality. Calculations are to be done on representative scenario for adaptive streaming.

## 5.5	Scenario #4: Pose correction optimisation

### 5.5.1	Overview

This scenario deals with a split-rendering case where the device is running a pose correction method (e.g., using ATW). While pose correction is a good solution to cope with the latency introduced by the roundtrip communication and the rendering, it can introduce visual artifacts if only 2D projected images are used. As an example, a rendered scene may be composed by multiple elements having different sensitivity to time warping. For instance, the user-interface (UI) does not need to be corrected as its position won’t change in the user’s Field of View (FoV). A 3D object near the user may benefit from a time warping as the pose correction would address parallax differences. The far away background similarly to the UI does not need warping as parallax fall off in the distance. This is illustrated in the Figure below.

Figure 1: Illustration of layering in rendering

To maintain the effectiveness of pose correction, a rendering engine may apply segmentation and generate multiple layers of projected texture images that may be handled differently based on their time-warping sensitivity. Those different texture layers could be encoded and processed separately in multiple video streams but could also be encoded in a single stream with additional depths and alpha channels.

To drive the pose-correction and maximize the QoE, additional information may be provided to support segmentation into layers and to support the pose correction, indicating how the different texture layers should be handled by the pose correction engine. However, such optional metadata is currently not supported by OpenXR APIs.

Generally, the carriage of depth and alpha channels in the video bitstreams for proper scene and UI restitution allows to improve pose correction. New video codecs have the potential to address this scenario in a bandwidth efficient manner.

Editor’s NOTE: MeCAR is in the process to define a simple format for this purpose. It should be referenced.

### 5.5.2	Review of previous work

The carriage of depth or alpha auxiliary channels has not been addressed until now.

### 5.5.3	Evaluation criteria and methodology

- Assessment/discussion of hardware impact: there are two possibilities for this:

  - There is existing hardware product-grade support for the tool. In that case, refer to the example hardware.

  - There is no existing hardware support. In this case, a discussion/description with justifications on the expected impact on hardware implementation is provided, or reference to existing demos etc.

- Codec performance evaluation can be evaluated in two possible ways:

  - For single layer case, the performance evaluation of impact on bandwidth will be determined by the overhead introduced by adding additional channels to the video (alpha, depth, …) compared to traditional approach. It is expected that the additional cost is negligible.

  - For multi-stream case, the performance evaluation of impact on bandwidth will be determined by measuring the overhead introduced by multiple encodings compared to a single-layer approach. It is expected that the additional cost is low.

# 6	Solutions

## 6.0	Mapping of Solutions to Scenarios

Table 6.0-1: Mapping of Solutions to Scenarios

| Solution # | Solution Title | Scenario(s) |
| --- | --- | --- |
| #1.1 | HEVC simulcast | #1.1 |
| #1.2 | Multiview HEVC coding | #1.1 |
| #2.1 | HEVC 4:2:0 coding | #2 |
| #2.2 | HEVC 4:2:2 coding | #2 |
| #2.3 | Native 4:4:4 coding - HEVC Main 4:4:4 profiles | #2 |
| #2.4 | Derived 4:4:4 coding - Layered use of HEVC 4:2:0 profiles | #2 |
| #3.1 | Scalable HEVC coding | #3 |
| #4.1 | MV-HEVC with auxiliary depth/alpha channels | #4 |

## 6.1	Solution #1.1: HEVC simulcast

### 6.1.1	Introduction

HEVC simulcast is considered as a baseline solution to addresses Scenario#1.

### 6.1.2	High-level Description

#### 6.1.2.1	Overview MV-HEVC

This baseline solution uses two independent High Efficiency Video Coding (HEVC) [3] streams to transport the left- and right-eye view of the stereoscopic content. It represents a baseline or reference scenario that does not exploit any redundancy of the views during coding. Based on this fact that this simplistic solution does not optimize the performance, and due to its impacts that are noted in later in the evaluation, it is never practically used and is documented for reference/benchmark purpose only.

#### 6.1.2.2	Transport of HEVC Simulcast

As noted in the overview, this solution is relevant for benchmark/reference purpose only and is not deployed, hence there is no existing support for its transport.

### 6.1.3	Evaluation

#### 6.1.3.1 Assessment/discussion of hardware impact

This solution would require two independent video decoders, each to decode a given view, and hence it requires twice as much hardware for decoding as for a single 2D video stream.

#### 6.1.3.2 Codec performance evaluation based on existing results

Subjective evaluation results using this technique as a reference to compare with 8-bit MV-HEVC are documented in [12].

## 6.2	Solution #1.2: Multiview HEVC coding

### 6.2.1	Introduction

This solution addresses Sceanrio#1.

### 6.2.2	High-level Description

#### 6.2.2.1	Overview MV-HEVC

The Advanced Video Coding (AVC) (H.264) [2] and the High Efficiency Video Coding (HEVC) (H.265) [3] standards were initially intended for the compression of two-dimensional (2D) video. Multi-view extensions for HEVC were then developed, referred to as Multiview Video Coding (MVC) and Multiview HEVC (MV-HEVC) [3][10], respectively. The fundamental principle of both MVC and MV-HEVC is to re-use the coding tools of the underlying 2D AVC and HEVC coding respectively, so that implementations can be realized by software changes to high-level syntax in the slice header level and above [10]. For the case of HEVC, multiview profiles exist for coding both 8- and 10-bit content.

As a reference, MVC has been studied in detail in TR 26.905 [3] and its normative support has been added for 3GPP DASH in TS 26.247 [3], the 3GPP file format in TS 26.244 [5], IMS in TS 26.114 [6], VR profiles in TS 26.118 [7], and MBMS in TS 26.347. MVC does not currently support the encoding of 10-bit content.

#### 6.2.2.2	Transport of MV-HEVC

##### 6.2.2.2.1	Carriage in ISO BMFF

The carriage of MV-HEVC is specified in detail in [13] as one of the "Layered HEVC ((L-HEVC) extensions", including SHVC, MV-HEVC, and 3D-HEVC. Clause 9 of [13] specifies this L-HEVC elementary stream and sample definitions.

##### 6.2.2.2.2	Adaptive Streaming

Encoding and encapsulation guidelines for MV-HEVC in HTTP Live Streaming (HLS) are documented in [14]. Currently, the following recommendations are currently provided for resolutions, bitrates and framerates for both SDR and HDR MV-HEVC content:

| 16:9 aspect ratio | MV-HEVC SDR 30 fps | MV-HEVC HDR 30 fps | Frame rate |
| --- | --- | --- | --- |
| 640 x 360 | 246 | 272 | ≤ 30 fps |
| 768 x 432 | 510 | 612 | ≤ 30 fps |
| 960 x 540 | 1020 | 1241 | ≤ 30 fps |
| 960 x 540 | 1530 | 1853 | ≤ 30 fps |
| 960 x 540 | 2720 | 3281 | Same as source |
| 1280 x 720 | 4080 | 4930 | Same as source |
| 1280 x 720 | 5780 | 6936 | Same as source |
| 1920 x 1080 | 7650 | 9180 | Same as source |
| 1920 x 1080 | 9660 | 11900 | Same as source |
| 2560 x 1440 | 13770 | 16490 | Same as source |
| 3840 x 2160 | 19720 | 23630 | Same as source |
| 3840 x 2160 | 28560 | 34000 | Same as source |

##### 6.2.2.2.3	Support in CMAF

CMAF (ISO/IEC 23000-19 [33]) signalling is required to convey the unique parameters for Multiview video encoded formats (e.g. how the views may be organized in switching sets, what possibilities are allowed or disallowed, which addressable units are relevant to application for stereoscopic vs. 2D displays, etc.).

As noted in clause 6.2.2.2.1, carriage of MV-HEVC is specified in ISO/IEC 14496-15 [13] (NAL-video file format) as one of the "Layered HEVC (L-HEVC) extensions", including SHVC, MV-HEVC, and 3D-HEVC. Clause 9 of 14496-15 specifies this L-HEVC elementary stream and sample definitions. Despite this, there is a need to enable CMAF-level functionality noted above. Based on this need, MPEG has started working on an MV-HEVC extension of CMAF in [34].

Editor's note: The progress on the MPEG CMAF preliminary WD needs to be tracked in the TR and concluded by SA4#127.

### 6.2.3	Evaluation

Editor's note: Further work on evaluation is FFS.

#### 6.2.3.1 Assessment/discussion of hardware impact

Support for the multiview profiles of HEVC mostly involves SW level modifications since the support of multiview coding only involves high-level syntax signalling and coding tool considerations [10].

#### 6.2.3.2 Codec performance evaluation based on existing results

The objective and subjective performance results comparing MVC, Simulcast HEVC (each view is coded independently) with MV-HEVC are documented in [12]. The test sequences used for this evaluation are 1080p 8-bit 4:2:0 either 25 or 30 Hz. IPP encoding is used to generate the results. The objective results demonstrate significant performance improvements achieved by MV-HEVC against both MVC and simulcast HEVC, demonstrated by the Bjøntegaard Delta (BD) bitrates table reproduced here:

| Test Sequence | BD-rate reduction of MV-HEVC [%] relative to |  |
| --- | --- | --- |
|  | MVC | Simulcast HEVC |
| S03: Undo_Dancer | -45.7 | -38.7 |
| S04: GT_Fly | -52.9 | -41.0 |
| S13: Band06 | -43.3 | -31.7 |
| S14: BMX | -60.6 | -25.6 |
| Average | -50.6 | -34.2 |

Hence at least 30% performance gains were observed against simulcast HEVC. The corresponding subjective tests using “Expert Viewing Protocol” (EVP) verified the objective gains via MOS for all the sequences above. For example, the results for the sequences "Undo Dancer" and "BMX" are copied in the following, other results in [12] follow these results similarly.

Figure 6.2.3.2-1: EVP results for sequence "Undo Dancer" [12]

Figure 6.2.3.2-1: EVP results for sequence "BMX" [12]

Although no formal evaluation exists for the Multiview Main 10 profile of MV-HEVC, considering the large gains achieved as noted above, it is expected that it’s performance should be similar to what is demonstrated for 8-bit content.

Editor's note: Further documentation of comparison with frame-packing is FFS.

## 6.3	Solution #2.1: HEVC 4:2:0 coding

### 6.3.1	Introduction

This solution of using 8 and 10 bit HEVC [xb] 4:2:0 coding, depending on the source material, is the baseline solution for scenario#2. Such solution is already widely deployed, typically using the HEIF format [22]. 10-bits are also used to support High Dynamic Range (HDR) and Wide Colour Gamut (WCG) formats.

### 6.3.2	High-level Description

HEVC coding for still images using the HEIF file format [22] is widely deployed and supported by the current mobile ecosystem [24], [25]. This file format is designed to enable the interchange of images and image sequences, using the ISO base media file format as its basis [23]. When the requirements of the HEVC-specific brands are applied, the file format can be referred to as the HEVC Image File Format.

### 6.3.3	Evaluation

This is the baseline solution, i.e. baseline for evaluation of other solutions.

Assessment of all other solutions should be based on using this baseline technology, by taking 4:4:4 still image content, both in standard dynamic range (SDR) and high dynamic range (HDR) and first downconverting them to 4:2:0, while retaining the original bitdepth (i.e. 8 or 10 bits) using agreed downsampling methods (see JVT-I018[20]). Then such content can be coded with the appropriate HEVC 4:2:0 profile using the HEVC reference encoder (HM). Given the prevalence of the full range in still image content, full range signals should be generated across all conversion steps. For 8 bit material, it might also be desirable to explore the use of JPEG encoding for the same content. Chroma location of type 1, which is also prevalent in still image compression should be used for 8 bit material. For 10 bit content, including HDR, chroma location type 1 should be used.

After decoding, the content will be upconverted to 4:4:4 using a well agreed methodology (see JVT-I019 [21]). Afterwards, metrics will be computed for the upconverted content such as PSNR for the three colour components, Y, Cb, Cr in the 4:4:4 domain using the original content. The bits needed for coding these representations would also be considered.

## 6.4	Solution #2.2: HEVC 4:2:2 coding

### 6.4.1	Introduction

This solution uses 4:2:2 capable profiles that are already defined in HEVC for the coding of still images. Such images are then encapsulated in a file format based on the HEIF specification.

### 6.4.2	High-level Description

The HEVC video coding standard specifies profiles capable of coding images in a 4:2:2 coding format. This includes the HEVC Main 422 10, Main 422 12, Main 422 10 Intra, and Main 422 12 Intra profiles. These profiles are however not typically supported by mobile devices. Interest is primarily in applications limited to up to 10 bits of precision and therefore only profiles that satisfy this constrain should be evaluated.

### 6.4.3	Evaluation

#### 6.4.3.1 Assessment/discussion of hardware impact

As noted above, there is a limited existing hardware support available for this solution and hence the hardware impact is potentially large.

#### 6.4.3.2 Codec performance evaluation

Assessment should be based on taking the same 4:4:4 still image content as in baseline solution 2.1. The material can be then downconverted to 4:2:2, while retaining the original bitdepth (i.e. 8 or 10 bits) using an agreed horizontal downsampling method (see JVT-I018[20]). Then such content can be coded with the appropriate HEVC 4:2:2 profile using the HEVC reference encoder (HM). As in the previous solution, and given the prevalence of the full range in still image content, full range signals should be generated across all conversion steps.

After decoding, the content will be upconverted to 4:4:4 using a well agreed methodology (see JVT-I019 [21]). Afterwards, metrics will be computed for the upconverted content such as PSNR for the three colour components, Y, Cb, Cr in the 4:4:4 domain using the original content. The bits needed for coding these representations would also be considered.

Editor's note: The impact of up/down conversion on PSNR metric is FFS to be concluded by SA4#127.

## 6.5	Solution #2.3: Native 4:4:4 coding - HEVC Main 4:4:4 profiles

### 6.5.1	Introduction

This solution explores the use of the various 4:4:4 capable profiles that are already defined in HEVC for the coding of still images. Such images are then encapsulated in a file format based on the HEIF specification.

### 6.5.2	High-level Description

#### 6.5.2.1	Overview

The HEVC video coding standard specifies the clear definition of several profiles capable of coding images in a 4:4:4 coding format. This includes the Main 4:4:4, Main 4:4:4 Still Picture, Main 4:4:4 10, Main 4:4:4 12, Main 4:4:4 10 Intra, and Main 4:4:4 12 Intra profiles, among others. Some of these profiles are already supported in some mobile devices but may not be widely available everywhere. These profiles are mostly targetting for the best coding performance, using the tools available in HEVC for the corresponding format(s) that they can support.

Interest is primarily in applications limited to up to 10 bits of precision and therefore only profiles that satisfy this constrain should be evaluated.

### 6.5.3	Evaluation

#### 6.5.3.1 Assessment/discussion of hardware impact

As noted above, there is a limited existing hardware support available for this solution and hence the hardware impact is potentially large.

#### 6.5.3.2 Codec performance evaluation

Assessment should be based on taking the same 4:4:4 still image content as in baseline solution 2.1 and coding them with the appropriate HEVC 4:4:4 profile using the HEVC reference encoder (HM). No bitdepth or format conversion needs to be performed. For such content then metrics such as PSNR for the three colour components, Y, Cb, Cr in the 4:4:4 domain should be computed using the original, 4:4:4, content. Unlike baseline solution 2.1, no upconversion or downconversion needs to be performed. The bits needed for coding these representations would also be considered.

## 6.6	Solution #2.4: Derived 4:4:4 coding- Layered use of HEVC 4:2:0 profiles

### 6.6.1	Introduction

This solution explores the use of derived 4:4:4 coding, where a base layer image, that is coded in 4:2:0 mode, is augmented using auxiliary images, to derive the 4:4:4 chroma format representation. Such capabilities can be achieved, for example, in HEIF, and are currently used for other applications. This permits decoders that are not capable of native 4:4:4 HEVC coding to still be able to encode and decode 4:4:4 content through simple software support.

### 6.6.2	High-level Description

#### 6.6.2.1	Overview

The HEIF specification permits a concept called derived images, which permits the signaling of instructions to the decoder on how to combine a set of images together to generate an alternative representation of that same image. The concept could easily be used also for the support of 4:4:4 images. In this scenario a derived image can be based on a base, 4:2:0, image and one or two more images that contain the chroma information in the 4:4:4 format. Additional instructions would exist that provide information to the decoder on how to extract this chroma information and how to apply them onto the base image to achieve the desired, 4:4:4, output.

As one approach, a single enhancement image may be used that contains both Cb and Cr components stacked together, e.g. in a side by side or over-under representation. Such data are placed in the “luma” plane of that image and dummy data, e.g. a value of 128 for 8 bit data, is added in the “chroma” planes of that same image. This new image is then coded independently from the base layer image. During decoding, a decoder may select to discard the 4:2:0 version of the chroma information and instead replace that information from the information provided in this enhancement image.

As a different implementation, the enhancement image may contain predicted residuals for the Cb and Cr components given upscaled versions of the chroma values in the 4:2:0 representation. However, we do not advocate for this approach, even if it may appear more efficient in terms of coding efficiency, since that creates reconstruction dependencies of the 4:4:4 chroma values with the coding and upscaling of the 4:2:0 chroma values. There is no guarantee, for example, that all implementations could use a particular chroma upscaler while any further transcoding of the 4:2:0 representation could have an adverse effect in the reconstruction of the 4:4:4 representation.

The two chroma planes could also be coded in separate enhancement images if that is desired. A decoder can select to decode one of both enhancement images and augment either one or both components.

HEIF is also capable in achieving region of interest enhancement if that is desired.

As in the previous cases, interest is primarily in applications limited to up to 10 bits of precision and therefore only profiles that satisfy this constrain should be evaluated.

### 6.6.3	Evaluation

#### 6.6.3.1 Assessment/discussion of hardware impact

Unlike solution 2.2, this approach allows existing HW, that support HEVC 4:2:0 profiles, to be used for the delivery of 4:4:4 content. The only requirement would be to perform the reconstruction in SW, after decoding of the multiple layers.

In this scenario additional images over scenario 1 should be coded that only contain the chroma planes. These chroma planes could either be coded as two separate images or stacked together in either a side by side or over under representation. The bit-depth of the original content will be retained also for the chroma planes. Metrics will be computed using the decoded chroma data from these additional coded images, while the bits of scenario one will be augmented by the bits also needed for coding these additional representations.

Figure 1. Enhancement layers for the creation of a 4:4:4 derived representation

Figure 2. Single enhancement layer using stacking for the creation of a 4:4:4 derived representation

#### 6.6.3.2 Codec performance evaluation

In this scenario, in addition to the bistreams used for solution 2.1, the chroma planes would also have to be coded in full resolution, either by packing the two chroma planes together and coding them as a single image or by coding each chroma plane independently. After decoding, the PSNR for these two chroma planes would have to be computed compared to the original 4:4:4 chroma planes and that value should be used in place of the Cb/Cr PSNR values of solution 2.1. In addition, the extra bit overhead of coding the full resolution chroma planes needs to be included in the evaluation and when comparing with either solution 2.1 or solution 2.2.

## 6.7	Solution #3.1: Scalable HEVC coding

### 6.7.1	Introduction

Several video coding standards and technologies, such as AVC and HEVC, include scalable extensions, which enable these technologies to provide “flexible” experiences to end users, such as allowing spatial, SNR, or bitdepth scalability. It is claimed that such functionalities can reduce the bitrate/storage needed by certain applications that may require multiple instances of the same video to be available to the end-user, e.g., in a multi-conferencing scenario simultaneously supporting multiple heterogeneous devices and networks. It has been argued, however, that such solutions have little benefits, if any, while adding a lot in terms of complexity, compared to existing solutions for adaptive streaming, such as Dynamic Adaptive Streaming over HTTP (DASH) and HTTP Live Streaming (HLS).

Such statements seem to be mostly based on the assumption that scalable coding would completely replace the existing adaptive streaming solutions. Instead, a more plausible alternative could be the use of scalability as a way of augmenting adaptive streaming systems by still using a solution with multiple independent bitstreams encoded at different bitrates and resolutions [28], while augmenting some or all of these bitstreams with 1 (preferably) or more enhancement layers.

Looking further in the future, in recent years new network protocols [29] are being discussed for the delivery of media and other services, such as QUIC and Multipath QUIC (MP-QUIC). Scalability can even better fit within such new protocols since it could better enable prioritization and delivery of different packets (i.e., the protocol could handle differently the base layer versus the enhancement layer or layers) with less waste in bandwidth.

Other benefits of scalability include power adaptation, simultaneous support of multiple screens with different capabilities (e.g., resolution, SDR vs HDR etc.). Scalability can be especially useful for multi-conferencing applications. On the other hand, the implementation cost of supporting scalable systems based on the Scalable HEVC profiles can be considered as minimal since that mostly involves SW level modifications in end devices because of its design.

### 6.7.2	High-level Description

#### 6.7.2.1	Overview using scalable HEVC for adaptive streaming

An example is shown in Table 1, where a scalable layer is introduced when a change of resolution occurs from one stream to the next.

Table 1. Example Bitrate ladder for a Scalable Adaptive Streaming solution

| Streams | 16:9 aspect ratio | HEVC (base layer) | Enhancement layer | Frame rate |
| --- | --- | --- | --- | --- |
| R1 | 640 x 360 | 145 | 77.5 at 768 x 432 | ≤ 30 fps |
| R2 | 768 x 432 | 300 | 150 at 960 x 540 | ≤ 30 fps |
| R3 | 960 x 540 | 600 |  | ≤ 30 fps |
| R4 | 960 x 540 | 900 |  | ≤ 30 fps |
| R5 | 960 x 540 | 1600 | 400 at 1280 x 720 | Same as source |
| R6 | 1280 x 720 | 2400 |  | Same as source |
| R7 | 1280 x 720 | 3400 | 550 at 1920 x 1080 | Same as source |
| R8 | 1920 x 1080 | 4500 |  | Same as source |
| R9 | 1920 x 1080 | 5800 | 1150 at 2560 x 1440 | Same as source |
| R10 | 2560 x 1440 | 8100 | 1750 at 3840 x 2160 | Same as source |
| R11 | 3840 x 2160 | 11600 |  | Same as source |
| R12 | 3840 x 2160 | 16800 |  | Same as source |

An advantage that this could introduce is that this could considerably reduce the storage required to support the additional intermediate bitrates that the enhancement layers could result in. In the above example, if additional streams would be introduced, that would increase bitrate requirements by 23.4Mbps, an increase of ~30% in storage compared to the current number of streams, while scalability would only require ~4Mbps, an increase in storage of only ~7%. Alternatively, a service may decide to convert some of the existing bitstreams to enhancement layers and save on storage, while retaining the content instead of phasing them out from their service a bit too early. Even if storage is becoming cheaper, deploying new storage systems can be quite expensive while such storage is preferred to be used to store new content.

In addition to storage savings, encryption/decryption complexity may also be reduced. It would be sufficient to only encrypt the base layer signals and not the enhancement layers, which would reduce the overall complexity of decrypting the video on the client.

#### 6.7.2.2	Transport of Scalable HEVC

##### 6.7.2.2.1	Carriage in ISO BMFF

The carriage of scalable HEVC is specified in detail in [13] as one of the "Layered HEVC ((L-HEVC) extensions", including SHVC, MV-HEVC, and 3D-HEVC. Clause 9 of [13] specifies the L-HEVC elementary stream and sample definitions.

##### 6.7.2.2.3	Support in CMAF

Carriage of scalable HEVC is specified by ISO/IEC 23000-19 (CMAF) [33] Annex H.

Currently however, the CMAF specification restricts the spatial resolution of the enhancement layer be to be either 1.5, 2, or 3 times that of the base layer both horizontally and vertically in Annex H.4.2.2 (General constraints). This raises some issues:

1. It omits the spatial resolution ratio of 1.0 for the enhancement layer that can be used for purposes beyond spatial resolution scalability, e.g., to provide bit-depth scalability.

2. These 3 ratios omit several other possible ratios, e.g., going beyond the ratio value of 3, or using some other typical ratios such as 1.25.

Based on this, MPEG has started studying this issue in [36] to ensure if such limitations can be addressed without creating any backward compatibility issues.

Editor's note: The progress on the MPEG CMAF technologies under consideration needs to be tracked in the TR and concluded by SA4#127.

### 6.7.3	Evaluation

Editor's note: Further work on evaluation is FFS.

#### 6.7.3.1 Assessment/discussion of hardware impact

The difference of HEVC and SHVC implementation is a high-level employing same low level coding tools, hence the hardware impact on implementations is manageable.

#### 6.7.3.2 Performance evaluation

Based on the representative scenario evaluation, using the scalable streams save 23% of the otherwise required additional storage. Finally, some information about the performance of SHVC in different application scenarios is documented in [30] and [31].

## 6.8	Solution #4.1: MV-HEVC with auxiliary depth/alpha channels

### 6.8.1	Introduction

This solution explores the use of auxiliary alpha or depth channels, complementary to an HEVC bitstream to enable rendering optimization based on the auxiliary alpha/depth channels. This can be done in two ways:

- Solution 4.1-A: An MV-HEVC bitstream carrying a single video layer and alpha/depth video channels.

- Solution 4.1-B: Multiple MV-HEVC bitstreams, each carrying a texture layer and with alpha/depth channels.

### 6.8.2	High-level Description

#### 6.8.2.1	Introduction

This solution explores the usage of MV-HEVC to carry the alpha and depth information as auxiliary channels. The carriage of such data is described in clause 6.8.2.2.

Additional information on possible SEI messaging transmitted to drive pose-correction is also documented for information in clause 6.8.2.3 but is not supported at this stage by OpenXR APIs and thus is excluded from this evaluation.

#### 6.8.2.2	Carriage of alpha and depth auxiliary channels with MV-HEVC

The usage of auxiliary pictures in HEVC is part of the multi-layer extensions. The carriage of auxiliary data such as depth or alpha channels is defined by the ScalabilityId signalled through the scalability_mask_flag in the Video Parameter Set (VPS). This is possible by configuring the scalability mask index to ‘3’, the value reserved for enabling “Auxiliary” as scalability dimension, as highlighted in yellow in Table 1.

Table 1: Mapping of ScalabilityId to scalability dimensions, as specified in HEVC (see Table F.1)

| Scalability mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | Texture or depth | DepthLayerFlag |
| 1 | Multiview | ViewOrderIdx |
| 2 | Spatial/Quality scability | DependencyId |
| 3 | Auxiliary | AuxId |
| 4-15 | Reserved |  |

The selection of alpha/depth auxiliary pictures is then set by the AuxId which can be configured as defined in the Table below. Setting value ‘1’ would signal the auxiliary picture is an Alpha plane while ‘2’ would indicate a depth picture. Additional information about how to interpret and process those channels can be carried in SEI messages, through the Alpha channel and depth representation information SEI messages.

Table 2: Mapping of AuxId to the type of auxiliary pictures, as specified in HEVC (see Table F.2)

| AuxId | Name of AuxId | Type of auxiliary pictures | SEI message describing interpretation of auxiliary pictures |
| --- | --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane | Alpha channel information |
| 2 | AUX_DEPTH | Depth picture | Depth representation information |
| 3..127 |  | Reserved |  |
| 128..159 |  | Unspecified |  |
| 160..255 |  | Reserved |  |

#### 6.8.2.3	Additional information on SEI messages

Additionally, alternative SEI messages can be carried to indicate how the picture texture should be rendered and processed in the device, based on information carried through the alpha or depth channel. In the case of pose-correction parameters, the GUI can be isolated from the rest of the picture through specific depth ranges, or alpha values. The strength or sensibility to the pose-correction can be also indicated for each depth or alpha range value.

A specific SEI message is needed to carry out this information, which can be done for example through a private ITU-T 35 message, or by defining a new one in MPEG. A message carrying the desired information is provided in the Table 3. The provided SEI handles all possible scenarios.

Table 3: Possible payload for pose-correction parameters SEI

| pose_correction_parameters( payloadSize ) { | Descriptor |
| --- | --- |
| pcp_metric | u(1) |
| pcp_n_intervals | u(16) |
| for ( i=0; i<pcp_n_intervals; i++){ |  |
| pcp_interval_upper_bound[i] | u(16) |
| pcp_interval_correction_sensitivity[i] | u(16) |
| } | u(1) |
| } |  |

With the following semantic:

- pcp_metric indicates what metric is used to extract different layers from video texture. 0 means alpha ranges are used, 1 means depth ranges are used.

- pcp_n_intervals indicates in how many intervals the layering is described for the selected metric.

- pcp_interval_upper_bound[i] indicates the upper bound value of the i-th interval.

- pcp_interval_correction_sensitivity[i] indicates the intensity of pose correction that should be applied on the i-th interval in the received frame. 0 means no pose correction should be applied, other values describes different degrees of pose-correction sensitivity.

The SEI messaging driving the pose-correction is depicted here for information but is currently not supported by OpenXR APIs and is then not included in the performance evaluation.

### 6.8.3	Evaluation

#### 6.8.3.1 Assessment/discussion of hardware impact

This potential solution requires the device to decode the auxiliary channels and forward them to the XR runtime. This task is expected to be straightforward and light in terms of processing. A software update on top of an existing 4:2:0 decoder is expected to be sufficient to enable the feature with minimal complexity and power consumption overhead. To assess this impact, a complexity evaluation needs to be included in the evaluation.

#### 6.8.3.2 Codec performance evaluation

In this scenario, additional data is carried, through auxiliary pictures. As two solutions are possible, the performance should be evaluated as follows:

- Solution 4.1-A: a MV-HEVC bitstream + alpha/depth channel

- Solution 4.1-B: multiple HEVC bitstreams

For the two scenarios, the performance is measured in terms of BD-RATE overhead, compared to the legacy single-layer approach. This performance measurement needs to be included in the evaluation.

Editor’s NOTE: MeCAR is in the process to define a simple format for this purpose. The usage of MV-HEVC with auxiliary pictures should be evaluated against this reference as well.

# 7	Conclusions

## 7.1	Conclusions for scenario #1:

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-08 | SA4#125 | S4-231295 | - | - | - | Skeleton | 0.0.1 |
| 2023-08 | SA4#125 | S4-231294 | - | - | - | Implements agreements in: S4aV230053 (On HEVC Multiview coding) | 0.0.2 |
| 2023-08 | SA4#125 | S4-231550 | - | - | - | Implements agreements in: S4-231289 (On HEVC Multiview coding), S4-231536 (On HEVC 4:4:4 coding), S4-231291 (On HEVC scalable coding) | 0.1.0 |
| 2023-11 | SA4#126 | S4-232006 | - | - | - | Implements agreements in: S4-231818 (Updates on MV-HEVC), S4-231819 (Latency sensitive multiview), S4-231820 (Updates on scalable HEVC coding), S4-232040 (Pose correction optimisation), S4-232036 (Scope and background) | 0.2.0 |
| 2023-12 | SA#102 | SP-231304 |  |  |  | Version 1.0.0 created by MCC | 1.0.0 |