| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.113 | CR |  | rev |  | Current version: | 1.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | JSON based Quality reporting scheme information for RTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | iRTCW |  |  |  |  |  | Date: |  |  | 29th February 2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This pCR proposes the JSON based quality reporting scheme information used for real-time media communication. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Update to clause 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Introduction

The 5GMS protocols specifications TS 26.510 and TS 26.512 discusses the procedures and APIs for metric measurement, collection and reporting in the 5G system networks for 5GMS services. The TS 26.113 addresses the procedures and APIs for Quality of Experience (QoE) metric measurement, collection and reporting in the 5G system networks for real-time media communication services which are defined in TS 26.506. The TS 26.113 documented the XML based quality reporting scheme information. In SA4 #126th meeting it was commented that the metrics reporting configuration should also support JSON based quality reporting scheme information.

Based on that discussion, this contribution proposes the JSON based quality reporting scheme information used in the metrics reporting configuration resource.

# Changes

First Change

# Provisioning interface (RTC-1)

## 6.7 Metrics Reporting Provisioning API

The semantics and XML syntax of the scheme information for the RTC quality reporting scheme are specified in Table 6.7-1 and Table 6.7-2, respectively. JSON based semantics and schema of the scheme information for the RTC quality reporting scheme are specified in Table 6.7-3, Table 6.7-4, Table 6.7-5 and Table 6.7-6 respectively.

Table 6.7-3: Semantics of Quality Reporting Scheme Information

| Property Name |  | Type | Cardinality | Description |
| --- | --- | --- | --- | --- |
|  | apn | string | 0..1 | This attribute gives the access point that should be used for sending the QoE reports. |
|  | format | string | 0..1 | This field gives the requested format for the reports. Possible formats are: "uncompressed" and "gzip". |
|  | samplepercentage | Percentage | 0..1 | Percentage of the clients that should report QoE. The client uses a random number generator with the given percentage to find out if the client should report or not. |
|  | reportingserver | Uri | 1..1 | The reporting server URL to which the reports will be sent. |
|  | reportinginterval | DurationSec | 0..1 | Indicates the time(s) reports should be sent. If not present, then the client should send a report after the streaming session has ended. If present, reportingInterval=n indicates that the client should send a report every n-th second provided that new metrics information has become available since the previous report. For each report sent, only the newly collected information since the previous report shall be reported. The value shall be greater than zero. |
|  | measureinterval | DurationSec | 0..1 | Indicates the time over which each metrics value is calculated. This field splits the session duration into a number of equally sized periods where each period is of the length specified by measureinterval field. If the "measureinterval" field is not present, the metrics resolution shall cover the period specified by the "measurerange" field. If the "measurerange" field is not present the metrics measure interval shall be for the whole session duration. |
|  | measurerange | TimeWindow | 0..1 | Indicates the time range in the stream for which the QoE metrics will be reported. There shall be only one range per measurement specification. If the "measurerange" field is not present, the metrics range shall be the whole session duration. |
|  | syncthreshold | integer | 0..1 | Indicates the maximum allowed sync loss duration between the playback time of the last played frame of the video stream and the playback time of the last played frame of the speech/audio stream. This parameter is set to control the maximum amount of allowed sync mismatch. This parameter is specified in ms. When the parameter has not been set, it defaults to 100 ms. |
|  | jitterthreshold | integer | 0..1 | Indicates the maximum allowed jitter duration between the actual playback time and the expected playback time. This parameter is set to control the amount of allowed jitter. This parameter is specified in ms. When the parameter has not been set, it defaults to 100 ms. |
|  | geographicarea | LocationFilter | 0..1 | When present, this property indicates the geographic area(s) or location(s) where quality metric collection is requested. When not present, quality metric collection is requested regardless of the device’s location. The LocationFilter object comprises one or more instances of any combination of targeted cell-IDs, polygons and circular areas. Each cell-ID entry in LocationFilter is announced in cellList, and each polygon and circular area entry is announced in the polygonList or and circularAreaList elements, respectively. |
|  | SliceScope | Snssai | 0..N | When present, this property indicates a list of network slices in which the QoE collection is requested. When not present, quality metric collection is requested for all network slices. The SliceScope is a list of S-NSSAIs. |

Table 6.7-4: Semantics of LocationFIlter

| Property Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| Cellist | Uint64 | 0..N | This property specifies a list of cell identified by E-UTRAN-CGI or CGI. |
| Shape | Shape |  | Geographic area comprising one or more instances of polygonList and/or circularAreaList property. |

Table 6.7-5: Semantics of Shape

| Property Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| polygonList | Polygon | 0..N | This property, when present, comprises a list of ‘Polygon’ shapes as defined by OMA MLP. |
| confLevel | Percentage | 0..1 | This attribute indicates the probability in percent that the RTC client is located in the corresponding polygon area. It is defined as ‘lev_conf’ by OMA MLP. If not present, it has default value of 60. |
| circularAreaList | CircularArea | 0..N | This property, when present, comprises a list of ‘CircularArea’ shapes as defined by OMA MLP. |
| confLevel | Percentage | 0..1 | This attribute indicates the probability in percent that the RTC client is located in the corresponding circular area. It is defined as ‘lev_conf’ by OMA MLP. If not present, it has default value of 60. |

Table 6.7-6: JSON schema for Quality Reporting Scheme Information

{ "title" : "QualityReportingSchemeInformation", "type" : "object", "description": "Quality Reporting Scheme Information ", "properties" : { "apn": { "type": "string", "default": , "description": "This attribute gives the access point that should be used for sending the QoE reports." }, "format": { "type": "string", "default": , "description": " This field gives the requested format for the reports. Possible formats are: "uncompressed" and "gzip"." }, "samplepercentage ": { "type": "percentage", "description": " Percentage of the clients that should report QoE." }, "reportingserver": { "type": "Uri", "description": "The reporting server URL to which the reports will be sent." }, "reportinginterval": { "type": "DurationSec", "description": "Indicates the time(s) reports should be sent." }, "measureinterval": { "type": "DurationSec", "description": "Indicates the time(s) reports should be sent." }, "measurerange": { "type": "TimeWindow", "description": "Indicates the time range in the stream for which the QoE metrics will be reported." }, "syncthreshold": { "type": "integer", "description": "Indicates the maximum allowed sync loss duration." }, "jitterthreshold": { "type": "integer", "description": "Indicates the maximum allowed jitter duration between the actual playback time and the expected playback time." }, "SliceScope": { "type": "Snssai", "description": "Iindicates a list of network slices in which the QoE collection is requested." }, "geographicarea": { "type": "LocationFilter", "description": "Indicates the geographic area(s) or location(s) where quality metric collection is requested." }, }, "required": [ "reportingserver"] }

End of First Change

# Proposal

We propose to agree the proposed changes in clause 2 into TS 26.113.