| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.510 | CR |  | rev |  | Current version: | 1.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to QoE metrics reporting configuration resource |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS_Pro_Ph2 |  |  |  |  |  | Date: |  |  | 29th January 2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This pCR proposes the changes to the metrics reporting configuration resource used in the metrics reporting provisioning API in Maf_Provisioning service. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Update to clause 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Introduction

The 5GMS protocol specifications TS 26.510 and TS 26.512 discusses the procedures and APIs for metric measurement, collection and reporting in the 5G system networks for 5GMS services. The TS 26.113 addresses the procedures and APIs for Quality of Experience (QoE) metric measurement, collection and reporting in the 5G system networks for real-time media communication services which are defined in TS 26.506.

This contribution proposes the updates to the metrics reporting configuration resource used in the metrics reporting provisioning API. The updated JSON schema of the Metrics reporting provisioning API was provided in the attached YAML file (TS26510_Maf_Provisioning_Service_MetricsReportingProvisioning.yaml).

# Changes

First Change

# Maf_Provisioning service

## 8.10 Metrics Reporting Provisioning API

### 8.10.3.1 MetricsReportingConfiguration resource

Table 8.10.3.1: Definition of MetricsReportingConfiguration resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| metricsReportingConfigurationId | ResourceId | 1..1 | An identifier for this Metrics Reporting Configuration assigned by the Media AF that is unique within the scope of the enclosing Provisioning Session. |
| scheme | Uri | 0..1 | The QoE metrics scheme associated with this Metrics Reporting Configuration. Omitting this property signals to the Media AF that metrics reporting is currently disabled for the Provisioning Session in question. |
| dataNetworkName | Dnn | 0..1 | Identifies the Data Network which shall be used when sending metrics reports. If not specified, the default Data Network shall be used. |
| reportingInterval | DurationSec | 0..1 | The time interval between successive metrics reports to be sent by the Media Session Handler. The value shall be greater than zero. If not specified, a single final report shall be sent after the media delivery session has ended |
| samplePercentage | Percentage | 0..1 | The proportion of media delivery sessions for which QoE metrics shall be reported, expressed as a floating-point value between 0.0 and 100.0. If not specified, reports shall be sent for all media delivery sessions. |
| urlFilters | array(String) | 0..1 | If present, a non-empty list of Media Entry Point URL patterns for which QoE metrics shall be reported. If not specified, reporting shall be done for all media delivery sessions initiated within the scope of the parent Provisioning Session. |
| samplingPeriod | DurationSec | 1..1 | The time interval the Media Client should wait between sampling the QoE metrics specified by this metrics reporting configuration. If samplingDuration value is present, the SamplingPeriod value shall be equal to or less than the samplingDuration value. |
| samplingDuration | TimeWindow | 0..1 | If present, the time duration specified in the media stream for which the QoE metrics will be reported by the Media Client. There shall be only one range per measurement specification. If the "samplingDuration" field is not present, the metrics range shall be for the whole session duration. |
| metrics | array(String) | 0..1 | If present, a non-empty list of metrics which shall be collected and reported. A controlled vocabulary of QoE metrics shall be specified by each metrics scheme for use with this property. If omitted, the complete (or default, as applicable) set of metrics associated with the specified metrics scheme shall be collected and reported. |

End of First Change

# Proposal

We propose to agree the proposed changes in clause 2 into TS 26.510.