Agenda item: 	10.7

Source: 	Qualcomm Inc.

Title: 	Potential LS on the PDU Set marking configuration within the Policy APIs

Document for	Discussion and Agreement

# Introduction

At the post SA4 #126 RTC Telco on January 10, 2024, it was agreed to send CT3 and CT4 an LS S4aR230168, “SA4 LS S4aR230168 “LS on PDU Set marking configuration within the Policy APIs”. The LS informs CT3 and CT4 of what information is carried in the PDU Set marking configuration, which includes a field pduSetSizeActive, an optional field in the PDU Set marking RTP header extension. The number of PDUs in the PDU Set (NPDS) was agreed at the SA4#126 meeting and it optional. It needs to be included in the PDU Set marking configuration.

# Proposal

We propose to send an updated LS to CT3 and CT4, with the following changes to the PDU Set marking configuration in S4aR230168:

Table 1: PDU Set marking configuration

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| serviceDataFlowDescription | ServiceDataFlowDescription | 1..1 |  | The 5-Tuple that identifies the service data flow for which the QoS dynamic policy is requested. |
| headerExtensionVersion | Integer | 1..1 |  | The RTP header extension version is set to 18 |
| localIdentifier | Integer | 1..1 |  | A unique identifier of the RTP header extension in the scope of the media session. |
| format | Boolean | 0..1 |  | Indicates if a short or a long header extension format is used. When set to false, a short 1-byte header extension format is being used. |
| pduSetSizeActive | Boolean | 0..1 |  | A flag to indicate if the PDU Set size in bytes is present in the RTP header extension. |
| numPdusInPduSetActive | Boolean | 0..1 |  | A flag to indicate if the Number of PDUs in the PDU Set is present in the RTP header extension. |