3GPP TSG-SA WG4 Meeting #127	S4-240294

# Sophia-Antipolis, FR, 29th January 2024 - 2nd February 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119 | CR | <CR#> | rev | <Rev#> | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous fixes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 22-01-2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-198 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adressing the open issues regarding normative aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Miscellaneous changes are proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.2, 6.1, 7.1.1, 10.2.5, A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE #1

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Frame of Reference: an abstract coordinate system whose origin, orientation, and scale are specified by a set of reference points

Reference Points: geometric points whose position is identified both mathematically and physically.

Swapchain: a queue of images shared between the XR Application and the XR Runtime

XR Application: application running on an XR Device which offers an XR experience based on an XR Runtime

XR Device: a device capable of offering an XR experience.

XR Runtime: Set of functions provided by the XR Device to the XR Application in order to create XR experiences.

XR Runtime API: the API to communicate with an XR Runtime

XR Session: an application’s intention to present XR content to the user.

XR System: a collection of resources and capabilities from the XR Runtime exposed to the XR Application for the duration of the XR Session.

XR View: a rendered view of the scene generated by the XR Application and passed on to the XR Runtime during a running XR Session

XR Space: A frame of reference with a known relationship with the real world

Warping: correcting the rendered image based on the latest head pose estimation

CHANGE #2

## 4.2	Media pipelines and rendering loop

### 4.2.1	General

In the context of this specification, media to be rendered and displayed by the XR Device through the XR Runtime is typically available in an compressed form on the device.Media is accessed using a 5G System, decoded in the device using media capabilities, and then the decoded media is rendered to be provided through swapchains to the XR Runtime as shown in Figure 4.2.1-1.

Figure 4.2.1-1 Media pipelines: Access, decoding and rendering

The rendering function is responsible to adapt the content to be presentable by the XR Runtime by making use of a rendering loop and using swapchains. The application configures pipeline of different processes, namely the media access, the decoding and the rendering. The static information describing the current rendering session need to be provided to the rendering step needs in order to configure the number of composition layers as well as each composition layer appropriately including:

-	View configuration

-	Blend modes

-	XR spaces

-	swap chain formats and images

-	projection layer types

CHANGE #3

## 6.1	XR system capabilities

The XR Runtime comprises functions and hardware components present on the XR Device. However, those functions and hardware components are not directly exposed to the XR Application. Instead, the XR Runtime offers its functions and hardware components via an XR System. A single XR Runtime may expose more than one XR Systems for targeting different purposes, e.g., a handheld device may have two XR Systems, one when the user holds the device and one when the device is inserted into an HMD. At the start of the XR Application, the XR Application is expected to query what XR Systems are available on the XR Device and select one of them to create the XR Session.

Table 6.1-1 provides capabilities for XR Runtimes exposed through an XR System. This table does not prescribe support for any specific capabilities of an XR device as defined in clause 4.3. The support of XR System capabilities is defined per device category in clause 10. A mapping of these high-level capabilities to XR frameworks are provided in Annex B.

Table 6.1-1	XR System capabilities

| Capability | Description | Parameters | Value type | Parameter definitions |
| --- | --- | --- | --- | --- |
| Create a XR System | An application can create a XR System from an XR Runtime. | xrSystemIdentifier | integer | Identifier of a given XR System exposed by a XR Runtime. |
| Query XR System’s graphics properties | An application can query an XR System about its graphics capabilities. | swapchainSupported | boolean | Indicates whether the XR System supports the swapchains. |
|  |  | maxSwapchainImageHeight | integer | The maximum swapchain image pixel height supported by this XR system. |
|  |  | maxSwapchainImageWidth | integer | The maximum swapchain image pixel height supported by this XR system. |
|  |  | maxLayerCount | integer | The maximum number of composition layers supported by this XR system |
| Query XR System’s tracking properties | An application can query an XR System on the tracking capabilities. | orientationTracking | boolean | Indicates whether the XR System supports orientational tracking of the view pose(s), or not. |
|  |  | positionTracking | boolean | Indicates whether the XR system supports positional tracking of the view pose(s), |
| Enumerate XR System’s supported environment blend modes | An application can query an XR System about its supported environment blend modes, see clause [xxx]. | blendMode | ['opaque', 'additive', 'alpha_blend'] | Indicates the type of blend mode supported by the XR System. The value 'opaque' relates to the opaque blend mode, the value 'additive' to the additive blend mode and 'alpha_blend' to the alpha blend mode. |
| Enumarate supported view configuration types | An application can query an XR System about the its supported primary view configurations. | viewConfigurationPrimary | ['monoscopic', 'stereoscopic', 'other'] | Indicates the type of primary view configuration of the XR System. The value 'monoscopic' relates to a single view, the value 'stereoscopic' to the left and right-eye views and 'other' to a type undefined in the scope of this specification. |
| Enumerate the view configuration properties | An application can list the properties associated with different view configurations advertised by an XR System. | recommendedImageRectWidth | integer | The optimal width of imageRect to use when rendering this view into a swapchain. |
|  |  | maxImageRectWidth | integer | The maximum width of imageRect supported when rendering this view into a swapchain. |
|  |  | recommendedImageRectHeight | integer | The optimal height of imageRect to use when rendering this view into a swapchain |
|  |  | maxImageRectHeight | integer | The maximum height of imageRect supported when rendering this view into a swapchain. |
|  |  | recommendedSwapchainSampleCount | integer | The recommended number of sub-data element samples to create for each swapchain image that will be rendered into for this view. |
|  |  | maxSwapchainSampleCount | integer | The maximum number of sub-data element samples supported for swapchain images that will be rendered into for this view. |
| Enumerate reference space types | An application can query an XR System about the supported reference space types, described in [xxx]. | referenceSpaceView | ['view', 'local', 'stage', 'unbounded', 'user_defined'] | Indicates the type of reference spaces supported by the XR System. The value 'view' relates to view reference space, the value 'local' to the local reference space, the value 'stage' to the stage reference space, the value 'unbounded'. |
| Query the spatial range boundaries | An application can query the spatial ranges in which an XR experience may be rendered. | 2DSpatialRangeBoundaries | rectangle | Provides the rectangle centered on the origin of a given reference space in which the user can freely move. |
| Enumerate swapchain image formats | An application can query the swapchain image formats supported by an XR System. | swapchainImageFormatIdentifier | enumeration | Provides an identifier of a swapchain image format that the XR System supports. |
| Enumerate swapchain images | An application can list the swapchain images allocated to a swapchain. | numberSwapchainImages | enumeration | Provides the number of images allocated for a given swapchain. |
|  |  | swapchainImages | object | Provide the implementation-specific swapchain image objects for a given swapchain. |
| Enumerate composition layer type | An application can list the composition layer types supported by an XR System. | compositionLayerProjection | ['projection', 'quad', 'cylinder', 'cube', 'equirectangular', 'depth'] | Indicates the type of composition layers supported by the XR Systems supports. The value 'projection' represents planar projected images, one rendered for each eye using a perspective projection. The value 'quad' represents quad composition layers which are useful for rendering user interface elements or 2D content on a planar area in the world. The value 'cylinder' represents cylinder composition layers which maps the texture onto the inside of a cylinder section. The value 'cube' represents cube composition layer which consists of a cube map with six views to be rendered by the application. The value 'equirectangular' represents equirectangular composition layers which consists of an equirectangular image that is mapped onto the inside of a sphere in the world. The value 'depth' represents depth composition layers which allows submitting depth maps as an extra composition layer to be used by the XR Runtime for pose correction. |

CHANGE #4

### 7.1.1	Single decoder instance

The following video decoding capabilities are defined:

-	AVC-FullHD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 4.0 [7] bitstreams.

-	AVC-UHD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 5.1 [7] bitstreams with the following additional requirements:

-	the maximum VCL Bit Rate is constrained to be 120 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively; and,

-	the bitstream does not contain more than 10 slices per picture.

-	AVC-8K-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 6.1 [7] bitstreams

.

-	HEVC-FullHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1 [8] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1 and general_frame_only_constraint_flag equal to 1.

-	HEVC-UHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 5.1 [8] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1 and general_frame_only_constraint_flag equal to 1.

-	HEVC-8K-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 6.1 [8] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1 with the following additional requirements:

-	the bitstream does not exceed the maximum luma picture size in samples of 33,554,432; and,

### -     the maximum VCL Bit Rate is constrained to be 80 Mbps with CpbVclFactor and CpbNalFactor being fixed to be 1000 and 1100, respectively.

CHANGE #5

### 10.2.5	Scene Description capabilities support

A device of type 1 should support glTF-based scene description as defined in clause 7.3.2.

If gltf-based scene description is supported, the following requirements and recommendation hold:

-	The SD-Rendering-gltf-Core capabilities should be supported.

CHANGE #6

END OF CHANGES