3GPP SA4#127	S4-240362

Revision of S4-240235

29 January – Feb 3, 2024, Sophia Antipolis, France

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | pseudo | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP] On NPDS in the RTP header extension for PDU Set marking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP |  |  |  |  |  | Date: |  |  | 31/01/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The number of PDUs in the PDU set (NPDS) field was adopted to be part of the PDU Set marking RTP header extension at the SA4 #126 meeting with the intent that the UPF can use it to correct the PDU Set size due to NAT46/NAT64, as recommended by the word “should” in Note 6 in clause 4.4.2.4. However, (1) SA2 does not think NAT46/NAT64 is the only cause of the PDU Set size inaccuracy, according to SA2 LS S4-231612/S2-2311897, quoted “the UPF cannot be aware of any PDU/PDU set size change caused by, e.g., the NAT function due to the IP conversion and/or other functionalities in N6 which may impact the PDU size”. (2) Even for NAT46/64, there are multiple potential solutions, as discussed in a session dedicated to this topic on Nov 15, 2023 during the SA4 #126 meeting, some of which do not involve UPF. At the moment, Note 6 is not in line with the current understanding in SA2 or SA4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise Note 6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Note 6 may be misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

.

# Proposed changes

# In clause 4.4.2.4:

# * * * * 1st change * * * *

NOTE 4:	This field may be optionally present given the signaling of the “pdu-set-size” extension attribute in the SDP offer/answer negotiation as per Clause 4.4.2.5.

NOTE 5:	Guidelines to set the PDU Set Size in bytes by an Application Server are provided in Clause 4.4.2.6.3.

NOTE 6:	 The use of NPDUS for NAT46/NAT64 correction is for FFS.

# * * * * End of 1st change * * * *