Source:	Rapporteur

Title:	ISAR Pdoc on Testing Aspects for Phase/Track 2/a, v0.1.0

## Agenda Item:	14.10

1.	Introduction

# This Permanent Document describes test plan aspects for the selection testing of IVAS specific ISAR solutions targeted in Phase/Track 2/a of the ISAR work [1]. It covers the organization of the selection tests and the relevant processing and test plan aspects.

## 2	Organization of tests

# Working assumption: The selection tests of the IVAS specific ISAR solution will be organized as in-house tests. This is based on the following confirmed prerequisites:

- Number of listening experiments suitable for in-house testing: 4 (confirmed)

- Applicable test methodology: BS.1534 (confirmed)

- Number of candidate solutions suitable for in-house testing: 2 (confirmed)

Availability of suitable cross-checkers with no stake in candidate solution under test (confirmed)

# 3 Processing and test plan aspects

## 3.1	Key Elements

# The following bullets constitute key elements of a of the processing and a test plan for IVAS specific split renderer solutions.

- Test methodology

  - BS.1534 (Mushra)

- Difference scenario between assumed and actual end-device poses

  - Static within range [+-20 degrees]

  - Dynamic within range [+-20 degrees]

    - Sinusoidal [0.25 Hz]

    - Triangular [0.5 Hz]

  - Real, i.e., derived from real head tracker trajectories with

- DOF

  - 1-DOF (yaw)

  - 2-DOF (yaw, pitch)

  - 3-DOF (yaw, pitch, roll)

- Rendering simulation

  - Trajectory nullification [2]

  - Unguided end-device pose

- Audio material

  - Categories

    - Clean and noisy speech, music, critical audio items

  - Number of items per experiment

    - [12]

  - Item selection and allocation to experiments

    - Done by Audio SWG

- Test item generation:

  - Selected audio items

  - Processed simulating combo of

    - Difference scenarios (Static, dynamic sinusoidal, dynamic triangular)

    - DOF cases (1-3 DOF)

    - Rendering simulations (trajectory nullification/unguided)

- Requirement on cross-checker

  - Demonstrably not technology contributor of system under test that is exposed by the experiment

- Experiments

  - 4+4 experiments in-house by proponent repeated by cross-checker

    - Experiment 1: Testing against performance requirement for HOA3

      - Hidden reference: Native coding system (IVAS@512kbps rendered to post renderer pose)

      - LP7 anchor: Hidden reference, 7Khz LP filtered

      - 0-DOF native transcoding reference (IVAS@512kbps binaurally rendered to pre-renderer pose, IVAS stereo coded@256kbps)

      - System 1 under test

      - System 2 under test

    - Experiment 2: Testing against performance requirement for MASA

      - Hidden reference: Native coding system (IVAS@512kbps rendered to post renderer pose)

      - LP7 anchor: Hidden reference, 7Khz LP filtered

      - 0-DOF native transcoding reference (IVAS@512kbps binaurally rendered to pre-renderer pose, IVAS stereo coded@256kbps)

      - System 1 under test

      - System 2 under test

    - Experiment 3: Testing against performance requirement for MC 7.1.4

      - Hidden reference: Native coding system (IVAS@512kbps rendered to post renderer pose)

      - LP7 anchor: Hidden reference, 7Khz LP filtered

      - 0-DOF native transcoding reference (IVAS@512kbps binaurally rendered to pre-renderer pose, IVAS stereo coded@256kbps)

      - System 1 under test

      - System 2 under test

    - Experiment 4: Testing against performance requirement for ISM-4

      - Hidden reference: Native coding system (IVAS@512kbps rendered to post renderer pose)

      - LP7 anchor: Hidden reference, 7Khz LP filtered

      - 0-DOF native transcoding reference (IVAS@512kbps binaurally rendered to pre-renderer pose, IVAS stereo coded@256kbps)

      - System 1 under test

      - System 2 under test

- Systems under test

  - System 1:

    - Proponent: Dolby Sweden AB, Ericsson LM, Fraunhofer IIS, Nokia Corporation, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation

    - Main contributors to system under test that is exposed by the experiment: [Dolby, Fraunhofer IIS]

  - System 2:

    - Proponent: Huawei

    - Main contributors to system under test that is exposed by the experiment: [Huawei]

- Lab assignment

  - In-house labs

    - Experiment 1: [tba]

    - Experiment 2: [tba]

    - Experiment 3: [tba]

    - Experiment 4: [tba]

  - Cross-check labs

    - Experiment 1: [tba]

    - Experiment 2: [tba]

    - Experiment 3: [tba]

    - Experiment 4: [tba]

# References

# [1]	Tdoc S4-240403: Work Plan for the ISAR v0.5.0

# [2] 	Tdoc S4-240254: Trajectory Nullification for Binaural Renderer Evaluation, Fraunhofer IIS

Stefan Bruhn, Dolby Sweden AB; email: stefan.bruhn@dolby.com