3GPP TSG SA WG4 129-e	S4-241619

Online 19 – 23 August, 2024

Title:	[Draft] LS Reply on Application-Layer FEC Awareness at RAN and XRM_Ph2

Response to:	LS (S2-2405604) from SA2, LS (R2-2405781) from RAN2, LS (S2-2405625) from SA2, LS (R2-2405779) from RAN2

Release:	Release 19

Work Item:	FS_XRM Ph2, NR_XR_Ph3-Core, FS_5G_RTP_Ph2

Source:	SA WG4

To:	SA WG2, RAN WG2

Cc:	RAN WG3

Contact Person:

Name:	Liangping Ma

E-mail Address:	lpma DOT AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- 1. Overall Description:

In S2-2405625, the question from SA2 is:

Can NG-RAN determine whether a PDU was successfully delivered over an unacknowledged mode data bearer? If so, does NG-RAN get this information sufficiently early to decide whether or not to drop subsequent AL-FEC packets?

In R2-2405781, the answer from RAN2 is:

RAN2 does not think it is possible for NG-RAN to determine reliably whether a PDU has been successfully delivered over an unacknowledged-mode data bearer.

To SA2, SA4 would like to make the clarifications:

- For RLC UM and MDS AL-FEC codes, there is no need to have 100% reliability in determining the successful delivery of individual PDUs, as long as the probability that as many PDUs as the source PDUs are successfully delivered is high enough. Additionally, the probability can be improved if the NG-RAN collects redundant acknowledgements.

The RAN2 LS reply R2-2405779, in response to S2-2405625, states that:

For DL, RAN2 thinks that some PSDB/PSER estimation by gNB implementation is possible, but its accuracy and reliability is unclear based on existing mechanism.

To RAN2, SA4 would like to seek clarifications on the following questions:

- Does it mean RAN2 rule out XR traffic can be served by RLC acknowledged mode (AM), as long as RLC AM is configured to meet the latency requirement? (SA2 directly send LS to RAN2)

- Given the above answer in R2-2405781 and the statement in R2-2405779, does it mean that for RLC AM, accuracy and reliability in estimating PSDB/PSER is unclear based on existing mechanism?

- Does the above statement in R2-2405779 means that PSDB/PSER based QoS is questionable for now?

- 2. Actions:

- To SA2:

ACTION: SA4 kindly asks SA2 to take the above clarification (1) into account and provide feedback if any.

- To RAN2:

ACTION: SA4 kindly asks RAN2 to provide clarifications on questions (2), (3) and (4).

- 3. Date of Next SA4 Meetings:

<add telco dates that have power to send/receive LSs>

SA4#130	18th Nov – 22nd Nov 2024				Orlando, US

SA4#131	17th – 21st February 2025 				Geneva, CH