3GPP TSG-SA WG4 Meeting #129-e	S4-241441

# e-Meeting, 19 – 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.506 | CR | 0006 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Terminology correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | GA4RTAR |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Terminologies are defined for RTC specification. However, there are some ambiguous and inaccurate definition and inconsistent description in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The definition of RTC endpoint and WebRTC Framework are modified. Usage of the terminology is modified for the alighment with the definition. Added descriptions clarifying the position of WebRTC non-browser and WebRTC browser by representing the correspondence and difference between RTC and IETF-defined functional blocks. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are possible misunderstanding and incorrect implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.1.1, 4.2.12, B.2, B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

RTC Application: A Native WebRTC Application or a Web App that is compliant with the profile of a WebRTC-based application defined in the present document.

RTC endpoint: An endpoint incorporating an instance of the WebRTC Framework. UE incorporating an RTC Client is an RTC endpoint. Additionally, the Media Function in RTC AS is also an RTC endpoint.

RTC Client: UE function comprising an RTC Access Function and an RTC Media Session Handler which interacts with functions in the network and UE applications.

RTC Access Function: A set of functions including an instance of the WebRTC Framework. The RTC Access Function exchanges real-time media with one or more RTC endpoints via reference point RTC-4m or RTC-12, and the RTC Access Function exchanges signalling messages with WebRTC Signalling Function via reference point RTC-4s. Also, the RTC Access Function exposes client APIs defined in the present document to the RTC Application at reference point RTC-7 and to the RTC Media Session Handler at reference point RTC-11.

WebRTC Framework: A well-defined subset of the WebRTC protocol stack for data transport and data framing that supports real-time media communication between an RTC endpoint and its peer(s). The WebRTC Framework enables RTC endpoints to exchange real-time media within RTC sessions.

* * * Next Change * * * *

4.1.1	Definition of RTC architecture

Real-Time media Communication (RTC) over 5G system in the context of this specification is defined as the delivery of delay-sensitive media from one peer to another with support of 5G network. AR conversational service described in TR 26.998 [2] is a typical use cases for RTC, which enables end-users to directly communicate real-time media including AR/MR media content as specified in TS 26.119 [3]. As identified in clause 8.4 of TR 26.998, there may be different options to enable such AR conversational service, for example re-use of parts of MTSI as defined in TS 26.114 [10] such as the IMS data channel or 5G Media Streaming for managed services.

The overall RTC architecture is shown in Figure 4.1.1-1 as below.

Figure 4.1.1-1: Real-time media communication (RTC) in 5G System

NOTE:	The functions indicated by the yellow filled boxes are in scope of the present document for RTC. The functions indicated by the grey boxes are defined in 5G System specifications. The functions indicated by the blue boxes are neither in scope of 5G RTC nor 5G System specifications.

The media data is exchanged between two or more RTC endpoints over a 5G System as defined in TS 23.501 [11]. An RTC endpoint is an instance incorporating the WebRTC Framework configured by the RTC System defined in the present document. An RTC endpoint is typically realised by the RTC Client of a UE, but an RTC AS, possibly deployed as an edge computing server as defined in clause 4.4.2, may also play the role of RTC endpoint. The Application Provider provides a RTC Application on the UE to make use of RTC endpoint and network functions using interfaces and APIs. RTC architecture provides the core functions and entities to support WebRTC-based service over 5G System, two main functions are defined in the trusted DN.

-	RTC AF: An Application Function as defined in TS 26.501 [6], but dedicated to real-time media communication.

-	RTC AS: An Application Server dedicated to real-time media communication.

NOTE:	If both the RTC AF and RTC AS are deployed in an external DN, this is out of scope of the present document.

The detailed RTC architecture mapping to the overall high-level architecture in figure 4.1.1-1 is shown in figure 4.1.1-2 below.

NOTE 1:	Some subfunctions may not be required depending on the collaboration scenario. Description of collaboration scenario and its architecture variant are specified in annex A.

NOTE 2:	Void.

NOTE 3:	Red ovals indicate API provider functions.

NOTE 4:	The RTC Access Function may be realised by a web browser in deployments of the RTC Client that support Web App through the W3C defined JavaScript APIs including WebRTC API.

Figure 4.1.1-2: RTC General Architecture

The WebRTC Signalling Function may be co-located with the RTC AF. In such deployments, the WebRTC Signalling Function acts as an RTC AF with access to the 5G Core, and some of the RTC AF interactions with the WebRTC Signalling Function may be replaced to avoid concurrent/redundant requests from the RTC endpoint in the UE. Specifically, media session handling interactions between the RTC AF and the UE at reference point RTC-5 may be replaced by the equivalent WebRTC signalling interactions defined at reference point RTC-4.

The subfunctions inside the RTC AF, RTC AS and the RTC Client are defined in clause 4.2 and the reference points shown in figure 4.1.1-2 are defined in clause 4.3.

Two types of RTC Application are defined in the present document:

-	Native WebRTC App: An RTC Application running on the UE that makes use of client APIs at reference points RTC-6 and RTC-7.

-	Web App: A web application running in a web browser on the UE that makes use of the W3C-defined WebRTC APIs.

NOTE:	Detailed deployment architecture for the Native WebRTC App and the Web App are described in annex B.

* * * Next Change * * * *

4.2.12	RTC Access Function

A RTC Access Function is a set of functions in RTC Client in terminal that offers:

-	Access to real-time media exchanged by WebRTC Framework with one or more RTC endpoints.

-	Provision of client APIs to the Native WebRTC App, as well as W3C-defined JavaScript API including WebRTC API to Web App via RTC-7, and client APIs to the RTC Media Session Handler via RTC-11.

-	Real-time media stream and associated signalling delivery to RTC endpoint(s) and WebRTC Signalling Function(s).

* * * Next Change * * * *

B.2	RTC Application is a Native WebRTC App

B.2.1	General

The Native WebRTC App implements reference point RTC-6 for invoking media session handling functionality and reference point RTC-7 for establishing and controlling RTC session by using API provided by the RTC Access Function. The Native WebRTC App also uses reference point RTC-7 and RTC-4s to exchange RTC session signalling with the WebRTC Signalling Function in the RTC AS.

Figure B.2.1-1. RTC architecture variants for Native WebRTC App

B.2.2	Correspondence with IETF-defined WebRTC terminology

The variant corresponds to the case that WebRTC Endpoint is WebRTC non-browser defined in IETF RFC 8825 [13]. The terminology correspondence is as follows.

WebRTC non-browser: An entity consisting of Native WebRTC App and RTC Access Function depicted in Figure B.2.1-1, which has following characteristics.

- The WebRTC non-browser is a native application (typically implemented by 3rd-party application developers other than library or OS developer) using programming languages other than JavaScript and linked with the library (corresponding to RTC Access Function depicted in Figure B.2.1-1) providing U-plane protocol stack.

- The WebRTC non-browser performs C-plane signalling and terminating audio/video media and data over WebRTC data channel.

- The WebRTC non-browser has access to system APIs provided by OS (e.g., calling OS's Socket API).

- The library linked with WebRTC non-browser may assist the Native WebRTC App's C-plane signalling.

- Within the WebRTC non-browser, some internal interface for calling the library functions corresponds to RTC-7 depicted in Figure B.2.1-1.

* * * Next Change * * * *

B.3	RTC architecture for Web App

B.3.1	General

The Web App does not instantiate reference points RTC-6. Instead, it uses the W3C defined JavaScript APIs (including WebRTC API) which is exposed by RTC Access Function via RTC-7 to exchange RTC session signalling with the WebRTC Signalling Function in the RTC AS.

Figure B.3.1-1. RTC architecture variants for Web App

B.3.2	Correspondence with IETF-defined WebRTC terminology

The variant corresponds to the case that WebRTC Endpoint is WebRTC browser defined in IETF RFC 8825 [13]. The terminology correspondence is as follows.

WebRTC browser: An entity corresponding to RTC Access Function depicted in Figure B.3.1-1, which has following characteristics.

- The WebRTC browser is a web browser (e.g., Chrome, Firefox, among others) generally not modified by 3rd-party application developers.

- The WebRTC browser hosts JavaScript Applications which provide the actual services.

- The WebRTC browser implements U-plane protocol stack for the capability of terminating audio/video media and provides the control of those media by exposing WebRTC API and related other APIs to JavaScript applications.

- The WebRTC browser implements U-plane protocol stack for the capability of exchanging data over WebRTC data channel and provides the transported data by exposing WebRTC API to JavaScript applications.

- The WebRTC browser exposes WebSocket API to provide the capability of transporting signalling messages to the JavaScript application.

JavaScript API: JavaScript API stated in Section 2.2 of IETF RFC 8825 [13]. This corresponds to APIs exposed at RTC Access Function and provided via RTC-7 depicted in Figure B.3.1-1, which has following characteristics.

- Those APIs are W3C-defined JavaScript APIs implemented and exposed by the WebRTC browser and which can be utilized by the JavaScript applications including WebRTC API, and WebSocket API.

JavaScript application: JavaScript application stated in Section 3 of IETF RFC 8825 [13]. This corresponds to Web App depicted in Figure B.3.1-1, which has following characteristics.

- The JavaScript application is a Web application running on the WebRTC browser (and typically implemented by 3rd-party application developers other than library or OS developer) using JavaScript.

- The JavaScript application has the capability to control of audio/video media by using the WebRTC API and related other APIs provided by the WebRTC browser.

- The JavaScript application has the capability of exchanging data over WebRTC data channel by using the WebRTC API provided by the WebRTC browser.

- The JavaScript application terminates C-plane signalling using WebSocket API for the transport of signalling messages provided by the WebRTC browser.

* * * End of Changes * * * *