Source:	Beijing Xiaomi Mobile Software

Title:		Update proposals for TR 26.933 v1.0.0

## Document for:	discussion& agreement

## Agenda Item:	7.6

# Introduction

According to the discussion during telco in June, this contribution aims to update the chapter 7.2.4.3 HOA and chapter 9 Conclusion summary and recommendations of TR 26.933.

# Content

## Proposed updates in clause 6.4 Microphone frequency response

In current smartphones with multiple microphones, each microphone may exhibit a different frequency response. Some microphones are positioned along the device's edge alongside stereo speakers, leading to varying hardware designs that affect their acoustic performance. For instance, a common configuration includes four microphones, with three located on the device's edge.

Figure x. microphone frequency responses on the edge of smartphones.

The fourth microphone, often placed along with the camera, which makes different acoustic diffraction due to its distinct surface position compared to the other ones.

Figure x. microphone frequency responses on the back of smartphones.

The frequency response of the raw microphone signal typically appears mostly flat with a pronounced rise in certain high frequency bins, usually exceeding 10dB. To enhance sound quality, adjusting the pronounced rise to higher frequency bins can minimize its perceptual distortion.

## Proposed updates in clause 7.2.4.3	 HOA

In ambisonic microphones, a minimum of (N+1)2 microphones is required to capture N order HOA. Consequently, a second-order HOA necessitates at least 9 channels of ambisonic microphones. The  number of microphones on most smartphones is limited to 4,therefore, obtaining HOA microphone array on current microphones configuration of smartphones may be difficulty, however, there are still some relevant researches on upmixing FOA signals to HOA signals such as:

the first solution is to interpolate the FOA to HOA signal , as suggested by various studies refer to[1][2][3];The second solution is to utilize of a generative model capable of translating FOA into HOA signal, as refer to [4]; Thirdly, some signal-dependent technologies, such as parametric spatial audio, have the capability to analyse the captured signals and subsequently convert them into an excepted HOA format.

## Proposed updates in clause 7.4 Noise suppression

## 7.4 Noise suppression

The needs of noise suppression(NS) depend on certain usage scenario or sometimes specific solutions, for example, for office scenario mainly consists of speech components, NS is highly needed, however, for concert application scenario which includes various of music instruments and other atmosphere audio components, NS may not be needed, in this case ,moreover, audio scene recognition may be also needed for identifying specific scenarios

## Proposed updates in clause 9 Conclusions and Recommendations

## 9 Conclusion summary and recommendations

To effectively deploy IVAS codec, an end-to-end immersive audio solution is needed. Currently, stereo and binaural audio have already been deployed in receiving end by most UEs which have at least two speakers for stereo playback and can connect to earbuds for binaural audio playback.

It is different in the sending end, most UEs can hardly support immersive audio capture. The lack of immersive audio capture capability may presents a significant barrier to the widespread deployment of IVAS, so to provide the capability of immersive audio capture, we may significantly enhance the user experience and pave the way for the widespread deployment of IVAS codecs on market.

UE hardware and software hold the potential to deliver immersive voice and audio services with corresponding technologies. Most current UEs have more than 2 microphones, and can connect to earbuds for immersive audio playback. Among the UEs listed in clause 4, smartphones have become an indispensable part of people’s daily lives, not only as a means of real-time communication, but also as an important device for recording lives, creating contents and enjoying entertainments.

Aiming at opening the immersive audio market and accelerating the deployment of IVAS codec, an immersive audio capture solution is necessary, so, it is recommended to start a normative work of immersive audio capture for smartphones.

# Conclusion

It is proposed to include contents in chapter 2.2 and 2.4 into corresponding clauses of 26.933.

Reference:

- C. D. Salvador, S. Sakamoto, J. Treviño, and Y. Suzuki, “Enhancement of spatial sound recordings by adding,” J. Inf. Hiding Multimedia Signal Process., vol. 8, no. 6, pp. 1392–1404, 2017.

- M. Geier, J. Ahrens, and S. Spors, “Enhancing binaural reconstruction from rigid circular microphone array recordings by using virtual microphones,” in Proc. AES: Conf. Audio Virtual Augmented Reality, 2018, pp. 194–202.

- Lübeck, T.; Arend, J.M.; Pörschmann, C. Spatial Upsampling of Sparse Spherical Microphone Array Signals. IEEE/ACM Trans. Audio Speech Lang. Process. 2023, 31, 1163–1174.

- Chen, X.; Ma, F.; Bastine, A.; Samarasinghe, P.; Sun, H. Sound Field Estimation around a Rigid Sphere with Physics-informed Neural Network. arXiv 2023, arXiv:2307.14013.