3GPP TSG-SA4 Meeting SA4#129-e	 S4-241446

E-meeting 19-23 Aug 2024

# Title:	Introduction of AVS codec

# Source:	China Mobile Com. Corporation, Peng Cheng Lab, China Unicom，China Telecom, ZTE Corporation

# Document for:	Discussion

Agenda Item:	20

- What is AVS/AVS3?

AVS (Audio Video Coding Standard) is the serials of audio and video coding standards developed by Audio Video Coding Standard Working Group (AVS Working Group), which was established in June 2002. AVS standards aim to provide high-quality video compression and more efficient transmission and storage. The AVS Working Group has developed three generation video coding standards, namely AVS1/AVS+, AVS2 and AVS3, which have been respectively adopted as the HD, 4K UHD and 8K UHD industry broadcasting standards in China.

AVS3 is the third generation of AVS standards, refers to the series of standards T/AI 109.x "Information Technology: Intelligent Media Coding", the first profile of AVS3 was completed earlier in 2019. The AVS3 standard has been applied in 8K UHD live broadcasting since 2021, and in mobile live streaming broadcasting service since 2022. Starting from the first trials in 2021, AVS3 technology has been used for the live broadcasting and live streaming of several major events, including for CMG Spring Festival Gala (2021), Beijing Winter Olympics (2022),  FIFA World Cup(2022), Asian Games 2023(Hangzhou), and UEFA European Championships (2024).

- How is AVS/AVS3?

The coding performance of AVS3 is nearly 40% higher than that of the international video coding standard HEVC/H.265 on 4K UHD.

- AVS3 offers large compression efficiency over HEVC

  - 40.09% bitrate saving for 4K test sequences.

  - 35.07% bitrate saving for 1080P HD test sequences.

  - 35.00% bitrate saving for 720P test sequences.

The results are from test report of TM-AVC1269r1, which were cross-checked in DVB.  The performance comparison is based on the AVS3 reference software HPM-12.2 (AVS3) and HEVC reference software HM-16.22 (HEVC) for 4K, 1080P and 720P contents.

Compared with HEVC (HM16.22), AVS3 High Profile (HPM12.2) with GOP32 is observed 40.09% for 4K test sequences, and 36.72% bitrate saving for overall 4K, 1080P and 720P test sequences.

Compared with HEVC (HM16.22), AVS3 High Profile (HPM12.2) with GOP16 is observed 38.71% for 4K test sequences, and 34.32% bitrate saving for overall 4K, 1080P and 720P test sequences.

AVS3 Test configuration:

Test sequences, configurations and test QPs are specified in the CTC as follows:

- Test Platform:

  - HPM-12.2 for AVS3 (HPM reference software)

  - HM-16.22 for HEVC (comparative reference model)

- Common Test configuration:    Random Access (RA)

- Configuation files for RA:

  - HPM12.2   encode_RA.cfg for GOP16, encode_RA_GOP32.cfg for GOP32 with QPs:  27, 32, 38, 45

  - HM16.22   encoder_randomaccess_main10.cfg  with QPs:  22, 27, 32, 37

- Bit depth setting: input_bit_depth is set to bitdepth of test sequence, internal_bit_depth is set to 10.

- Test sequences include: 4K, 1080P, 720P. All test sequences are in color format YUV420.

- AVS3 Commercial Live Streaming

Interaction with standards organizations:AVS3 was officially included into the core specifications of the International Digital Video Broadcasting Organization (DVB) in 2022, which specifications were then following published as ETSI standards in 2023. Including AVS3 standard into DVB/ETSI bring more advanced and efficient video coding technology to the global telecommunications, broadcasting and Internet industries, and further promote the progress and development of the industry.

In July, 2022, DVB send 3GPP SA4 a liaison on its next-generation video codecs (NGVC) to notify the adoption of ITU-T H.266 | ISO/IEC 23090-3 Versatile Video Coding (VVC) and T/AI 109.2-2021: Intelligent Media Coding - Part 2: Video (AVS3) in DVB specifications, which liaison was presented as S4-221081 in 3GPP TSG SA4 #120 e-meeting from Aug 15-26, 2022.

Commercial deployment: Currently, through China Mobile’s video application Migu Video, the AVS3 standard has been successfully used in several large-scale commercial events:

-In the 2022 Beijing Winter Olympics, AVS3 was used for the first commercial trial in live streaming on mobile devices.

-In the 2022 FIFA World Cup, the AVS3 standard achieved its scale commercial use on mobile devices through the Migu Video APP.

-In the 2023 Hangzhou Asian Games, China Mobile achieved large-scale commercial use of AVS3 live streaming on mobile devices, supporting 1080p live streaming on mobile device.

-In June 2024 UEFA European Championships, China Mobile provides live streaming service of  all 51 matches, which services were provided in the AVS3 standard zone of Migu App.

-In 2024, during the ongoing Olympic Games, China Mobile has once again upgraded the support of AVS3. While creating the AVS3 on-demand zone and AVS3 multi viewpoint live streaming perspective, it has also added AVS3 multi-channel live streaming.

Live Streaming Implementation:

-On the terminal side, AVS3 is supported through an optimized software decoder, currently enabling over 270 models mobile phones in the market, including iOS and Android devices

-On transmission technologies, by building the AVS3 streaming service architecture, it supports large-scale parallel streaming services for non same source AVS3 live streams, and supports dual links high reliability transmission of AVS3 low latency live streaming. Hereby, smooth AVS3 live streaming experience is provided for video users.

-On server side, intra frame and inter frame parallization processing is used to achieve real-time AVS3 10bit encoding @50fps on general CPU servers.