3GPP TSG-SA WG4 Meeting #129-e	S4-241454

# Online, 19 – 23 August 2024

Source:	Nokia

Title:	Draft LS reply to SA2 on Periodicity and Time to Next Burst

Agenda item:	10.6

Document for:	Agreement

# Introduction

SA2 sent an LS to SA4 on Periodicity and Time to Next Burst (TTNB) [S4-241422/S2-2407351]:

SA2 has started to evaluate solutions for key issues for the study of XR and Media Services Ph2 (TR 23.700-70). SA2 thanks the LS replies for XRM Ph2 from RAN2, RAN3 and SA4 and SA2 would like to further coordinate on the following aspects: - Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension? - Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6? - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server? Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

The purpose of this contribution is to explore possible answers to these two questions.

# Background

The dynamic provision of periodicity and TTNB in GTP-U is related to the key issue #5 for QoS Handling when Traffic Characteristics Change Dynamically [23.700-70]:

Certain Rel-18 XRM features work best when the traffic pattern is mostly static. However, the traffic pattern of an XR session change (i.e. it can be dynamic) based on the usage/applications. One example case where the traffic pattern can change dynamically is the size of media frames vary due to scene changes that may occur at times such as initial start-up or due to user drags on a progress bar. Another example is related to file download (e.g. AI model update) before the session can continue. This key issue proposes to study whether and how to support dynamic change (via user plane) in traffic characteristics (e.g. burst related parameters), provided by the application in the DN. The key issue includes the following aspects: - Identify what traffic characteristics are dynamically changed. - Whether and how the 5G network can be enhanced to know about dynamic changes in traffic characteristics of GBR and non-GBR flows: - What information from the Application in the DN, if any, is needed by the 5G network to be able to know the dynamic changes in traffic characteristics and how this information is provided by the Application in the DN. - At what granularity does the 5GS need to know of changes in traffic characteristics (e.g. QoS Flow granularity). - What handling may be needed when the 5G network knows about changes in the traffic characteristics. NOTE: This Key Issue may require coordination with RAN WGs and/or SA WG4.

# Discussion

Periodicity is defined in TS 23.501 clause 5.27.2.1 as the “the time period between start of two data bursts”. It was introduced in Rel-17 as part of the Time Sensitive Communications Assistance Information (TSCAI) which describes the traffic characteristics that may be provided by the AF for use by the gNB, to allow more efficiently scheduled radio resources for periodic traffic. If enabled, TSCAI is sent to the RAN using control plane signaling. More information on TSCAI can be found here: https://www.3gpp.org/technologies/ind-5g

Time to Next Burst (TTNB) refers to the time interval between sending time of the last packet of the current burst and the first packet of the next burst by the AS. A summary of the related Rel-18 discussions in SA4 were presented in S4-241018, and the proposal for signaling TTNB in the RTP header extension for PDU Set marking was incorporated into TR 26.822 as solution #6.

As per the definition in TS 23.501, periodicity is equal to TTNB minus the duration of the data burst. In that sense, TTNB and periodicity can be considered to carry equivalent information. However, our understanding is that periodicity is intended to be a rather static value, transmitted once and remaining valid until a new value is signaled. Therefore, it does not capture the flexible transmission nature of video streams, nor the impact of rate adaptation mechanisms.

For example, if paced sending is used, as in WebRTC, the sender may decide to vary the packet sending rate to smooth the flow of packets sent onto the network rather than sending all packets of a video frame at the beginning of the frame interval. Furthermore, in streaming, gaming and video conferencing applications, adapting the frame rate helps maintain a smooth playback experience even when bandwidth is limited.

In the cases described above, a static periodicity value does not apply to all traffic types and would mislead the network. Signaling a dynamic periodicity is possible, but in our view, this would overload the definition of periodicity which refers to transmission at regular intervals and is typically associated with sampling/frame rate in audio/video processing, rather than the actual transmission time, which is what matters in this case. On the other hand, TTNB is designed to capture both periodic and infrequent/rapid non-periodic variations between data bursts, making it more suitable for dynamic signaling and a more precise timing tool for the network.

# Proposal

Based on the discussion above, it is proposed to provide the reply below to SA2. An LS draft containing this reply is provided in ZIP file.

To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

SA4 answer: Yes, the periodicity can be included, provided that the sender implementation supports it. However, SA4 views periodicity as static information tied to the media frame rate rather than a value that can vary with each burst. Therefore, SA4 would not recommend sending it over the user plane for dynamic adjustments.

To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6?

SA4 answer: Yes, it is possible. Senders can estimate how much time it will take to send all packets in a data burst and determine the start time of the next burst. Therefore, the application server can provide a reliable estimate of TTNB in the first packet of the burst.