3GPP TSG-SA WG4 Meeting #129-e	S4-241455

# Online, 19 – 23 August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.264 | CR | 0001 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial Corrections to TS 26.264 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, IBACS |  |  |  |  |  | Date: |  |  | 2024-08-13 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Several editorial corrections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added spec numbers next to the in-text citations. - Corrected minor grammatical issues. - Fixed some abbreviations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification has inconsistent references and style. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2, 5.1, 6.2, 6.3.1.2, 6.3.1.3, 6.4, 6.5.1, 7.1, 7.2, 8.2, A.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

# 4	System description

## 4.1	General

Typical conversational AR scenarios as envisioned in this document consist of an immersive AR call that may include the following conversational components:

-	Real-time speech/audio that can comprise mono, stereo, and/or spatial audio.

-	Real-time 2D video or 360-degree video that can be rendered as rectangular or spherical overlay in the AR experience.

-	A real-time volumetric video of the user or an object that can be rendered in AR or MR.

In addition to the above conversational media, non-real-time objects may be exchanged over the data channel as well.

Both two-party and multiparty calls are possible. The AR experience may be unidirectional, i.e., only one party receives AR media and renders it, or it may be bidirectional, i.e., both parties receive and transmit AR media. The term AR-MTSI client includes both:

-	an AR-MTSI client in terminal which is an AR device as defined in TS 26.119 [3] e.g., AR glasses, phone, Head Mounted Display (HMD) that has an XR Runtime for rendering an AR experience.

-	AR MF/MRF that provides support for AR conversational services.

As an AR-MTSI client in terminal is a DCMTSI client in terminal with additional features for AR communication, the following requirements for a MTSI client terminal also apply for an AR-MTSI client in terminal:

-	the interworking requirements in clause 12 of TS 26.114 [2],

-	the jitter buffer management requirements in clause 8 of TS 26.114 [2],

-	the packet loss handling requirements in clause 9 of TS 26.114 [2],

-	the media and rate adaptation requirements in clause 10 and 17 of TS 26.114 [2], and

-	the network preference management object in clause 15 of TS 26.114 [2],

NOTE:	If an AR-MTSI client in terminal supports functionalities for MSMTSI client in terminal as specified in Annex S of TS 26.114 [2], the media and rate adaptation requirements in Annex S.8 of TS 26.114 [2] also apply for an AR-MTSI client in terminal.

CHANGE 2

## 4.2	Terminal architecture

The detailed XR client architecture is not in the scope of this specification. The XR baseline client architecture can be found in TS 26.119 [3]. The pre/post-processor component in terminal provides AR capabilities for processing output of peripherals and the input/output of encoders/decoders, which may include:

-	XR runtime

-	Scene manager

-	Presentation engine

-	XR source management

The AR-MTSI client has XR Runtime capabilities for rendering AR experience, e.g., spatial localization and mapping, etc., and can support local AR rendering and network-assisted split rendering based on client’s capabilities. A UE may support multiple microphones, cameras or sensors.

An AR-MTSI client supports the protocol stack of a basic MTSI client as described in clause 4.2 of TS 26.114 [2]. For the specific  AR communication instance, AR-MTSI client can select different IMS media channel to delivery AR data to IMS network or peer UE. In general, AR media components with real-time characteristics are transported via RTP session and AR metadata is transported via data channel or RTP session with AR media.

Figure 4.2.1: Functional components of an AR-MTSI client in terminal

CHANGE 3

# 5	Immersive AR Media

## 5.1	General

An AR-MTSI client supports simultaneous transfer of multiple media components with real-time characteristics. An AR-MTSI client supports the core media components in TS 26.114 [2] for a conversational AR scenario including text, image, video and speech (also referred to as audio).

CHANGE 4

## 6.2	Metadata data channel message format

For the carriage of metadata defined in this clause the AR-MTSI clients shall use the data channel. The data channel sub-protocol shall be identified as “3gpp-ar-metadata”, which shall be included in the dcmap attribute of the SDP.

The transmission order for the data channel shall be set to in-order and the transmission reliability shall be set to reliable.

The metadata message format shall be set to text-based and the messages shall be UTF-8 encoded JSON messages.

A data channel message may carry one or more AR metadata messages as defined in Table 6.2-1.

Table 6.2-1 AR Metadata Messages Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| messages | Array(Message) | 1..n | A list of AR metadata messages. Each message shall be formatted according to the Message data type as defined in Table 6.2-2. |

Each metadata message shall follow the format specified in Table 6.2-2.

Table 6.2-2 AR Metadata Message Data Type

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | A URN that identifies the message type. |
| payload | object | 1..1 | The message payload depends on the message type. |
| sendingAtTime | number | 0..1 | The wall clock time when the AR metadata message is transmitted. (clause 9.3.2.1 in TS 26.565 [6]) |

CHANGE 5

#### 6.3.1.2	Available visualization space

An AR-MTSI client in terminal may send available visualization space, user position and other trackable poses to AR MF/MRF for scene creation and update.

The available visualization space defines an occlusion-free space around the user for rendering the AR scene as a geometric primitive. The format for available visualization space is defined in clause 12.4 of TS 26.119 [3]. The type of the message containing visualization space as a payload shall be “urn:3gpp:ar:v1:visualization-space”. The availableVisualizationSpace object [3] shall contain a xrSpaceId. The xrSpaceId is used for determining the local coordinate axis of the visualization space. The xrSpaceId shall be a unique identifier for an XR space of one AR-MTSI client in terminal. If the visualization space is sent, then initial user pose shall be sent. The user pose and visualization space are in reference to the same xrSpaceId.

If the visualization space is anchored to another trackable (instead of the user) not anchored around the user or if the viewer is not the centre of the visualization space, an initial pose for a trackable as defined in clause 6.3.1.3 may be used.

CHANGE 6

#### 6.3.1.3	Initial Pose

Trackable is a real-world object (e.g., the UE, floor, controllers, table etc.) that the UE can detect, which can be used as a reference to anchor virtual objects to the real world.

The AR-MTSI client in terminal that sends the available visualization space may also send at least one pose for a trackable within the visualization space. The AR-MTSI client in terminal may send additional poses for anchoring virtual objects. The poses shall be sent using the format defined in clause 12.2 of TS 26.119 [3]. The poseInfo (as defined in Table 12.2-1 of TS 26.119 [3]) shall contain an xrSpaceId that is the same as the one used for visualization space. The poses may additionally contain a label string to identify the type of anchor. The labels are application-dependent, but for example, user, floor, left controller etc., can be used as labels. The type of the message for a pose sent for scene creation shall be set to “urn:3gpp:ar:v1:initial-pose”.

CHANGE 7

## 6.4	Scene descriptions

An AR-MTSI client in terminal that is a compliant device type of TS 26.119 [3] shall support the capabilities requirements for scene description as described in clause 10 of TS 26.119 [3] for its respective device type.

When used in an AR call, the scene description should be the entry point to the AR session (after establishment of a regular call/conference) and shall be exchanged over the data channel as described in TS 26.114 [2]. The Scene Description is exchanged over a stream with a stream id in the range 1 to 1000 and shall be provided by the AR AS through the MF/MRF to the AR-MTSI client in terminal. In this case, no web application needs to be downloaded in this case.

Based on the information in the Scene Description, the UE may decide to add additional media streams through a re-INVITE. However, at least the RTP session for the voice of every participant should be present and should be linked to an audio source in the scene description.

NOTE:	Support for advanced audio codecs, such as IVAS, in scene description is for further study.

Each participant should be associated with their own camera node, identified through the node name, which is also provided as part of the SDP through the “sd-nodes” attribute of media session of the data channel that carries the Scene Description.

The “sd-nodes” attribute shall conform to the following ABNF syntax:

sd-nodes = “a=sd-nodes:” SP participant-label SP 1*(node-name [“,”])

participant-label = char-val

node-name = char-val

The AR MF/MRF should apply pose updates from the received pose information of each participant to their respective camera nodes, as negotiated by the SDP sd-nodes attribute.

A scene description of an AR session may be sent from the AR MF/MRF to the AR-MTSI clients in terminal.

An AR MF/MRF that supports scene description shall support:

-	The capability to generate a scene description file that conforms to the SD-Rendering-glTF-Core capability as defined in TS 26.119 [3].

-	The capability to generate and update a scene description file that conforms to the SD-Rendering-glTF-Ext1 as specified in TS 26.119 [3].

An AR MF/MRF that supports scene description may additionally support the generation of scene description files and updates that conform to the SD-Rendering-glTF-Ext2 capabilities as defined in TS 26.119 [3].

An AR MF/MRF that supports scene description may additionally support the generation of scene description files and updates that conform to the SD-Rendering-glTF-Interactive capabilities as defined in TS 26.119 [3].

In addition, an AR MF/MRF that supports scene description shall support the referencing of RTP streams in the scene description through the MPEG_media extension as defined in ISO/IEC 23090-14 AMD 2 [7]. The external media shall be RTP media streams supported by an AR-MTSI client and signalled in the SDP.

When scene description is not used as the entry point, the scene description shall be sent by the AR MF/MRF to the AR-MTSI client in terminal over the application data channel. The type of the message shall be set to “urn:3gpp:ar:v1:sd”.

An AR MF/MRF that supports scene descriptions should create and distribute the scene for an AR call with audio and video streams based on the visualization space, viewer position and AR media properties. The AR MF/MRF should create the scene description for each participant (AR-MTSI client in terminal) such that the shared experience is symmetrical for the different users in the call, e.g., to maintain relative position of users and objects.

CHANGE 8

## 6.5	Network media rendering

### 6.5.1	General

The AR-MTSI client in terminal supporting network media rendering (of AR Media objects or 3D scenes) shall support metadata formats for split rendering as specified in clause 8.3 of TS 26.565 [6].

NOTE:	In case network media rendering is used, AR Media objects or 3D scenes are rendered in a split rendering server (AR AS) based on the XR pose information and actions of the AR-MTSI client in terminal. The split rendering server renders a view of the scene (or object) based on the viewing angles and XR pose, provided by the AR-MTSI client in terminal. The resulting images are sent as video streams from the split rendering server to the AR-MTSI client in terminal, which renders the videos at their respective position in AR.

CHANGE 9

# 7	Media configurations

## 7.1	General

The media configuration requirements for MTSI clients in terminals specified in TS 26.114 [2], clause 6, also apply for AR-MTSI client in terminal.

An SDP framework for AR data exchange for AR communication is presented to negotiate codec support for AR media, AR metadata, as well as RTP/RTCP signalling necessary for AR media rendering processing.

AR-MTSI client in terminal shall use RTP for the real-time transport of AR media for AR communication. Any AR media as an overlay may refer to the overlay configuration described in Annex Y.6.4.3 of TS 26.114 [2].

AR-MTSI client in terminal shall use data channels for exchange of AR metadata and rendering negotiation. The SDP attribute “3gpp_armetadata_types” should be used to indicate the types of AR metadata which defined in clause 6 (e.g. pose, action and scene description) within the data channel.

The syntax for the SDP attribute is:

a=3gpp_armetadata_types: <metadata-1> / … / <metadata-N>

Poses as part of AR metadata may be transmitted via RTP session in a RTP header extension as specified in clause 4.3 of TS 26.522 [8].

CHANGE 10

## 7.2	Network media rendering configuration

The AR-MTSI client in terminal shall indicate its support for AR calls by including the “webrtc-datachannel” in the “+sip.sub-type” Contact header field.

A new Contact header field parameter, “+sip.3gpp-ar-support” is used to indicate the level of support for AR calls. The possible values for the “3gpp-ar-support” parameter are:

-	“ar-capable”: indicates that the terminal is fully capable of receiving and rendering AR media as described by the capabilities in [2] clause 9.2.

-	“ar-assisted”: indicates that the terminal is capable of transmitting AR metadata on the uplink. However, the UE has no support for processing and rendering a 3D scene. The participation in an AR call requires the deployment of network rendering. The rendered view(s) are controlled by the pose information that is shared by the terminal.

In the absence of the “+sip.3gpp-ar-support”, it shall be assumed that the terminal has no support for AR calls. In this case, the MTSI client can only participate in the AR call if network rendering is offered.

An AR-MTSI terminal that intends to participate in an AR call shall register with the “ar-capable” value for the “+sip.3gpp-ar-support” parameter and shall offer/answer an SDP that includes a data channel with the sub-protocol “mpeg-sd”. Any updates that the AR-MTSI terminal intends to share, including pose updates, will be sent as scene updates to the AR AS. An AR-MTSI terminal that intends to participate in an AR call with the support for network rendering shall register with the “ar-assisted” value for the “+sip.3gpp-ar-support” parameter and shall offer/answer an SDP that includes a data channel with the sub-protocol “3gpp-sr-metadata”. Pose updates that are to be used for the rendering are shared as pose predictions with the MF/MRF.

As specified in Annex AC.9 of TS 23.228 [4], the AR application server may provide network assisted rendering. An AR-MTSI client in terminal can decide to request network media rendering based on user selection and its status such as power, signal, computing power, internal storage, etc. The AR-MTSI client in terminal shall complete an AR media rendering negotiation with the AR AS before it initiates subsequent procedures to activate the network media rendering. The data channel should be established for rendering negotiation with SDP offer/answer between AR-MTSI client in terminal and MF/MRF with the sub-protocol “3gpp-sr-conf”, and continue to be used for rendering re-negotiation until the end of the AR communication.

An AR-assisted terminal that intends to deploy network rendering for AR media rendering, shall use the negotiation processes between the AR-MTSI client in terminal and the AR AS to determine the split rendering configuration. The split rendering configuration shall be in JSON format as specified in clause 8.4.2 of TS 26.565 [6]. The exchange of the configuration information shall take place using the established MTSI data channel. The split rendering configuration message shall be formatted according to clause 8.4.2.2 of TS 26.565 [6] and shall have the type: “urn:3gpp:split-rendering:v1:configuration”. The output description message shall be formatted according to clause C.1.4 of TS 26.565 [6] and shall have the type: “urn:3gpp:split-rendering:v1:output”.

For a terminal that does not support AR calls, the IMS AS may trigger network rendering on behalf of the terminal upon receiving an (re)INVITE for an AR call. The output format for the rendered media shall be conformant to clauses 10.4.3 and 10.4.4 of TS 26.119 [3]. The MF/MRF that performs the remote rendering shall select a suitable rendering viewpoint for the session, e.g. a selected viewpoint in the scene or the initial viewpoint for the participant as assigned by the AR AS in the scene description. In case no network rendering can be setup, the IMS AS should reject the call.

The IMS AS detects support for AR capabilities based on the presence or absence of the “+sip.3gpp-ar-support” parameter of the Contact Header Field in the REGISTER message.

CHANGE 11

## 8.2	RTP transport

Additionally to the requirements specified in TS 26.114 [2], clause 7, the RTP Header Extension for PDU Set Marking (clause 4.2) and RTP Header Extension for XR Pose (clause 4.3) specified in TS 26.522 [8] also apply for AR-MTSI clients in terminals.

CHANGE 12

## A.1.4	Scene Description Processing

Figure A.1.4-1 illustrates a detailed call flow for scene description processing procedure.

Figure A.1.4-1: Call Flow for Scene Description Processing

The steps are as follows:

1.	The UE1 initiates an AR communication session and establishes audio and video session connections with the UE2. Then the bootstrap and application data channels are established for the UE1 and UE2.

2.	The split rendering negotiation procedure has been finished.

3.	MF/MRF prepares the scene description based on media descriptions and assets for the call.

4.	MF/MRF delivers the scene description to the UEs.

5.	A UE may trigger a scene update e.g., when a new object is added/removed in the scene, or a spatial information update is sent or UE capabilities change. The figure shows the update is triggered by UE1, but this can be either UE.

6.	The MF/MRF will process the new information and creates a scene description update. It is also possible for the MF/MRF to initiate an update without an update from the UEs, for example if link conditions change.

7.	MF/MRF distributes scene description update to all UEs.

NOTE:	Spatial data related updates may be required for collaborative AR calls, e.g., when multiple users are physically collocated and also part of the same AR experience. The type of spatial description updates is for further study.

8.	Subsequent procedures continue.