3GPP TSG-SA4 Meeting #129-e	S4-241470

# Online, , 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0017 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] SCONE-PRO and 5G Media Streaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At recent IETF meetings, the issue of Secure Communication of Network Properties (SCONE-PRO) had been discussed. It was highlighted in several inputs that bandwidth is and remains a scarce resource, and that video is and will stay the dominant from of media on the Internet. Despite continuous capacity investments, it is hard to keep up with demand needed for video delivery. The Secure Communication of Network Properties (SCONEPRO) Working Group's primary objective is to specify a 'maximum achievable throughput' property for QUIC-based streaming video and an on-path protocol for securely communicating this property from a network device to a client endpoint. As 5G Media Streaming provides functionalities that are similar and aligned to the objectives of the IETF work, this key issue will address aspects that investigate how the requirements from SCONE-PRO are met by 5G Media streaming, to what extent SCONE-PRO can be combined with 5G Media Streaming and extensions to 5G Media Streaming would be suitable to address combination with SCONE-PRO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addresses the work item objectives for this key issue - Documents the key issue in more detail, in particular how they relate to the 3GPP Media Delivery architecture and/or the MBS User Service architecture - Studies collaboration scenarios between the Application Service Provider and the 5G System and for each of the key topics. - Based on existing architectures, provides one or more deployment architectures that address the key topics and the collaboration models. - Maps the key topics to basic functions and develop high-level call flows. - Identifies the issues that need to be solved. - Provides candidate solutions including call flows, protocols and APIs for each of the identified issues. Identifies gaps and recommend potential normative work for stage-2 and stage-3, including which existing specifications would be impacted and/or if any new specifications would preferably be developed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.X (new), 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Updated scope, references and abbreviations are in CR 0014 References Abbreviations: SCONE Secure Communication of Network Properties CMCD Common-Media-Client-Data The CR is submitted for endorsement. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

## 2	References

[SCONE-PRO]	Secure Communication of Network Properties (sconepro), https://datatracker.ietf.org/group/sconepro/about/.

## =====  CHANGE =====

## 5.X	Key Issue #X: Secure Communication of Network Properties (SCONE-PRO) and 5G Media Streaming

### 5.X.1	Description

At recent IETF meetings, the issue of Secure Communication of Network Properties (SCONE-PRO) [SCONE-PRO] had been discussed. It was highlighted in several inputs that bandwidth is and remains a scarce resource, and that video is and will stay the dominant from of media on the Internet. Despite continuous capacity investments, it is hard to keep up with demand needed for video delivery. In the following, a few key issues are highlighted that motivate the work with references to the material:

-	ABR Video Shaping: https://datatracker.ietf.org/meeting/119/materials/slides-119-sconepro-how-networks-shape-traffic-02. This presentation introduces ABR video shaping, for which nowadays deep packet inspection and heuristics methods are used to throttle the video flow with a shaper or policer. It also addresses the downsides of policing and shaping and points to the lack of interoperability.

-	How YouTube™ coordinates with some MNOs: https://datatracker.ietf.org/meeting/119/materials/slides-119-sconepro-youtube-plan-aware-streaming-01. This presentation provides insights how YouTube coordinates with some MNOs. An API exists documenting the maximum media rate, provided out-of-band from operator to service provider, and updates to this value be provided. The max bitrate is not exceeded by the format, but at the same time no policing/shaping is applied. The resulting reduced rates reduce costs and improve user experience.

-	SCONE-PRO Problem Statement: https://datatracker.ietf.org/meeting/120/materials/slides-120-sconepro-problem-statement-00. The presentation also again highlights traffic shaping issues, including

-	ABR schemes are not perfect and don’t converge quickly, causing poor user experience and stalling as it “ping pong” between qualities.

-	Congestion Controllers are better suited to simple queueing and often make the “ping ponging” worse.

-	CC's (and ABR's) bandwidth estimation often overshoot significantly due to the burst allowance of the TBF.

-	The limit imposed by the TBF is artificial – it can support instantaneously more bandwidth, leading to periods of underutilization and difficulty for radio equipment to optimize spectrum usage.

The document further indicates that there are benefits that the video content provider receives maximum instantaneous throughput property from the network, while the shaper is removed or “dialed back”. This would result to move from a congestion-limited approach to an application-limited approach.

-	An initial draft charter was provided in https://datatracker.ietf.org/meeting/120/materials/slides-120-sconepro-draft-charter-v1-july-20-00

-	Video traffic is 70% of the overall traffic volume on the Internet and is expected to grow to 80% by 2028. Across developed and emerging markets video traffic forms 50-80% of traffic volume on mobile networks. New formats like short form videos have seen tremendous growth in recent years. These growth trends are likely to increase with new populations coming online on mobile-first markets.

-	Local access network conditions may constrain the maximum throughput for a given client, or be so volatile as to rapidly change the maximum throughput throughout the course of a session. In addition, despite capacity augmentation work such as deployment of new generations or new bands of spectrum, capacity augmentation efforts are not keeping pace with growth in demand. These network operators have found it faster and less expensive to invest in shaping (also called throttling) of video traffic on a per-flow basis, which negatively affects video stream quality. This is done for both network management and business motivations. Network operators cannot explicitly measure the degradation to end user quality of experience (QoE) caused by traffic shaping, making this approach open loop.

-	Video traffic usually employs adaptive bitrate (ABR) schemes to dynamically adjust the video quality (and thus the data rate) in response to changing network conditions. Ideally, when a network operator performs traffic shaping, the ABR scheme should adapt the video quality in use to reflect the data rate allowed by shaping, and converge on a bitrate allowed by the shaper. In practice this convergence is extremely difficult to achieve while maintaining a good user experience. Application providers are even designing algorithms to detect the presence of such traffic shapers and estimate the targeted shaping rate, however, these algorithms are likely to be both inaccurate and complex. Instead, it would be beneficial, for both the application provider and network operator, to signal network attributes to the application to self-adapt its video traffic to conform to the specified characteristics. The application provider has the ability to measure end user QoE and therefore can self-adapt with QoE feedback.

-	The Secure Communication of Network Properties (SCONEPRO) Working Group's primary objective is to specify a 'maximum achievable throughput' property for QUIC-based streaming video and an on-path protocol for securely communicating this property from a network device to a client endpoint.

-	Core solution characteristics are doccumented including:

- 	Flow associativity. The network communicates applicable properties as they relate to specific QUIC connections. This ensures that applications can authorize and apply actions on a per-QUIC connection basis.

-	Single communication channel for both client initiation and network properties. The communication channel is initiated by a client device, just as the end-to-end application flows are also typically initiated by a client. The same communication channel is used to provide network properties to the client.

-	Network properties sent from the network. The network provides the properties to the client. The client might communicate with the network but won't be providing network properties.

-	On-path establishment. That is, no off-path element is needed to establish the communication channel between the entity communicating the properties and the client.

-	Optionality. The communication channel is strictly optional for the functioning of application flows. A client's application flow must function even if the client does not establish the channel.

-	Properties are not directives. A client is not mandated to act on properties received from the network, and the network is not mandated to act in conformance with the properties.

As 5G Media Streaming provides functionalities that are similar and aligned to the objectives of the IETF work, this key issue will address aspects that investigate how the requirements from SCONE-PRO are met by 5G Media streaming, to what extent SCONE-PRO can be combined with 5G Media Streaming and extensions to 5G Media Streaming would be suitable to address combination with SCONE-PRO.

Also, other technologies discussed for example in Common Media Server Data (CMSD) may support requirements defined in SCONE-PRO.