3GPP TSG-SA4 Meeting #129-e	S4-241478

# online,  , 19 - 23 Aug 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.265 | CR | pseudo | rev | - | Current version: | 0.2.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [VOPS] Completion of existing Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | VOPS |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

## 5	Video Coding Capabilities

## 5.1	Overview

This clause defines video decoding capabilities and video encoding capabilities for 3GPP media delivery.

## NOTE: 	These clause does not specifiy whether these capabilities are required, recommended or suggested to be supported. This aspect is left specific service specifications or external specifications to refer to the capabilities defined in this clause.

## 5.2	Codecs, Profiles and Levels

### 5.2.1	Codec & profile

### This specification defines capabilities based on the following video codecs and video codec profiles:

-	AVC/H.264 Progressive High Profile [h264],

-	HEVC/H.265 Main Profile Main Tier [h265],

-	HEVC/H.265 Main-10 Profile Main Tier [h265].

### 5.2.2	Codec & profile & Levels

### This specification defines capabilities based on the following video codec profile and levels:

-	AVC/H.264 Progressive High Profile Level 3.1,

-	AVC/H.264 Progressive High Profile Level 4.0,

-	AVC/H.264 Progressive High Profile Level 4.2,

-	AVC/H.264 Progressive High Profile Level 5.1,

-	AVC/H.264 Progressive High Profile Level 6.1,

-	HEVC/H.265 Main Profile Main Tier Level 3.1,

-	HEVC/H.265 Main-10 Profile Main Tier Level 4.1,

-

HEVC/H.265 Main-10 Profile Main Tier Level 5.1,

-	HEVC/H.265 Main-10 Profile Main Tier Level 6.1.

## 5.3	Single-Instance Decoding Capabilities

Editor’s Note: This is copy and paste from S4-240619, clause 5.2.3. More edits are needed.

The following decoding capabilities are defined:

-	AVC-FullHD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 4.0 [h264] bitstreams.

-	AVC-UHD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 5.1 [h264] bitstreams with the following additional requirements:

-	the maximum VCL Bit Rate is constrained to be 120 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively; and,

-	the bitstream does not contain more than 10 slices per picture.

-	AVC-8K-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 6.1 [h264] bitstreams with the following requirements:

-	the maximum VCL Bit Rate is constrained to be 120 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively; and,

-	the bitstream does not contain more than 16 slices per picture.

-	the bitstream shall not include horizontal motion vector component values that exceed the range from −2048 to 2047, inclusive, or that have vertical motion vector component values that exceed the range from −512 to 511, inclusive, in units of ¼ luma sample displacement. This constraint should be indicated by using values of log2_max_mv_length_horizontal less than or equal to 11 and values of log2_max_mv_length_vertical less than or equal to 9.

-	HEVC-HD-Dec: the capability to decode H.265 (HEVC) Main Profile, Main Tier, Level 3.1 [h265] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

-	HEVC-FullHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1 [h265] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

-	HEVC-UHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 5.1 [h265] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

-	HEVC-8K-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 6.1 [h265] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1 with the following further limitations:

-	the bitstream does not exceed the maximum luma picture size in samples of 33,554,432,

-	the maximum VCL Bit Rate is constrained to be 80 Mbps with CpbVclFactor and CpbNalFactor being fixed to be 1000 and 1100, respectively.

## 5.4	Single-Instance Encoding Capabilities

Editor’s Note: This is copy and paste from S4-240619, clause 5.2.4. More edits are needed.

The following encoding capabilities are defined:

-	AVC-FullHD-Enc: the capability to encode a video signal to a bitstream that is decodable by a decoder that is AVC-FullHD-Dec capable as defined in clause 5.3 with the following additional constraints:

-	up to 245,760 macroblocks per second;

-	up to a frame size of 8,192 macroblocks;

-	up to 240 frames per second;

-	the chroma format being 4:2:0; and

-	the bit depth being 8 bit;

-	HEVC-HD-Enc: the capability to encode a video signal with

-	up to 33,177,600 luma samples per second;

-	up to a luma picture size of 983,040 samples;

-	up to 120 frames per second;

-	the Chroma format being 4:2:0; and

-	the bit depth being 8 bit;

to a bitstream that is decodable by a decoder that is HEVC-HD-Dec capable as defined in clause 5.3.

-	HEVC-FullHD-Enc: the capability to encode a video signal to a bitstream that is decodable by a decoder that is HEVC-FullHD-Dec capable as defined in clause 5.3 with the following additional constraints:

-	up to 133,693,440 luma samples per second;

-	up to a luma picture size of 2,228,224 samples;

-	up to 240 frames per second;

-	the Chroma format being 4:2:0; and

-	the bit depth being either 8 or 10 bit;

-	HEVC-UHD-Enc: the capability to encode a video signal to a bitstream that is decodable by a decoder that is HEVC-UHD-Dec capable as defined in clause 5.3 with the following additional constraints:

-	up to 534,773,760 luma samples per second;

-	up to a luma picture size of 8,912,896 samples;

-	up to 480 frames per second;

-	the Chroma format being 4:2:0; and

-	the bit depth being either 8 or 10 bit;

## 5.5	Multi-Instance Decoding Capabilities

Editor’s Note: This is copy and paste from S4-240619, clause 5.2.5. More edits are needed.

The following multi-instance decoding capabilities are defined:

-	AVC-FullHD-Dec-2: The capability of supporting up to two (N=2) concurrent decoder instances with the aggregate capabilities of AVC-FullHD-Dec as defined in clause 5.4.

-	AVC-UHD-Dec-4: The capability of supporting up to four (N=4) concurrent decoder instances with the aggregate capabilities of AVC-UHD-Dec as defined in clause 5.4.

-	HEVC-UHD-Dec-4: The capability of supporting up to four (N=4) concurrent decoder instances with the aggregate capabilities of HEVC-UHD-Dec as defined in clause 5.4.

-	UHD-Dec-4: The capability supporting up to four (N=4) concurrent decoder instances with either:

-	the aggregate capabilities of AVC-UHD-Dec-4 as defined in this clause,

-	the aggregate capabilities of HEVC-UHD-Dec-4 as defined in this clause, or,

-	the capability of decoding up to 4 bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-FullHD-Dec or HEVC-FullHD-Dec as defined in clause 5.4.

-	AVC-8K-Dec-8: The capability of supporting up to eight (N=8) concurrent decoder instances with the aggregate capabilities of AVC-8K-Dec as defined in clause 5.4.

-	HEVC-8K-Dec-8: The capability of supporting up to eight (N=8) concurrent decoder instances with the aggregate capabilities of HEVC-8K-Dec as defined in clause 5.4.

-	8K-Dec-8: The capability supporting up to eight (N=8) concurrent decoder instances with either:

-	the aggregate capabilities of AVC-8K-Dec-8 as defined in this clause,

-	the aggregate capabilities of HEVC-8K-Dec-8 as defined in this clause, or,

-	the capability of decoding up to:

-	eight bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-FullHD-Dec or HEVC-FullHD-Dec as defined in clause 5.4; or,

-	four bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-UHD-Dec or HEVC-UHD-Dec as defined in clause 5.4.

## 5.6	Multi-Instance Encoding Capabilities

## This specification does not define multi-instance encoding capabilities.