3GPP TSG-SA WG4 Meeting #129e	S4-241506

# Only – 19-23 July 2024

Source:	InterDigital Europe

Title:	Haptic Use case - Immersive Entertainment

Spec:	3GPP TR 26.854

Agenda item:	15.11

Document for:	Agreement

1. Introduction

This contribution proposes a use case “Immersive entertainment”, which combines the description of use cases from 3GPP TR22.856 and TR22.847 and elaborate on its characteristics.

2. Reason for Change

Provide a third use case for Haptic.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 26.854

* * * First Change * * * *

## 5.4		Immersive Entertainment

Use Case Name:

Immersive entertainment (live events, sport, gaming, movies, music) From: TR 22.847 5.6 “Live Event Selective Immersion” TR 22.856 5.6 “Mobile Metaverse for Immersive Gaming and Live Shows” TR 22.856 5.22 “Mobile Metaverse Live Concert”

Description:

Media including here movies, TV series, social networks, gaming... The later could be multi-party or single user. In this scenario users are playing content (local or streamed) with video (2D or 3D immersive), audio (potentially spatialized) and haptics. The haptic signal is synchronized with the content to add physical feedback in the form of some haptic effects. The user is using at least a smartphone with vibrotactile actuators, or more advanced devices such as XR glasses and potentially multi wearable devices to spatialized haptic effects. Mobile metaverse services allow people to enjoy an online digital concert with their avatars beyond the limitation of time and space. In order to provide immersive interactive location agnostic service experience to mobile metaverse service customers, large amount of computing resources is needed to perform real-time processing for audio, video, and interactive data, etc. The thing to realize here is that different customers will use terminals e.g. XR glasses with different brands and different processing capabilities, some of the glasses will not have enough computing resources to perform the real-time rendering. Through split rendering, most of the computing work task can offload to the network, the high speed and low latency transmission provided by 5G system can cooperate with the edge cloud side for real-time rendering, and combine with the local optimized rendering of the XR terminal side to provide the immersive and unbounded XR experience. In addition to this, similar to the real world, people are more likely to watch a concert together with their friends, further, the mobile metaverse live concert service is also provides private boxes for group of avatars to enjoy the concert privately, and different types of social authority can be provided in the private box on demand. Ed Notes: the above text is copy pasted from the references. Haptic modalities included here: mostly thermal, kinesthetic and vibrotactile, but acceleration can be used for some events Haptic characteristics: Mono-directional, low data-rates.

Categorization

Type: VR, AR, mobile, TV Delivery: Device: HMD, headphone, cushion, chair, VR controller, mobile phone

Preconditions

- Terminal equipment capable of initiating and playing an event session. - Capture sensors for feedback from the live events - Participant: Haptic actuators, displays and headphones. - Phone participant: Phone with haptic actuators (generally vibrotactile).

Requirements

Audio-Tactile delay = 50 ms Tactile-Audio delay = 25 ms Visual-Tactile delay = 15 ms Tactile-Visual delay = 50 ms Maximal end-to-end latency = 5ms (esp. for gaming) Reliability = 99.9% Bit-rates = In the range of 8kbps-16kbps for reasonable quality (32-64 kbps for high quality) per channel. Number of Channels = between 1 and 20+ with non-continuous packets. Live applications could be continuous if direct sensors information is recorded and distributed. Format: both signal (time samples) and parametric.

Feasibility

Enabling technologies have reached a sufficient maturity in terms of: - Haptics experiences creation, coding, transmission and rendering - Editing tools to create Haptic effects - Integration into Media applications - Integration into hardware devices - Availability of sensors to capture live information (accelerometer, pressure, temperature…) Connected devices in various form factors exist that are capable of decoding and rendering haptics (cushion, controllers, suits…). On site production units can ingest sensors signals and integrate synthesizers to generate haptic effects directly in the production vans for live haptics.

Interoperability considerations

Haptic data should be delivered and rendered with various applications, environments and devices. Adaptation to the rendering capabilities should be supported. User adaptation could be provided.

Potential Standardization Status and Needs

- Haptics data representation and coding: (Section 6 and 7) - Haptics data carriage, transport and streaming (Section 7) - Integration in 5G architecture and services. (Section 8)

Table 5.4: Use-case - Immersive Entertainment

* * * End of Changes * * * *