3GPP TSG-SA WG4 Meeting #129e	S4-241507

# Only – 19-23 July 2024

Source:	InterDigital Europe, Tencent

Title:	Haptics input formats

Spec:	3GPP TR 26.854

Agenda item:	15.11

Document for:	Agreement

1. Introduction

This contribution proposes a first text and information on Haptics input format.

2. Reason for Change

Provide initial text for Haptics input formats to populate section 6, some references for section 2 and abbreviations in section 3 of TR 26.854.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.854

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[SMPTE ]	SMPTE ST 2100-1:2017 “Definition and Representation of Haptic-Tactile Essence for Broadcast Production Applications”.

[AHAP]	Apple Haptic and Audio Pattern JSON format for Haptics https://developer.apple.com/documentation/corehaptics/representing_haptic_patterns_in_ahap_files

[IVS] 	IVS Immersion Corporation haptic format, https://www.immersion.com/

[23090-31]	ISO/IEC 23090-31 Information technology — Coded representation of immersive media Part 31: Haptics coding

* * * Second Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

AIF	Audio Interchange file Format

HJIF	Haptic JSON Interchanged Format

JSON	JavaScript Object Notation

LRA	Linear Resonant Actuators

ERM	Eccentric Rotating Mass

WAV	Waveform Audio File Format

* * * Third Change * * * *

# 6	Haptic input formats and device types

## 6.1	introduction

Haptics refers to the sense of touch, and encompasses the generation, manipulation, and perception of tactile sensations, forces, and motions. In the media domain it relates to the capture and rendering of physical information about objects and environments due to interactions with the user. The capture is generally performed with sensors and the rendering with hardware actuators allowing to render different modalities.

In general, three types of modalities are considered, tactile (vibration, temperature, pression), kinaesthetic (force) and proprioception (motion, acceleration). Those modalities are rendered in the human body through different mechanoreceptors. The density and properties of those receptors are different depending on the modality and the body parts. In addition, each individual has different sensibility, which can be managed through users’ profiles and adaptation.

Finally, the rendering is performed with specific hardware actuators embedded into user devices. The different Haptic modalities are rendered with different actuators. Typically, tactile sensations can be rendered with vibrotactile (e.g. linear resonant actuators (LRA), eccentric rotating mass (ERM), temperature (piezzo), wind (fan), force (robotic)) … devices.

## 6.2		Haptic input formats

### 6.2.1	Introduction

Haptic input formats are essential in capturing and transmitting tactile sensations in a variety of applications, ranging from simple vibrations in mobile devices to complex force feedback in virtual reality systems. Two types of input formats exist, time-sampled signals (typically PCM encoded) captured by sensors, and parametric description of haptic effects.

### 6.2.2	Time-sampled signals

The PCM input signals for Haptics are very often proprietary defined for each type of sensor. But the PCM format is very simple, it was designed for raw audio coding and consists of a direct representation of the digital sample values of a recording, stored in file formats such as .WAV and .AIF files. The amplitude of the analog signal is sampled at uniform time intervals, and each sample is quantized to the nearest value within a range of digital steps. SMPTE specified one format for Haptic tactile essence data (raw captured data) in its specification [SMPTE]. However, this SMPTE PCM format is limited to acceleration, velocity and position.

### 6.2.3	Parametric description of haptic effects

For descriptive haptic, several proprietary formats have been defined, such as the Apple [AHAP] JSON format for Haptics and the Immersion corporation [IVS] XML-based descriptive format. The new [ISO/IEC 23090-31] standard defined by MPEG specifies a descriptive format, compatible with the AHAP and IVS formats, for the description of Haptics. This standard is also based on JSON and is called HJIF (Haptic JSON Interchanged Format).

### 6.2.4	Comparison and applications

The choice between time-sampled signals and parametric descriptions depends on the application and the desired level of control. Time-sampled signals are more suitable for applications requiring high fidelity and real-time capture of physical interactions, such as in medical simulations or fine-grained tactile feedback systems. Parametric descriptions, on the other hand, are ideal for applications where flexibility and ease of use are paramount, such as in mobile notifications or gaming, where the haptic feedback needs to be easily adjustable and consistent across a variety of devices.

* * * End of Changes * * * *