Source:	Bytedance

Title:	[IVAS ph2] Proposal for IVAS mode negotiation with reduced first frame latency

Document for:	Discussion and Agreement

Agenda Item:	7.5

# 1 Introduction

During the post SA4#127-bis-e Audio SWG telco held on 29 April 2024, the IVAS level issue was discussed, and it was agreed that:

    - Onion principle shall apply, support of a higher level always implies support of a lower level

      - Note: The implication of the onion principle will depend on decision of how levels are defined.

    - A minimum core set needs to be defined (level 1), what features to include are TBD

    - A receiver of level 1 needs to be able to decode what a sender of level 1 produces

Some negotiation schemes have been proposed before [1], but with the definition of onion principle, the source believe in order to proceed further, we presents some thoughts on this topic and proposes an approach for consideration.

# 2 Discussions

As mentioned before in [1] on this topic, IVAS offers a large number of operational modes, which are proposed to be grouped into levels [2]. Although the negotiation scheme in [1] is versatile, it requires the negotiation to be fully complete before audio is transmitted. This will introduce latency between the caller make the call, and the receiver hears the sound. We name this latency as “first-frame-latency”.

Can this “first-frame-latency” be further minimized? Even though the exact definitions of levels are still TBD, the a-for-mentioned “onion principle” of level definitions is agree. Based on this principle, there will have one minimum level audio stream that can be encoded and decoded by any IVAS-enabled UEs, and this fact can be guaranteed before any negotiation happens.

Based on this fact, we propose a negotiation scheme that has lower first-frame-latency than [1], yet still leverage most of the architecture of it.

# 3 Negotiation Scheme

Our negotiation scheme is evolved based on [1] but included 2 changes.

- Instead of grouping encoding and decoding abilities into audio format and bit rates, we group them in groups.

- Instead of sending audio RTPs after negotiation is fully completed, we propose UE to send the minimum-supported level audio stream as soon as the negotiation is started.

## 3.1 Initial Negotiation

Our negotiation scheme to establish the call can be described as follows:

Figure 1. Our proposed coding negotiation scheme with reduced first-frame-latency

- UE-A sends an invite to UE-B, and simultaneously sends the MinLevel code stream.

- The fmtp field of the invite SDP: level={(MinLevel, maxLevel-A-send), (MinLevel, maxLevel-A-receive)}

- The highest IVAS codec Level of UE-A is affected by many factors such as its own hardware configuration, residual battery power, user settings, etc.

- When UE-B receives the MinLevel code stream, it immediately starts decoding. At this time, UE-B can already receive the sound of UE-A, although it is not the best sound quality/spatial sense.

- UE-B calculates the best Level for A->B and B->A based on the highest IVAS Level of UE-A and itself.

- Like UE-A, the highest IVAS codec Level of UE-B is also affected by many factors such as its own hardware configuration, residual battery power, user settings, etc.

- newLevel-B2A = min(maxLevel-B-send, maxLevel-A-receive)

- newLevel-A2B = min(maxLevel-A-send, maxLevel-B-receive)

- UE-B sends 18X/200/ACK information to UE-A to confirm the connection. The fmtp field of the ACK SDP: level={(newLevel-A2B, maxLevel-B-send), (newLevel-B2A, maxLevel-B-receive)}. And simultaneously sends the audio code stream of newLevel-B2A to UE-A.

- Because newLevel-B2A <= maxLevel-A-receive, UE-A can definitely decode the audio sent back by UE-B. At this time, UE-A can already hear the best sound quality and spatial sense after the negotiation is completed.

- UE-A sends the audio code stream of newLevel-A2B to UE-B.

- At this time, UE-B can also hear the best sound quality and spatial sense after the negotiation is completed.

## 3.2 Re-negotiation

Our re-negotiation scheme is similar to the initial negotiation, which is described as follows:

Figure 2. Our proposed coding re-negotiation scheme with reduced first-frame-latency

- UE-A and UE-B are transmitting the audio stream of the currentLevel.

- A configuration change occurs in UE-A, and the maxLevel-A-send and maxLevel-A-receive change, triggering re-negotiating.

- If currentLevel-A2B > maxLevel-A-send

  - UE-A changes encoding level to maxLevel-A-send

- If currentLevel-B2A > maxLevel-A-receive

  - UE-A stops decoding.

- The same as the initial negotiation process.

- If the decoding of UE-A stops, resume decoding.

## 3.3. Options for MinLevel

We would like to hear the group’s thought on the definition of “MinLevel”. We have the following 2 options for consideration, but we are open to more suggestions.

Option 1. MinLevel is equal to the lowest IVAS level.

Option 2. MinLevel is EVS compatibility mode, since most UEs (4G / 5G) on the market supports EVS.

# 4 Experiment

To verify the reduction of first-frame latency, we developed a UE simulator program that negotiate and send RTP. We also replaced the encoding/decoding combinations in [1] using levels, which forms our “Conventional” negotiation scheme as follows:

Figure 3. Conventional negotiation scheme which combines [1] and levels

The UE simulators can negotiate either according to the scheme in [1] (noted as Conventional), or according to our proposed scheme (noted as Bytedance).

We run our experiment on two computers. UE-A simulator is located in Bytedance Shanghai office, UE-B simulator is located in Bytedance Beijing office. Ping latency between two computers is 30.48 milliseconds (average of 100 consecutive measurements).

Since the absolute time on two computers is not completely aligned, we take the sum of the ping latency and the time delta between UE-A sends the invite and the UE-B receives the first RTP. Measurement can be expressed by the following equation:

t1st frame latency=tping+(t1st Rtp-tinvite time stamp)

For each negotiation scheme, we average 10 measure of first-frame latency to form our result. Our experiment shows that:

- When using “Conventional” negotiation scheme, first frame latency average to 81.12 milliseconds.

- When using our proposed negotiation scheme, first frame latency average to 35.37 milliseconds.

Our experiment suggest that our method can reach more than 50% first-frame latency reduction in real world setup.

# 5 Proposal

- It is proposed to accept our negotiation scheme that reduces first-frame latency.

- Definition for the MinLevel audio stream need to be choose.

References

[1] SA4aA240027, [IVAS] On IVAS Operating Mode Negotiation, Huawei Technologies Co., Ltd.

[2] S4-240981, [IVAS] Proposal for IVAS service options and operating mode negotiation, Huawei Technologies Co., Ltd.