3GPP TSG-SA WG4 Meeting #129-e	S4-

# Online, 19th August 2024 - 23rd August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0020 | rev | <Rev#> | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] WT13: Initial section x.2 Collaboration Scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 14-05-2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | FS_AMD includes a new topic about opportunities with QUIC for segmented streaming (WT 13). |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution provides an initial section x.2 on Collaboration Scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No progress on WT13 of FS_AMD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change #1

### 5.x.2	Collaboration Scenarios

#### 5.x.2.1	General

A 5GMS Application Provider runs an adaptive media streaming service between a 5G Media Streaming AS and an UE using 5G Media Streaming over M2d and M4d. However, only M4d is relevant for this key topic since it focuses on the media delivery to the UE and not on the ingest of the media itself. Also, since M5 is not meant to transport media, the M5 interface is also excluded from this key topic.

For the purpose of describing the scenario, it is only assumed that the 5GMS Client supports the QUIC protocol. Whether higher level protocols based on QUIC are supported by the 5GMS Client, for instance HTTP/3 or WebTransport, is on purpose left out and the analysis should be applicable to any of them.

#### 5.x.2.2	QUIC-agnostic 5GMS Client

In this scenario, the Media Stream Handler of the 5GMS Client operates a QUIC session but the 5GMS Client has no specific feature regarding QUIC. This has the advantage that 5GMS Client is generic and implements the same logic whether QUIC is used or not for the delivery of the media.

#### 5.x.2.3	QUIC-aware 5GMS Client

In this scenario, the Media Stream Handler of the 5GMS Client operates a QUIC session and the 5GMS Client is able to detect whether QUIC is used and in case it is used, the 5GMS Client can apply different logics. The QUIC-aware 5GMS Client is limited in the control API exposed by the Media Stream Handler regarding QUIC functionality.

With some control over the QUIC streams, such 5GMS Client would be able to:

- Set relative priorities between the different QUIC streams, especially if HTTP/3 is used on top by the Media Stream Handler and assuming the QUIC client API offers such functionality. Relative stream priorities can be useful for differentiate audio and video, base layer and enhancement layer, etc.

- Receive MPD updates proactively by the server, especially if HTTP/3 is used with the server push feature.

[Editor’s note: More possible special features to be added]

#### 5.x.2.4	QUIC-optimized 5GMS Client

In this scenario, the Media Stream Handler of the 5GMS Client operates a QUIC session and the 5GMS Client is able to control the delivery of the media within the QUIC session. The QUIC-optimized 5GMS Client provides the finest control over the delivery of media within the QUIC session.

With fine control over the QUIC streams, such 5GMS Client would be able to:

- Set relative priorities between the different QUIC streams. Relative stream priorities can be useful for differentiate audio and video, base layer and enhancement layer, etc.

- Receive MPD updates proactively by the server, especially if HTTP/3 is used with the server push feature.

- Use one QUIC stream for all the media segments of a given component (e.g. per CMAF Track).

[Editor’s note: More possible special features to be added]

END OF CHANGES